punn stiENTiFit me.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

MSDS #:    242.00

Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Chromium Sulfate

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Chromium Sulfate

Synonym: chromium (III) sulfate hydrate, chromic sulfate

CAS#: 10101-53-8

Section 3 — Hazards Identification

Light green crystals. Odorless. Irritant to body tissues.

flinn at-a-glance

Health-1 Flammability-0 Reactivity-0 Exposure-1 Storage-0

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth, give 1 to 2 cups of water or milk, induce vomiting. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Non flammable solid.

When heated to decomposition, emits toxic fumes of SOx and Cr.

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE

and SCBA with full facepiece operated in positive pressure mode.

NFPA code

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates. Store in a cool, dry place. Use and dispense in a hood.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 0.5 mg/m3 (OSHA)

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 of 2

FLinn SCIENTIFIC int.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Chromium Su Irate

MSDS #:    242.00

Revision Date:   November 25,2002

Section 9 — Physical and Chemical Properties

Light green crystals. Odorless. Solubility: Water soluble. Formula: Cr2(SO4)3 nH2O Formula Weight: 662.39

Specific Gravity: 1.9 Melting Point: 100 C

Section 10 — Stability and Reactivity

Avoid heat and strong oxidizing agents. Emits toxic fumes when heated. Shelf life: Indefinite

Section 11 —Toxicoloqical Information

Acute effects: Irritant Chronic effects: Target organs: Skin

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #27f is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-Listed, EINECS-Listed (233-401-6), RCRA D007.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510

(800) 452-1261 Fax (866) 452-1436

D 7009 Flinn Scientific Tnc   All Rights Reserved.

e 2 r>F 2