MATERIAL SAFETY DATA SHEET

Section I

Product Identification

Product Name:  305	LIME DELETE

Generic Name:  ACID CLEANER-DELIMER

opliers Name:   ALCO Chemical, Inc.

Jress:	45 North Summit Street

Akron, Ohio 44308
Phone:	(216) 253-3535

Proper Shipping Name:COMP. CLEANING LIQUID UN1760
Chemical Family:	N/A

Formula:	N/A

EMERGENCY/CHEM*TEL: 1-800-255-3924

Evaporative Rate (water = 1): <1 Vapor Density (air = 1): >1

Specific Gravity: 1.07 Volatiles: 95+

| Section Ha Ingredients Listed as Hazardous According to OSHA 29CFR1910 |  |  |  |
| --- | --- | --- | --- |
| Chemical Name | CAS# | TLV(PPM) WT% PEL (PPM) TWA | STEL CARCINOGENIC |
| Phosphoric Acid* | 7664-38-2 | 30-40 1 1 | 3 No |
| (* Indicates SARA Title 313) |  |  |  |
| Section lib | Ingredients Not Listed | as Hazardous |  |
| Chemical Name Nonionic Surfactant (linear Alcohol | CAS# | WT% |  |
| Alkoxylate) | 52232-09-4 | 0-1 |  |
| Dye | NA | 0-1 |  |
| (per OSHA Hazard Communication Standarc | 1910.1200(1)) |  |  |
| Section III | Physical Data |  |  |

Boiling Range:      212 (initial)	% Solids by Weight:   N/A

Vapor Pressure:      26 mm Hg @ 68F      % Solubility in Water: 100% Physical Description: green liquid with characteristic odor

pH:1

| Section IV | Fire & Explosion Hazards |  |
| --- | --- | --- |
| Flash Point: N/A Flammability Limits (LEUUEL N/A /N/A Special Fire Fighting Procedures: N/A Unusual Fire & Explosion Hazards: N/A | Method Used: N/A Extinguishing Media: N/A | Health: 2 Flammability: 0 Reactivity: 0 M.P.P.;B |
| »n V | Health Hazard Data |  |

Primary Routes of Entry: Skin , Eyes, Ingestion, Inhalation

Permissible Exposure Limit: N/A Other Limit: N/A

Threshold Limit Value: N/A

Emergency and First Aid Procedures: SKIN; remove contaminated clothing.  Flush exposed area with large quantities of water.  EYES; Flush eyes with large quantities of water, holding eyelids open. If irritation persist, seek medical attention. INHALATION; Get to fresh air, if breathing is difficult, give oxygen.  INGESTION; DO NOT INDUCE VOMITING.  Give large quantities of water or milk,  Seek medical attention at once.

Section VI

Reactivity Data

Stability:STABLE	Incompatibility (material to avoid): strong alkalis, most metals, bleach.

Hazardous Polymerization:Will Not Occur	Hazardous Decomposition Products:hydrogen or chlorine gas

Section VII

Spill or Leak Procedures

Steps to be taken in case material is released or spilled: mop up or otherwise absorb and hold for disposal.

Waste disposal method: Any method in accordance with applicable laws	Volatile organic compounds (VOC):   N/A

RCRA hazardous waste number (40 CFR 261.33): N/A	CERCLA (superfund) reportable quantity (in Ibs.):  N/A

Special Protection Data

Section VIII

Respiratory protection (specific type):   No special requirements	Protective gloves (specific type):   Waterproof gloves recommended

Eye protection: Safety glasses recommended	Other protective equipment: No special requirements

Ventilation:   Provide local exhaust to keep TLV of section Ha ingredients, if applicable, below acce

Section IX

Special Precautions

KEEP AWAY FROM CHILDREN	KEEP CONTAINER CLOSED DURING STORAGE

FOR USE BY TRAINED PERSONNEL ONLY	FOR INSTITUTIONAL USE ONLY

The above information pertains to this product as currently formulated, and is based on the information available at this time. Additions of reducers

or other additives to this product may substantially alter the composition. We make no warranties, expressed or implied, and assume no liability in

connection with any use or the information.

tion I

Product Identification

10/05/96

ALCO/MANDALA PRODUCTS       45W. Summit Street Akrorr, Ohio 44303   Ph: (216) 253-3535   (800)589-2526