FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    244.00

Material Safety Data Sheet (MSDS)	Revision Date: November25,2002

Section 1 —Chemical Product and Company Identification

Citric Acid

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Citric Acid monohydrate and/or Citric Acid anhydrous Synonym: 2-hydroxy-l,2,3-propanetricarboxylic acid CAS#: 77-92-9 (anhydrous) or 5949-29-1 (monohydrate)

Section 3 — Hazards Identification

White granules. Odorless.	FLINN Ar-A-GLANCE

A severe eye and moderate skin irritant.	Health-0

Flammability-1 Reactivity-0 Exposure-1 Storage-1

0 is low hazard; 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Combustible solid.	NFPA code

When heated to decomposition, emits acrid smoke and fumes.	None Established

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn suggested chemical storage pattern: Organic #1. Store with acids, anhydrides and peracids. Store in a cool dry place. Store in a Flinn Chem Saf Bag.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

© 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Citric Acid

MSDS #:     244.00

Revision Date:   November 25,2002

Section 9 — Physical and Chemical Properties

White granules. Odorless. Solubility: Water soluble. Formula: C6H8O7 Formula Weight: 192.14

Specific Gravity: 1.54 (anhydrous)

Section 10 — Stability and Reactivity

Avoid contact with oxidizers, bases and reducing agents. Shelf life: Substance efflorescent.

Section 11 —Toxicoloqical Information

Acute effects: Severe eye irritant Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (201-069-1).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510

(800) 452-1261 Fax (866) 452-1436

2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2