MATERIAL SAFETY DATA SHEET

Section I

| Manufacturer's Name : R&D Poly Inc. | Emergency Telephone Number | (404) 229-2017 |
| --- | --- | --- |
| Address (Number. Sfreet. City, State, and ZIP Code; 31 Clarendon Avenue | Telephone Number for Information | (404) 298 5381 |
| Avondale Est., GA 30002 | Data Prepared October 10, 2002 |  |
|  | Chemical Name: Chemical Family: |  |

Section II — Hazardous Ingredients and Identity Information

Cornponent (s)    Chemical Name                  Registration No.                           OSHA PEL

Ethyl Alcohol  CAS 64-17-5

Chloroexylenol  CAS  88-04-0

This product is not known to contain a substance subject to Section 313 of Title III of the Superfund Amendments and Reauthoriztionn Act of 1986 SARA and 40 CFR 372 at or above minimal amounts.

% Approx

35 to 45

44 to 36

PEL STEL/TWA

1000

Section III — Physical Data

| Boiling Point | NA | Specific Gravity (H2O = 1) | 0.942 |
| --- | --- | --- | --- |
| Vapor Pressure (mm Hg.) | No data | Percent Volatile | 68.75 |
| Vapor Density (AIR - 1) | 1.59 (ALC) | Evaporation Rate (Butyl Acetate - t) | 1.7 (ALC) |
| Solubility in Water Swells on contact with water | Slight | Appearance and Odor Off white pre-moistened towelette, mild fesh fragrance |  |

.

Section IV — Fire and Explosion Hazard Data

| Flash Point (Method Used) | 97.5 | Flammable Limits NA | LEL 3.3 | UEL 19 |
| --- | --- | --- | --- | --- |

Extinguishing Media

Dry Chemical, alcohol-type or all purpose foam, CO2.

Special Fire Fighting Procedures

Use water spray to cool fire-exposed containers and structures.  Use wate spray to disperse vapors:  re-ignition is possible.

Unusual Fire and Explosion Hazards

Flammable material.  Toxic gases (such as carbon monoxide and formaldehyde) may be released in a fire.  Concentrated vapors may be flammable

Section V —Reactivity Data

| Stability | Unstable |  | Conditions to avoid: Extreme temperatures, sources of ignition |
| --- | --- | --- | --- |
|  | Stable | X |  |

Incompatability (Materials to Avoid)

Strong oxidizers, strong acids

| Hazardous Decomposition or Byproducts Decomposition will not occur if handled/stored properly. Thermal decomposition may emit typical products of combustion |  |  |  |
| --- | --- | --- | --- |
| Hazardous Polymerization | May Occur |  | Conditions to Avoid Heat |
|  | Will Not Occur | X |  |

Section VI – Health Hazard Data

Routes of Entry – Inhalation Brief (minutes) inhalation will not cause adverse effects, although constant inhalation may cause irritation to upper respiratory tract.  Eyes:  Brief contac may cause mild irritation, prolonged contac may cuse moderate irritation assocated with redness.  Ingestion.  Small amounts swalld incidental to normal handlin isnot likely to cause injury.Swallowing larger amounts may cause injury.

Health Hazards (Acute and Chronic) – acute effects are possible irritation and discomfort, no known chronic effext have been established..

Emergency and First Aid Procedures  - Inhalation:  Remove victim to fresh air; provide oxygen if breathing is difficult seek medical attention if required.  EYES:  Flush eyes wit clear running water for a minimum of 15 minutes while holding eyelids open.  If irritation persists, seek medical attention.  Ingestion:  DO NOT induce vomiting, seek immediate medical attention.  Vomiting may be induced only under the supervision of a physician.

Carcinogencity:  No additional information

Medical Conditions Aggravated by Exposure.  Pre-existing skin, eye, or repiratory disorders may become aggravated through prolonged exposures

Section VII – Precautions for Safe Handling and Use

Spill Procedures:  Remove all sources of ignition.  Small spills may be flushed with large amounts of water; larger spills should be collected for disposal.

Disposal:  Dispose of in accordance with Loacal, State, and Federal regulations.  Products classified as non-hazardous may become hazardous waste upon contract with other products.  Refer to “40 CFR Protection of Environment:  Parts 260-299”  for complete waste disposal regulations.  Consult your Local, State or Federal Environmental ProtectionsAgency before disposing of any chemicals.

Precautions for Safe Storage and Handkling:  Keep container(s) tightly closed when not in use.  Avoid inhalation of high concentration of vapors.  Store in cool, dry area away from possible sources of ignition.  Keep this and other chemicals out of reach of children.

Section VIII — Control and Protective Measures

Respiratory Protection(Specify Type) – Ordinarily, none required

Ventilation – Local exhaust required:  Mechanical (general):  Yes.

Protective Gloves – Ordinarily, none required

Eye Protection – Ordinarily, none required.

Other Protective Equipment – Ordinarily, none required

Other Hygienic Practices – Practice safe work habits.  Use according to label instructions

Section IX — HIMIS Hazard Rating

Health   0				0=Insignificant		3=High

Flammablility  3				1=Slight			4=Extreme

Reactivity   0				2=Moderate

Transportation Information

Proper Shipping Name:  Flammable Liquid, N.O.S. (Ethanol)

Hazard Class/Pkg. Group:  3/III

Indentification Number:  NA1993

Ref:  173.150(b)(c)

Label:  None required.  Ground