SSS05234 MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28070

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT TO YOU IN COMPLIANCE WITH 29 CFR 1910.1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985.

IDENTITY AND DISTRIBUTOR'S INFORMATION

NFPA rating: Health, 1    Flammability, 2    Reactivity, 0

special , HMIS rating: Health, 1    Flammability, 2    Reactivity, 0

Personal Protection, B

MANUFACTURED FOR:  Triple S

2 Executive Park Drive

Bill erica, MA 01862 DOT HAZARD CLASSIFICATION:  ORM-D

IDENTITY (TRADE NAME AS USED ON LABEL): SSS INSECT SPRAY DATE PREPARED:     09/10/2001         PREPARED BY:  KG / IB MANUFACTURER'S MSDS NUMBER:  A00413    REVISION - 7 DISTRIBUTOR'S MSDS REFERENCE:    21715 INFORMATION CALLS:  (978) 667-7900 EMERGENCY RESPONSE NUMBER :  1 (800) 255-3924 NOTICE:  JUDGMENT BASED ON INDIRECT TEST DATA

SECTION 1 - MATERIAL IDENTIFICATION AND INFORMATION

COMPONENTS	SARA       OSHA        ACGIH

CHEMICAL AND COMMON NAMES      III List   PEL (ppm)   TLV (ppm)

(Hazardous components 1% or greater; Carcinogens 0.1% or greater)

Petroleum Distillate	No        100       100

(CAS # 64742-47-8)

D-Trans Allethrin	Yes	N/E	N/E

(CAS  #   584-79-2)

Piperonyl Butoxide	Yes        N/E       N/E

(CAS # 51-03-6)

isobutane / Propane Blend       No        800       800

(CAS # 75-28-5)

(CAS # 74-98-6)	No        1000       1000

N-octyl   Bicycloheptene	No	N/E	N/E

Dicarboximide (CAS # 113-48-4)

carcinogen Reference Source:     None of the above listed.

-1-

SECTION 2 - PHYSICAL/CHEMICAL CHARACTERISTICS

BOILING POINT: N/A

SPECIFIC GRAVITY, (H20=l): Concentrate Only = 0.953 vapor pressure: PSIG @ 70 deg. f. (Aerosols): Max. 60 vapor PRESSURE (Non-Aerosols) (mm Hg and Temperature): N/A

VAPOR DENSITY (Ai r=l): N/E EVAPORATION RATE, (     =1) : N/E

solubility IN WATER: Moderate

WATER REACTIVE: No

APPEARANCE and ODOR: Fine mist with floral fragrance.

SECTION 3 - FIRE AND EXPLOSION HAZARD DATA

FLAMMABILITY as per USA FLAME PROJECTION TEST (Aerosols):

NON-FLAMMABLE

AUTO IGNITION TEMPERATURE:  N/E FLAMMABILITY LIMITS IN AIR BY % IN VOLUME:

Page 1

SSS05234

% LEL: N/E        % UEL: N/E

FLASH POINT AND METHOD USED (non-aerosols): N/A EXTINGUISHER MEDIA: Foam, dry chemical, carbon dioxide, water

spray. SPECIAL fire FIGHTING PROCEDURE: Self-contained breathing

apparatus. unusual fire and explosion hazards: do not expose aeros9ls to

temperatures above 130 degrees F. or the container may

rupture.

SECTION 4 - REACTIVITY HAZARD DATA

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not OCCUr

incompatibility (materials TO avoid): Strong oxidizers, alkaline

media, heavy metals. conditions TO avoid: Open flame, welding arcs, heat, sparks.

HAZARDOUS DECOMPOSITION PRODUCTS: CO, C02.

SECTION 5 - HEALTH HAZARD DATA

primary routes OF ENTRY: inhalation

ACUTE EFFECTS:

inhalation: Excessive inhalation of vapors can be

harmful and may cause headache, dizziness, asphyxia, anesthetic effects, nausea.

Eye Contact: Irritation.

Skin C9ntact: Mild irritation.

ingestion: possible chemical pneumonitis if aspirated

into lungs.  Nausea, dizziness, loss of muscle coordination. CHRONIC effects : None known.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: May

aggravate existing eye, skin or upper respiratory conditions.

EMERGENCY FIRST AID PROCEDURES:

eye contact: Flush with water for 15 minutes,  if irritated,

seek medical attention. skin contact: wash with soap and water,  if irritated, seek

medical attention. inhalation: Remove to fresh air.  Resuscitate if necessary.

Get medical attention. ingestion: DO not induce vomiting.  Drink two large glasses of

water. Get immediate medical attention. -2-

SECTION 6 - CONTROL AND PROTECTIVE MEASURES

RESPIRATORY protection (specify type): If vapor concentration

exceeds tlv, use respirator approved by U.S. Bureau of

Mines for organic vapor.

protective GLOVES: Latex, if skin easily irritated. EYE PROTECTION: Safety glasses recommended. ventilation requirements: Adequate ventilation to keep vapor

concentration below TLV.

OTHER PROTECTIVE CLOTHING AND EQUIPMENT: None HYGIENIC WORK PRACTICES: Wash with soap and water before handling

food.  Remove contaminated clothing.

SECTION  7 - PRECAUTIONS FOR SAFE HANDLING AND USE

STEPS TO BE TAKEN IF MATERIAL IS SPILLED OR RELEASED:

Absorb with suitable medium.  Incinerate or landfill Page 2

SSS05234 according to local, state or Federal regulations. DO

NOT FLUSH TO SEWER.

waste disposal methods: Aerosol cans when vented to atmospheric pressure through normal use, pose no disposal hazard.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: DO not puncture

or incinerate containers. Do not store at temperatures above 130 degrees F.

OTHER PRECAUTIONS AND/OR SPECIAL HAZARDS: KEEP OUT OF REACH OF CHILDREN.  Avoid food contamination.  Avoid inhalation of spray mist. Avoid water contamination.

We believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind.