SELIG

MATERIAL SAFETY DATA SHEET

PAGE   1   OF  2

DATE      : Oi/02/,87 SUPERSEDES:

OLE SOL AEROSOL PRODUCT NUMBER: 0

EMERGENCY

SELIG CHEMICAL INDUSTRIES	LOCAL POISON CONTROL CENTER TELEPHONE

840 SELIG DRIVE, S. W.

ATLANTA, GA, 30378	TRANSPORTATION EMERGENCY

TELEPHONE (404)691-9220	CHEMTREC: TOLL-FREE 1-8OO-424-93OO ALL CALLS

BETWEEN 8: 00A.M. - 5: OOP. M.	DISTRICT OF COLUMBIA (202)483-7616 ALL CALLS

(EASTERN TIME ZONE)

CHEMICAL INDUSTRIES

A DIVISION OF

NATIONAL SERVICE INDUSTRIES

SECTION   I

CONTACT

RECORDED RECORDED

SECTION   II-HAZARDOUS        INGREDIENTS

- TOLUENE

- TREITHANOLAMINE

- ETHYLENE   GLYCOL   N-BUTYL   ETHER

- STEARYL   PROPIONATE

- THIS PRODUCT CONTAINS NO KNOWN OR SUSPECTED CARCINOGENS.
6

7 8 9 10

CAS

108-88-3 102-71-6 111-76-2

. 1

SECTION   III

PHYSICAL

DATA

(FOR FILL MATERIAL ONLY)

SPECIFIC GRAVITY

BOILING POINT (F)   :	UNK

VAPOR PRESSURE(MMHG):	UNK

VAPOR DENSITY(AIR=1):	UNK

SOLUBILITY IN WATER :	COMPLETE

PERCENT VOLATILE BY VOLUME (7.)
EVAPORATION RATE(NA	= 1 )

PH(CONCENTRATE)	: UNK

PH(USE DILUTION OF NA	): NA

APPEARANCE ?y. ODOR: MILKY WHITE LIQUID FOAM

. 982

82

UNK

SECTION   IV

FLASH POINT(F)(METHOD USED): NONFLAMMABLE
FLAMMABLE LIMITS     LEL NA	UEL NA

FIRE

AND

EXPLOSION

DATA

(CPSC FLAME EXT.

EXTINGUISHING MEDIA SPECIAL FIRE FIGHTING UNUSUAL FIRE HAZARDS

FOAM, C02, DRY CHEMICAL, WATER FOG

WATER SPRAY ON CANS MAY PREVENT BURSTING.

WHEN AEROSOLS ARE EXPOSED TO TEMP. ABOVE 120F. CANS MAY BURS

HEALTH

HAZARD        DATA

SECTION  V

SYMPTOMS

SKIN  :CAN CAUSE IRRITATION UPON PROLONGED CONTACT. EYES  :CAN BE IRRITATING UPON CONTACT.

INHALE: CNS DEPRESSION, ANESTHESIA, RESP. TRACT INFECTION INGEST: CAN CAUSE NAUSEA AND VOMITING.

FIRST AID

SKIN  :WASH WITH SOAP AND WATER.

EYES  :FLUSH IMMEDIATELY WITH PLENTY OF WATER FOR 15 MINUTES. CONSULT PHYSICIAN. INHALE:MOVE TO FRESH AIR. GIVE OXYGEN, IF NEEDED.  SEE DOCTOR. INGEST:GIVE PLENTY OF WATER. DO NOT INDUCE VOMITING. SEEK MEDICAL ATTN.  IMMEDIATELY

TLV: INGR. #i~10QPPM, INGR. #2~UNK, INGR. &3-25PPM (SKIN , INGR. 84-UNK.

SECTION  VI   -REACTIVITY

DATA

STABILITY	: STABLE

INCOMPATIBILITY<AVQID> : STRONG OXID. AGENTS, STRONG ACIDS, BASES, SEL. AMINES POLYMERIZATION         : WILL NOT OCCUR HAZARDOUS DECOMPOSITION: INCOMPLETE COMBUSTION MAY YIELD CARBON MONOXIDE.

SECTION   VII-SPILL

ANDDISPOSALPROCEDURESSTEPS TO BE TAKEN IN CASE MATERAIL IS RELEASED OR SPILLED

LARGE SPILLS ARE UNLIKELY DUE TO AEROSOL PACKAGING.   IF SPILLED, ABSORB WITH AN ABSORBENT MATERIAL (I.E. SELIG'S AP ABSORBENT); PICK UP AND DEPOSIT IN SEAL-ABLE CONTAINER FOR DISPOSAL AS HAZARDOUS WASTE.  THOROUGHLY CLEAN SPILL AREA WITH DETERGENT SOLUTION.  RINSE THOROUGHLY WITH CLEAN WATER.

WASTE DISPOSAL METHOD

PRODUCT IS CONSUMED IN USE.  WHEN DISPOSING OF SPENT CONTAINERS, DO NOT CRUSH, PUNCTURE OR INCINERATE.  SMALL HAZARDOUS WASTE GENERATORS SHOULD CONSULT C. F. R. TITLE 40, PART 261.5 FOR POSSIBLE EXEMPTIONS.  SINCE REGULATIONS VARY CONCERNING THE DISPOSAL OF THESE CHEMICALS, CONSULT LOCAL, STATE AND FEDERAL AGENCIES FOR PROPER DISPOSAL METHOD IN YOUR AREA.

FEDERAL HAZARDOUS WASTE NUMBER(S): F005

SECTION VIII-SPECIAL   PROTECTION   INFORMATION

RESPIRATORY PROTECTION :	NIOSH APROV. CARTRIDGE RESPIRATOR OR MASK IF ABOVE TLV

VENTILATION	:	GENERAL OR MECHANICAL

PROTECTIVE CLOTHING    :	RUBBER OR NEOPRENE GLOVES

EYE PROTECTION         :	SAFETY GOGGLES

SECTION   IX-SPECIAL        PRECAUTIONS

KEEP OUT OF REACH OF CHILDREN.  HARMFUL IF SWALLOWED. AVOID CONTACT WITH SKIN AND EYES. CONTAINER MAY BURST IF HEATED ABOVE 120F. AVOID PROLONGED EXPOSURE TO VAPORS.

THE INFORMATION HEREIN IS GIVEN IN GOOD FAITH BUT NO WARRANTY, EXPRESS OR IMPLIED, IS MADE.