[Gliddenl    The Glidden Company Cleveland, Ohio 44115

MATERIAL SAFETY DATA SHEET

Paints World Leader

FIRE AND EXPLOSION HAZARD DATA

Extinguishing media: Dry chemical or foam water fog. Carbon dioxide.

Unusual fire and explosion hazards: Closed containers may explode when exposed to extreme heat or fire. Vapors are heavier than air and may travel long distances to a source of ignition and flash back. Closed containers may burst if exposed to extreme heat or fire. May decompose under fire conditions emitting irritant and/or toxic gases. In closed tanks, water or foam may cause frothing or eruption.

Special flre fighting procedures : Water may be used to cool and protect exposed containers. Firefighters should use full protective clothing, eye protection, and self-contained breathing apparatus. Self-contained breathing apparatus recommended.

HEALTH HAZARD DATA

Primary route(s) of exposure : Inhalation, skin contact, eye contact, ingestion.

Effects of overexposure:

Inhalation: Irritation of respiratory tract. Prolonged inhalation may lead to mucous membrane

irritation, drowsiness, dizziness and/or lightheadedness, uncoordination, coughing, respiratory

problems, anesthetic effect or narcosis, bronchitis, loss of consciousness. Skin contact: Irritation of skin. Prolonged or repeated contact can cause blistering, severe skin

irritation. Possible sensitization to skin. Eye contact: Irritation of eyes. Prolonged or repeated contact can cause tearing of eyes, redness of

eyes, severe eye irritation. Ingestion: Ingestion may cause drowsiness, dizziness and/or lightheadedness, headache, nausea,

gastro-intestinal disturbances, anesthetic effect or narcosis, loss of consciousness.

Supplemental health information: Contains crystalline silica which is considered a hazard by inhalation. IARC has classified crystalline silica as probably carcinogenic for humans (2a). This classification is based on the findings of laboratory animal studies that were considered sufficient and data from epidemiological studies that were considered limited for carcinogenicity. Crystalline silica is also a known cause of silicosis, a noncancerous lung disease. NTP has classified crystalline silica a reasonably anticipated human carcinogen. Treatment related nasal tumors were observed in rats and mice exposed to vinyl acetate via inhalation at 600 ppm for 2 years.

Medical conditions aggravated by exposure : Eye, skin, respiratory disorders lung disorders

FIRST AID PROCEDURES

Inhalation: Remove to fresh air. Restore and support continued breathing. Get emergency medical attention. Have trained person give oxygen if necessary. Get medical help for any breathing difficulty. Remove to fresh air if inhalation causes eye watering, headaches, dizziness, or other discomfort.

Skin contact: Flush from skin with water. Then wash thoroughly with soap and water. Remove contaminated clothing. Wash contaminated clothing before re-use.

Eye contact: Flush immediately with large amounts of water, especially under lids for at least 15 minutes. If irritation or other effects persist, obtain medical treatment.

Ingestion: If swallowed, obtain medical treatment immediately.

REACTIVITY DATA

Stability: Stable

Incompatibility : Oxidizers, acids, reducing agents, bases, aluminum, zinc, peroxides, magnesium, caustics, mineral acids, sodium, potassium.

Conditions to avoid : Elevated temperatures, contact with oxidizing agent, contact with aluminum or zinc, ultraviolet light, freezing, sparks, open flame.

Hazardous decomposition products : Carbon monoxide, carbon dioxide, acrid fumes, oxygen, aldehydes, acids.

Hazardous polymerization: Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material Is released or spilled : Comply with all applicable health and environmental regulations. Eliminate all sources of ignition. Ventilate area. Evacuate all unnecessary personnel. Place collected material in proper container. Complete personal protective equipment must be used during cleanup. Large spills - shut off leak if safe to do so. Dike and contain spill. Pump to storage or salvage vessels. Use absorbent to pick up excess residue. Keep salvageable material and rinse water out of sewers and water courses. Small spills - use absorbent to pick up residue and dispose of properly.

Waste disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters.

SPECIAL PROTECTION INFORMATION

Respiratory protection: Control environmental concentrations below applicable standards. Where respiratory protection is required, use only NIOSH/MSHA approved respirators in accordance with OSHA standard 29 CFR 1910.134.

Ventilation: Provide dilution ventilation or local exhaust to prevent build-up of vapors.

Personal protective equipment: Eye wash, safety shower, safety glasses or goggles. Impervious gloves, impervious clothing, apron.

SPECIAL PRECAUTIONS

Handling and storage : Store below lOOf. Keep away from heat, sparks and open flame. Keep from freezing.

Other precautions : Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Avoid conditions which result in formation of inhalable particles such as spraying or abrading (sanding) painted surfaces. If such conditions cannot be avoided, use appropriate respiratory protection as directed under special protection information. Ground equipment when transferring to prevent accumulation of static charge.

The information contained herein is based on data available at the time of preparation of this data sheet which The Glidden Company believes to be reliable. However, no warranty is expressed or
implied regarding the accuracy of this data. The Glidden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your
own determination of its suitability and completeness for your own use, for the protection of the environment, and the health and safety of your employees and the users of this material.
Complies with OSHA hazard communication standard 29CFR1910.1200.	prepared 07/05/95	EMERGENCY TELEPHONE NO. (800)545-2643

Physical Data

| Product Code | Description | Wt/Gal. | voc er./ltr. | % Volatile by Volume | Flash Point | Boiling Range | HMIS | DOT, proper shlppine name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| uh 6800 | white | 11.61 | 97.01 | 64.22 | none | 163-477 | *210 | paint,**freezable** |
| uh 6802 | antique white | 11.61 | 96.88 | 64.19 | none | 163-477 | *210 | paint,**freezable** |
| uh 6808 | shell white | 11.61 | 96.85 | 64.17 | none | 163-477 | *210 | paint,**freezable** |
| uh 6810 | white tint base | 11.60 | 97.07 | 64.23 | none | 163-477 | *210 | paint,**freezable** |
| uh6813 | bone white | 11.62 | 96.71 | 64.15 | none | 163-477 | *210 | paint,**freezable** |
| uh 6821 | navaio white | 11.62 | 96.79 | 64.15 | none | 163-477 | *210 | paint,**£reezable** |
| uh 6825 | white-high hiding | 11.61 | 97.02 | 64.22 | none | 163-477 | *210 | paint, **freezable** |
| uh 6880 | deep tint base | 9.69 | 68.29 | 67.98 | none | 212-477 | *110 | paint,**freezable** |
| uh 6887 | intermediate tint base | 10.41 | 70.21 | 65.73 | none | 212-477 | *110 | paint,**freezable** |

Ingredients

Product Codes with % by Weight

|  | uh 6800 | uh 6802 | uh 6808 | uh 6810 | uh 6813 | uh 6821 | uh 6825 | uh 6880 | uh 6887 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| acetic acid ethenvl ester | .1-1.0 | .1-1.0 | .1-1.0 | .1-1.0 | .1-1.0 | .1-1.0 | .1-1.0 |  |  |
| limestone | 5-10 | 5-10 | 5-10 | 5-10 | 5-10 | 5-10 | 5-10 |  |  |
| kaolin | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |  |  |
| ceramic materials and wares, chemicals | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |  |  |
| propanoic acid, 2-methyl-, monoester with 2,2,4-trimethvl-l ,3-pentanediol | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 | 1-5 |
| quartz |  |  |  |  |  |  |  | 10-20 | 10-20 |
| titanium oxide | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 1-5 | 10-20 |
| vinyl acrylic copolymer |  |  |  |  |  |  |  | 10-20 | 10-20 |
| 2-propenoic acid, butyl ester, polymer with ethenvl acetate | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 |  |  |
| 2-propenoic acid, 2-methyl-, methyl ester, polymer with butyl 2-propenoate and ethenvlbenzene |  |  |  |  |  |  |  | 1-5 | 1-5 |
| water | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 | 10-20 |  |  |
| water | 30-40 | 30-40 | 30-40 | 30-40 | 30-40 | 30-40 | 30-40 | 50-60 | 50-60 |

Chemical Hazard Data

|  |  |  | ACGIH-TLV |  | OSHA-PEL |  |  |  | S.R. STD. | S 2 | S 3 | C C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHEMICAL NAME | COMMON NAME | CAS. NO. | 8-HOUR TWA | STEL | 8-HOUR TWA | STEL | C | S |  |  |  |  | N | I | 0 |
| acetic acid ethenyl ester | vinyl acetate | 108-05-4 | lOppm | 20ppm | lOppm | 20ppm | ne | ne | 5ppm | V | V | V | n | n | n |
| limestone | limestone | 1317-65-3 | 10 mg/m3 | ne | 5 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| kaolin | clay | 1332-58-7 | 10mg/m3 | ne | 5 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| ceramic materials and wares, chemicals | calcined kaolin | 66402-68-4 | 5 mg/m3 | ne | 10 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| propanoic acid, 2-methyl-, monoester with 2,2,4-trimethvl-1.3-pentanediol | texanol | 25265-77-4 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| quartz | quartz | 14808-60-7 | 0.1 mg/m3 | ne | 0.1 mg/m3 | ne | ne | ne | ne | n | n | n | Y | y | n |
| titanium oxide | titanium dioxide | 13463-67-7 | 10 mg/m3 | ne | 10 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| vinyl acrylic copolymer | vinyl acrylic copolymer | sup. conf. | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| 2-propenoic acid, butyl ester, polymer with ethenvl acetate | butyl acrylate-vinyl acetate polymer | 25067-01-0 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| 2-propenoic acid, 2-methyl-, methyl ester, polymer with butyl 2-propenoate and ethenylbenzene | butyl acrylate-methyl methacrylate-styrene cop | 27136-15-8 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| water | water | 7732-18-5 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| water | water | 7732-18-5 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |

Footnotes:

C = Ceiling - Concentration that should not be

exceeded, even instantaneously.

S = Skin - Additional exposure, over and above airborne exposure, may result from skin

S.R.STD. = Supplier Recommended Standard n/a = not applicable ne = not established

ppm = parts per million

mg/m3 = milligrams per cubic meter

S2 = Sara Section 302 EHS

S3 = Sara Section 313 Chemical CC = CERCLA Chemical

Carcinogenicity Listed By:

N = NTP, I = IARC, O = OSHA

y = yes, n = no

Form: A16 , Page 3, prepared 07/05/95