Suburban Propane

FOR YOUR

MATERIAL SAFETY DATA SHEET   INFORMATION

AND FILES

EFFECTIVE AUGUST,   1991

| Suburban ProDane p.o..' box w®iiiss Whippany, NJ 07981 | TRANSPORTATION CHEMTREC NO. GENERAL EMERGENCY NO.: (800) 424-9300 INFORMATION: |  | SAFETY DEPT. (201} 887-5300 |
| --- | --- | --- | --- |
|  |  | HEALTH S\ HAZARD^ / a \ yx O X 4 - Severe 3 - Serious 2 - Moderate | v FIRE ^V HAZARD 0\ S/N REACTIVITY 1 - Slight 0 - Minimal |
| DANGER! Extremely flammable liquefied gas under pressure. Keep away from heat, sparks, flame, and all other ignition sources. Vapor reduces oxygen available for breathing and may cause suffocation in confined spaces. Use only with adequate ventilation. Odor may not provide adequate warning of potentially hazardous concentrations. Vapor is heavier than air and may collect at low levels. Liquid may cause freeze burn similar to frostbite. Do not get liquid in eyes, on skin, or on clothing. Avoid prolonged breathing of vapor. Keep container valve closed when not in use. |  |  |  |
| SECTION 1 - IDENTIFICATION |  |  |  |

PRODUCT:      Commercial Propane

SYNONYMS:   Liquefied Petroleum Gas;   LP-Gas; LPG

CHEMICAL  FAMILY: CHEMICAL  FORMULA:

Paraffinic Hydrocarbon C3H8

SECTION II - INGREDIENTS

| MATERIAL | CAS NUMBER | PERCENT |
| --- | --- | --- |
| ETHANE | 74-84-0 | 0-5.0 |
| PROPANE | 74-98-6 | 87.5-100 |
| PROPYLENE | 115-07-1 | 0-5.0 |
| BUTANES | Various | 0-2.5 |
| ETHYL MERCAPTAN | 75-08-01 | 0-50 ppm |
| SECTION III - HEALTH INFORMATION |  |  |

| INHALATION: | Asphyxiant in high concentrations due to dilution of available oxygen. At excessive vapor concentrations, this product has anesthetic, asphyxiating properties and may cause sleepiness. At levels above 100,000 ppm (i.e. 10%), propane is mildly irritating to the respiratory tract and may result in dizziness, headache, drowsiness, nausea, shortness of breath, muscular incoordination, excessive salivation, disorientation, vomiting, and excitation. In extreme cases, convulsions, unconsciousness and death may occur as a result of asphyxiation. Persons with chronic respiratory disease should avoid exposure.

IINGESTION: | Liquid may cause freeze burn similar to frostbite. Ingestion not expected to occur in normal use.

[EYE CONTACT: || jguiH may cause freeze burn similar to frostbite.

ISKIN CONTACT:!         Liquid may cause  freeze burn similar to frosbite.

|OTHER:| Product is not listed by IARC, NTP or OSHA as a potential carcinogen. Propane and some of the minor components have been reported to be cardiac sensitizers in experiments.

| SECTION IV - OCCUPATIONAL EXPOSURE LIMITS |  |  |
| --- | --- | --- |
| MATERIAL | PEL/TWA | TLV/TWA |
| ETHANE | Not Established | Simple Asphyxiant |
| PROPANE | 1000 ppm | Simple Asphyxiant |
| PROPYLENE | Not Established | Simple Asphyxiant |
| BUTANES | 800 ppm | 800 ppm |

SECTION V -  EMERGENCY AND FIRST AID PROCEDURE

FOR OVEREXPOSURE BY:

ieye contact:!

|INHALATION:|	Remove  victim  from  further

exposure and into fresh air. Provide oxygen if breathing is difficult If victim is unconscious, get prompt medical attention.

immediately    with medical attention.

For contact with liquid, flush water.         Obtain    immediate

|INGESTION:|     |f swallowed, get immediate medical attention.

[SKIN CONTACT:] If freeze burn occurs, remove contaminated clothes, shoes and jewlery. Immerse burned area in warm (not hot) water. Keep immersed. Get prompt attention.

SECTION VI ^PHYSICAL DATA

BOILING POINT:

MELTING POINT:

VAPOR PRESSURE:

SPECIFIC GRAVITY (H20 =  1):

VAPOR DENSITY (AIR =   1):

SOLUBILITY IN WATER:

APPEARANCE AND ODOR:

ODORANT WARNING

-44 F

N/A

196 psig @  100 F

0.504

1.50

Slight, 0.1 to  1.0%

Colorless, odorless in natural form.

Odorant is added to aid in detection of leaks. One common odorant is ethyl mercaptan, CAS No. 75-08-1. Odorant has a foul, skunk like odor. The odorant is effective in most instances, but not everyone can smell the odor. The ability of people to detect odors varies widely. Also, certain chemical reactions with material in the propane system can reduce the propane odor level. No odorant will be 100% effective in all circumstances. If odor level appears to be weak, notify propane supplier immediately.

SECTION VII  -  FIRE AND EXPLOSION HAZARDS

FLASH POINT & METHOD USED: IGNITION TEMPERATURE IN AIR: FLAMMABLE LIMITS IN AIR, % BY VOLUME:

NFPA RATING (Under Fire Conditions. Does not apply to exposure hazards other than during fire):      HEALTH:

FIRE: REACTIVITY:

-156 F (estimated)

920 -  1,120 F

LOWER: 2.2%         UPPER: 9.6%

1  Slightly toxic

4 Extremely flammable

0 Stable

FIRE FIGHTING PROCEDURES:

Eliminate sources of ignition. Evacuate area Notify fire department Allow only trained, properly protected personnel in area Shut off source of gas, if possible. Allow fire to burn itself out after gas flow is shut off. If gas flow cannot be shut off, do not extinguish fire. Allow fire to burn itself out using high volume water supply to cool heat-exposed pressure containers and nearby equipment Approach a flame enveloped container from the side, never the head ends. Use extreme caution when applying water to a container which has been exposed to heat or flame for more than a short time. For uncontrollable fires and when flame is impinging on container, withdraw all personnel and evacuate vicinity immediately.

USUAL FIRE & EXPLOSION HAZARDS:

Firefighters should wear self-contained breathing apparatus in the positive pressure mode with a full facepiece when there is a possibility of exposure to smoke, fumes or hazardous decomposition products. Uncontrolled vapors spread rapidly, are heavier than air and are extremely flammable.

SECTION VIII - REACTIVITY

STABILITY:	Stable

HAZARDOUS POLYMERIZATION:	Will not occur

CONDITIONS & MATERIALS TO AVOID:	Keep away from high heat, sparks, open flame,

strong oxidizing agents.

HAZARDOUS DECOMPOSITION PRODUCTS:	Incomplete combustion may yield carbon monoxide,

a toxic gas.

^•£^£:^^j$i&!^^ - EMPLOYEE PROTECTION

CONTROL MEASURES:

Use local and dilution ventilation to maintain exposures below acceptable criteria.

RESPIRATORY PROTECTION:

If concentrations are high enough to warrant supplied-air or self-contained breathing apparatus, atmosphere may be flammable (see Section VII). Appropriate precautions must be taken regarding flammability. For situations where flammability has been safely addressed and where control measures are not feasible or sufficient to achieve full conformance with acceptable criteria (Section IV), use NIOSH/MSHA approved respiratory protection (supplied-air or self-contained breathing apparatus as appropriate). Respirators should be selected based on form and concentration of contaminant in air and in accordance with OSHA (29 CFR 1910.134).

PROTECTIVE CLOTHING

Avoid skin contact with liquid because of possibility of freeze burn. Wear gloves and protective clothing which are impervious to the product for the duration of the anticipated exposure.

EYE PROTECTION:

Use chemical safety goggles meeting the specifications of ANSI Z87.1 when connecting or disconnecting lines under pressure. Spectacle type safety glasses do not provide satisfactory protection.

SECTION X -  ENVIRONMENTAL PROTECTION

ENVIRONMENTAL EFFECTS:

Avoid uncontrolled releases of this material. Liquid release will have possible effect on plant and animal life. Large liquid release will quickly vaporize to produce a large, vapor cloud. Vapor cloud is both a fire and asphyxiation hazard.

SPILL OR LEAK PROCEDURES:

Product is extremely flammable. Vapor is heavier than air and may collect at lower levels. Flammable concentrations may be present below nose level. If there is a leak but no fire, do not ignite the escaped gas. Eliminate all ignition sources. Do not smoke, use a nearby phone or turn electrical switches on and off. Evacuate area If possible, remove leaking container to safe area. Stop flow of gas or allow vapor to disperse in a safe area. Water spray can be used to help dilute vapor concentration in air.

WASTE DISPOSAL:

Dispose of gas in accordance with applicable laws and regulations. Vent vapor in a safe location and insure that gas dissipates below the lower flammable limit Controlled burning is preferred.

SECTION XI - REGULATORY INFORMATION

DOT PROPER SHIPPING NAME:	Liquefied Petroleum Gas

DOT HAZARD CLASS:	Flammable Gas

DOT I.D. NUMBER:	UN  1075

DOT EMERGENCY RESPONSE GUIDE:	See Guide No. 22

SARA TITLE III  INFORMATION:

This  product  may  contain  over   1.0%  propylene.     This  is  subject to  the  reporting  requirements  of Section 313.

HAZARD CATEGORY FOR SECTION 311/312 REPORTING:

Immediate (acute) health hazard.     Fire hazard.     Sudden release of pressure hazard

RCRA INFORMATION:

This product, when disposed of by incineration or flaring, is defined as an ingnitable hazardous waste in Federal regulations. Hazardous waste number is D001. Refer to latest Federal or State regulations regarding proper means of disposal.

TSCA STATUS:

All components of this product are listed on the TSCA inventory.

SECTION XIIv:l HANDLING AND STORAGE PRECAUTIONS

Store in an authorized location (outside, detached storage is preferred with adequate ventilation. Isolate from heat and ignition sources. Isolate from combustible materials. Provide separate storage locations for other compressed or flammable gases. Inspect cylinders frequently for leaks, dents, gouges and corrosion with emphasis on bottom of cylinder. Store cylinders in upright position or with pressure relief valves in vapor space. Do not drop or abuse cylinders. Keep container valve closed and plugged when not in use. Install protective caps when cylinders are not connected for use. Empty containers retain some residue, so they should be treated as if they wete full.

The information presented herein is believed to be factual as it has been derived from the works and opinions of persons believed to be qualified experts; however, nothing contained in this information is to be taken as a warranty or representation for which the company bears legal responsibility. The user should review any recommendations in the specific context of the intended use to determine whether they are appropriate.

PREPARED BY:     Regulatory Department	ISSUED:	08/91

P.O. BOX206	SUPERSEDES:      05/90

Whippany, NJ    07981