938 P10         MflR 27   '02     13:57

MZ7

MATERIAL SAFETY DATA SHEET

Radiator Specialty Company

1900 WILKINSON BLVD. CHARLOTTE, NC 28208 (704) 377-6555

POISON INFORMATION & EMERGENCY: 303-623-5716

MATERIAL SAFETY DATA SHEET

May be uaed to comply with OSHA's Hazard Communication Standard 29 CFR 1910.1200. Standard must be consulted for specific requirements.

SECTION I GENERAL INFORMATION

US DEPARTMENT OF LABOR

Occupational Safety and Health Administration. (Non-Mandatory Form) Form Approved OMB No. 1218-0072

PRODUCT NAME  POWER STEERING FLUID I PART NUMBER    M 27 -12, -13, - 32, - 34, - 40, - 49

NOTE: Blank spaces are not permitted. If any item Is not applicable or no information is available, the space must be marked to indicate that.

SECTION II HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

| COMPONENT | WT% C.A.5. NO. | TLV (ACG1H OSHA- -) |
| --- | --- | --- |
| Petroleum Oil | 90 - 99 64742-52-5 / 64742-53-6 | 5 mg/m3 |
| Zinc Compounds* | 0.1 - 0.3 Confidential |  |
| Comments: THIS CHEMICAL | IS SUBJECT TO REPORTING REQUIREMENTS OF | SECTION 313, SARA TITLE 111 |
| Components not identified are non-hazardous according to 29 CFR 1910.1200 |  |  |

SECTION III PHYSICAL/CHEMICAL CHARACTERISTICS

Specific Gravity (H2O=1)   0.89-0,90

Appearance and Odor   Amber oily liquid with petroleum odor.

Solubility in Water      Insoluble

Flash Point (Method) - F°    320T ( COC)

Melting Point - F°  0°F

Vapor Pressure (mmHg)   0,008

Evaporation Rate (Butyl Acetate=1)    Very Slow

pH  Neutral

Solubility in Solvent    Petroleum % Volatiles By Wt.   Negligible Boiling Point - F°   465°F Vapor Density (Air=1)    4.3

SECTION IV FIRE AND EXPLOSION HAZARD DATA

EXTINGUISHING MEDIA:

Water Fog  X	Foam   X

CO2

Dry Chemical  X

SPECIAL FIRE FIGHTING PROCEDURES: protective clothes

Wear self-contained, positive pressure breathing apparatus and

UNUSUAL FIRE AND EXPLOSION HAZARDS: Do not use forced stream as this could cause the fire to spread.

938 Pll         MflR 27   '02     13:57

M27

SECTION V REACTIVITY DATA

Stable   X

Unstable

Corrosive   No

Hazardous Polymerization? Yes        No X

Incompatibilities   oxidizing materials

Hazardous Decomposition or Byproducts  Smoke, carbon oxides, traces of sulfur, phosphorus, and nitrogen oxides.

SECTION VI HEALTH HAZARD INFORMATION

Recommended TLV of Product    5mg/m3 as oil mist

EYE CONTACT  Mild irritant

SKIN CONTACT Mild irritant on long exposure

INHALATION If oil mist is Inhaled for prolonged periods,         INGESTION  HARMFUL IF SWALLOWED ! may cause irritation

OTHER   Contains traces of zinc compounds which may have adverse reproductive effects.

SECTION VII EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT | Flush eyes with water for 1 5 | minutes while lifting eyelids. Consult a physician. |
| --- | --- | --- |
| SKIN CONTACT | Wash with soap and water |  |
| INHALATION | Move to fresh air, if adverse | effects persist consult physician. |
| INGESTION DO NOT INDUCE VOMITING, |  | consult physician immediately. |

SECTION VIII SPECIAL PROTECTION INFORMATION

|  | CONSUMER | BULK HANDLING (Prolonged (Exposure) |
| --- | --- | --- |
| RESPIRATORY PROTECTION | N/A | If TLV is exceeded, wear respirator |
| VENTILATION | Use with adequate ventilation | usb with adequate ventilation |
| EYE PROTECTION | N/A | Goggles or Full Face Shield Recommended |
| PROTECTIVE CLOTHING | N/A | Neoprene gloves and apron |

SECTION IX PRECAUTIONS FOR SAFE HANDLING AND USE

SPILL OR LEAK PROCEDURE :   Contain spill, remove Ignition sources, ventilate area, wipe up with rags or absorbent and transfer to a waste drums.

WASTE DISPOSAL METHOD : Dispose of in accordance with all applicable government laws and regulations.

STORAGE AND HANDLING PRECAUTIONS:      Store away from oxidizing agents.

OTHER PRECAUTIONS:      KEEP AWAY FROM CHILDREN AND ANIMALS.

HAZARD INFORMATION LABEL DATA

Supersedes   JUNE 1997

JREWJ1MTY

4- Extreme 8 = High

1-0 "

ffiWH

8PBOUL

OSHA Revised   DEC-1998 Tltlo: R. GEER . CHEMIST

While Radiator Spectefly Company be»v»a this ctefa is accurate as of th» revision eW», we makt rto warranty wffft respect to the data and v«) expressly disclaim til UabWy for reliance thefecm. The data /» offeree/ solely tor /hfomtedon. inwttlgaUon, and vervtcatton.