National Welders

SAFETY DATA SHEET

P.O. Box 31007

Charlotte, NC 28231, 704-333-5475

Emergency Telaphone: 800-366-4422

N001 4002

ChemTrec: 800-424-9300

issued:    July 1994

MAJOR SUPPLIERS OF CRYOGENICS AND WELDING EQUIPMENT

| SECTION 1. MATERIAL IDENTIFICATION |  |  |
| --- | --- | --- |
| PRODUCT NAME | CAS *> | NATIONAL FIR6 PROTECTION ASSOCIATION CODE TCM HMIS / ™ \_ /**. /\ H: 0 / \/\ p. Q <( MULTH ^^RMCTIWY^ R- 0 X/SMCW1. >/ S: (OX) \^/ |
| Oxygen | 7782-44-7 . |  |
| TRADE NAME AND SYNONYMS | DOT I.D. NO. |  |
| See Page 4 | UN 1072 |  |
| CHEMICAL NAME AND SYNONYMS... | DOT HAZARD CLASS |  |
| Oxygen | Division 2.2 |  |
| FORMULA CHEMICAL FAMILY | DESCRIPTION |  |
| 02 Oxidizer | Oxidant; Vital element. |  |

SECTION 2. HEALTH HAZARD INFORMATION

TIME WEIGHTED AVERAGE EXPOSURE LIMIT

None established (ACGIH 1993-1994). Oxygen is the "vital element" in the atmosphere in which we live and breathe (approximately 21 molar % of the atmosphere). OSHA 1993 does not list a TWA for oxygen.

SYMPTOMS OF EXPOSURE

Breathing high concentrations (greater than 75 molar percent) causes symptoms of hyperoxia which includes cramps, nausea, dizziness, hypothermia, amblyopia, respiratory difficulties, bradycardia, fainting spells, and convulsions capable of leading to death.

For additional information on hyperoxia, see Compressed Gas Association's Pamphlet P-14.

TOXJCOLOGICAL PROPERTIES

The property is that of hyperoxia which leads to pneumonia. Concentrations between 25 and 75 molar percent present a risk of inflammation of organic matter in the body.

Oxygen is not listed in the IARC, NTP or by OSHA as a carcinogen or potential
carcinogen.	(Continued on Page 4)

RECOMMENDED FIRST AID TREATMENT

PROMPT MEDICAL ATTENTION IS MANDATORY IN ALL CASES OF OVEREXPOSURE TO OXYGEN. RESCUE PERSONNEL SHOULD BE COGNIZANT OF EXTREME FIRE HAZARD ASSOCIATED WITH OXYGEN-RICH ATMOSPHERES.

(Continued on Page 4)

Emergency Telephone: 800-888-4422

ChemTrec: 800-424-9300

Page ' of 4

N0014002

Oxygen

| SECT JON 3, PHYSICAL DATA |  |
| --- | --- |
| BOJUNQ POINT | LIQUID DENSITY AT BOILING POINT |
| -297. 3°F (-182. 9°C) | 71.23.1b/ft-* (1141 kg/rn^) |
| VAPOR PRESSURE © 70 °F U1.1°C) = AbOV6 the | GAS DENSITY AT 70 "F. 1 fc£ft |
| critical temp, of -181. 1°F (-118. 4°C) | 0.828 Ib/ft-* (1.326 kg/m3) |
| •SOLUBILITY IN WATER | FREEZING POINT |
| Slightly | -361. 8°F (-218. 8°C) |
| EVAPORATION RATE | SPECIFIC GRAVITY (AIR « 1J |
| N/A (Gas) | @ 70°F (21.1°C) = 1.11 |
| APPEARANCE AND ODOR f , , nrfnrlp-<. m" |  |
| SECTION 4. FIRE AND EXPLOSION HAZARD DATA |  |
| FLASH POINT AUTO IGNiTiON TEMPERATURE | FLAMMABLE UNITS % BY VOLUME |
| N/A N/A | LEL N/A UEL N/A |
| extinguishing media r-Qnious niiantiti*1? of water | ELECTRICAL CLASSIFICATION |
| Lfor fires with oxyqen as the oxidizer | Nonhazardous |
| SPECIAL F1REF1GHTING PROCEDURES T-F ,,«<^,-Kl« | UNUSUAL FIRE AND EXPLOSION HAZARDS |
| stop the flow of oxygen which is supporting the fire. | See Page 4 |
| HAZARDOUS MIXTURES OF OTHER LIQUIDS, SOLIDS OR GASES | Dntact with all flammable materials not flammable in air will burn in pure |
| Oxygen vigorously accelerates combustion. C should be avoided. Some materials which are oxygen or oxygen-enriched atmospheres. |  |
| ["section s. reactivity data |  |
| STABILITY CONDITIONS TO AVOID | HAZARDOUS POLYMERIZATION CONDITIONS TO AVOID |
| Unstable D Stable £) None | May Occur D Will Not Occur fa None |
| INCOMPATIBILITY {Materials to Avoid) HAZARDOUS DECOMPOSITION PRODUCTS |  |
| All flammable materials None |  |
| SECTION 3. SP5LL, LEAK AND DISPOSAL PROCEDURES |  |
| STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED |  |
| Evacuate all personnel from affected area. Use appropriate protective equipment. If leak is in user's equipment, be certain to purge piping with an inert gas prior to attempting repairs. If leak is in container or container valve, contact your closest supplier location or call the emergency telephone number listed herein. |  |
| WASTE DISPOSAL | quantities. Return in the shipping con-et plugs or caps secured and valve pro- |
| Do not attempt to dispose of waste or unused tainer properly labeled, with any valve out! |  |
| tection cap in place to your supplier. For | emergency disposal assistance, contact mergency telephone number listed herein. |
| your closest supplier location or call the e |  |
| SECT3ON 7. SPECIAL PROTECTION INFORMATION |  |
| RESPIRATORY PROTECTION; |  |
| " ' ' ~ ' N/A |  |
| VENTILATION ¦ OTAI FXHAUST T a<~<- 1 | SPECIAL N/A |
| • - io prevent accumu la-See Local tion above 25 molar percent. |  |
| E^hiU^t MECHANICAL N/A | OTHER N/A |
| PROTECTIVE GLOVES As required, any material | EYE PROTECTION Safety goggles or glasses |
| OTHER PROTECTIVE EQUIPMENT Safety shoes, safety shower |  |

Oxygen		NOQ14002

SECTION 8.  SPECIAL PRECAUTIONS AND COMMENTS

SPECIAL LABELING INFORMATION

DOT Shipping Name:  Oxygen, Compressed    DOT Hazard Class: Division 2.2

DOT Shipping Label: Nonflammable gas; Oxidizer; OR: Oxygen (2) I.D. No.: UN 1072

SPECIAL HANDLING RECOMMENDATIONS

Use only in wel1-ventilated areas. Valve protection caps and valve outlet threaded plugs must remain in place unless container is secured with valve outlet piped to use point. Do not drag, slide or roll cylinders. Use a suitable hand truck for cylinder movement. Use a pressure reducing regulator when connecting cylinder to lower pressure (<3,000 psig) piping or systems. Do not heat cylinder by any means to increase the discharge rate of product from the cylinder. Use a check valve or trap in the discharge line to prevent hazardous back flow:into the cylinder.

For additional handling recommendations, consult Compressed Gas Association's Pamphlets P-l, P-14, and G-4.

SPECIAL STORAGE RECOMMENDATIONS

Protect cylinders from physical damage. Store in cool, dry, well-ventilated area away from heavily trafficked areas and emergency exits and away from full or empty stored cylinders which contain flammable products. Do not allow the temperature where cylinders are stored to exceed 125F (52C). Cylinders should be stored upright and firmly secured to prevent falling or being knocked over. Full and empty cylinders should be segregated. Use a "first in - first out" inventory system to prevent full cylinders being stored for excessive periods of time.

For additional storage recommendations, consult Compressed Gas Association's Pamphlets P-l, P-14, and G-4.

SPECIAL PACKAGING RECOMMENDATIONS

Carbon steels and low alloy steels are acceptable for use at lower pressures. For high pressure applications use stainless steels, copper and its alloys, nickel and its alloys, brass, bronze, silicon alloys, Monel®, Inconel®, or beryllium.

(Continued on Page 4)

OTHER RECOMMENDATIONS OR PRECAUTIONS

Oxygen should not be used as a substitute for compressed air in pneumatic equipment since this type generally contains flammable lubricants. Equipment to contain oxygen must be "cleaned for oxygen service." See Compressed Gas Association Pamphlet G-4.1.

(Continued on Page 4)

SPECIAL NOTES:

Reporting under SARA, Title III, Section 31-3 not required.

About the Information in this Bulletin:

'Various Government agencies (i.e., Department of Transportation. Occupational Safety and Health Administration Food and Drug Administration and Others) may have specific regulations concerning the transportation, handling, storage or use of this product which will not be reflected • n this data sheet- The customer should review these regulations to ensure that he is ;n lull compliance.

;r'ormation contained in this material safety aata sheet is offered without charge 'or use by technically qualified personnel a! '.heir discretion and risk. All statements. ;ecm!cal information ana recommendations contained hersin are based on tests and data which we believe to be reliable, out the accuracy or completeness (hereof is not guaranteed and no warranty of any kind rs made with respect thereto This information is not tended as a license to operate under or a recommendation to practice or infringe any patent of this Company or others covering any process, composition of matter or use.

Since the Company shall have no control of the use of the product described herein, the Company assumes no liability for loss or damage incurred
from the proper or improper use of such product.	* «•

Pace 3 of 1

Oxyqen

Jy	N0014002

SPECIAL NOTES:

MATERIAL IDENTIFICATION

TRADE NAME AND SYNONYMS: Oxygen; Oxygen, compressed (D.O.T.)

HEALTH HAZARD INFORMATION TOXICQLOGICAL PROPERTIES:  (Continued)

Persons in ill health where such illness would be aggravated by exposure to oxygen should not be allowed to work with or handle this product.

RECOMMENDED FIRST AID TREATMENT:  (Continued)

Conscious persons should be assisted to an uncontaminated area and breathe fresh air. They should be kept warm and quiet. The physician should be informed that the victim is experiencing (has experienced) hyperoxia.

Unconscious persons should be moved to an uncontaminated area and given assisted respiration. When breathing has been restored, treatment should be as above. Continued treatment should be symptomatic and supportive.

SPECIAL PRECAUTIONS

SPECIAL PACKAGING RECOMMENDATIONS:  (Continued)

Lead and silver or lead and tin alloys are good gasketing materials. Teflon® and Kel-F® are the preferred nonmetal gaskets.

Special Note: It.should be recognized that the ignition temperature of metals and non-metals in pure oxygen service decreases with increasing oxygen pressure.

OTHER RECOMMENDATIONS OR PRECAUTIONS:  (Continued)

Compressed gas cylinders should not be refilled except by qualified producers of compressed gases. Shipment of a compressed gas cylinder which has not been filled by the owner or with his (written) consent is a violation of Federal Law (49CFR).

Always secure cylinders in an upright position before transporting them. NEVER transport cylinders in trunks of vehicles, enclosed vans, truck cabs or in passenger compartments. Transport cylinders secured in open flatbed or in open pick-up type vehicles.

FIRE AND EXPLOSION HAZARD DATA UNUSUAL FIRE AND EXPLOSION HAZARDS:

Vigorously accelerates combustion. If cylinders are involved in a fire, safely relocate or keep cool with water spray.

Paae 4 of 4