Monsanto's Roundup Herbicide MSDS Effective 25jan01

s

Roundup MSDS Effective 25jan01 [ Strongly suggested reading on Roundup ]

MONSANTO Company Pages: 1-10 ROUNDUP ORIGINAL[TM] Herbicide Version: 1.2 Effective date: 01/25/2001

Page 1 of 12

MONSANTO Company

Material Safety Data Sheet Commercial Product

1.	PRODUCT AND COMPANY IDENTIFICATION

Product name

ROUNDUP ORIGINAL[TM] Herbicide

EPA Reg. No.

524-445 Product use

Herbicide Chemical name

Not applicable Synonyms

None Company

MONSANTO Company, 800 N. Lindbergh Blvd., St. Louis, MO, 63167 Telephone: 800-332-3111, Fax: 314-694-5557

Emergency numbers

FOR CHEMICAL EMERGENCY, SPILL LEAK, FIRE, EXPOSURE, OR ACCIDENT

Call CHEMTREC- Day or Night 1-800-424-9300 toll free in the continental U.S., Puerto Rico, Canada, or Virgin Islands. For calls originating elsewhere: 703-527-3887 (collect calls accepted). FOR MEDICAL EMERGENCY - Day or Night: 314-694-4000 (collect calls accepted).

2.	COMPOSITION/INFORMATION ON INGREDIENTS

Active ingredient

Isopropylamine salt of N-(phosphonomethyl)glycine: (Isopropylamine salt of glyphosate

Composition

COMPONENT

Isopropylamine salt of glyphosate

Surfactant

Water

CAS No.

38641-94-0 61791-26-2 7732-18-5

% by weight (approxima

41

8

51

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25jan01.htm

10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25janO 1	Page 2 of 12

i Trade secret composition.

OSHA Status

This product is hazardous according to the OSHA Hazard Communication Standard, 29 CFR 1910.1200.

3.	HAZARDS IDENTIFICATION
Emergency overview

Appearance and odour (colour/form/odour): / Liquid / Slight

WARNING!

Keep out of reach of children.

CAUSES SUBSTANTIAL BUT TEMPORARY EYE INJURY

HARMFUL IF SWALLOWED

HARMFUL IF INHALED

REFORMULATION IS PROHIBITED

SEE INDIVIDUAL CONTAINER LABEL FOR REPACKAGING LIMITATIONS

Potential health effects

Likely routes of exposure

Skin contact, eye contact Eye contact short term

Causes temporary eye irritation. Skin contact short term

Not expected to produce significant adverse effects when recommended use instructions are followed. Inhalation, short term

Not expected to produce significant adverse effects when recommended use instructions are followed.

Refer to section 11 for toxicological and section 12 for environmental information

4.	FIRST AID MEASURES
Eve contact

Immediately flush with plenty of water.

Continue for at least 15 minutes.

If easy to do, remove contact lenses.

If there are persistent symptoms, obtain medical advice.

Skin contact

Wash affected skin with plenty of water.

Wash clothes before re-use.

Take off contaminated clothing, wristwatch, jewellery.

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25janO 1 .htm10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 3 of 12

• Inhalation

Remove to fresh air. Ingestion

Immediately offer water to drink.

Do NOT induce vomiting unless directed by medical personnel.

If symptoms occur, get medical attention.

Advice to doctors

This product is not an inhibitor of cholinesterase. Antidote

Treatment with atropine and oximes is not indicated.

5.	FIRE FIGHTING MEASURES
Flash point

Does not flash. Extinguishing media

Recommended: Water, dry chemical, carbon dioxide (C02), foam Unusual fire and explosion hazards

None.

Environmental precautions: see section 6.

Hazardous products of combustion

Carbon monoxide (CO), nitrogen oxides (NOx), phosphorus oxides (PxOy) Fire fighting equipment

Self-contained breathing apparatus.

Equipment should be thoroughly decontaminated after use.

6.	ACCIDENTAL RELEASE MEASURES
Personal precautions

Use personal protection recommended in section 8.

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25jan01 .htm	10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 4 of 12

t Environmental precautions

SMALL QUANTITIES:

Low environmental hazard.

LARGE QUANTITIES:

Minimize spread.

Keep out of drains, sewers, ditches and water ways.

Methods for cleaning up

SMALL QUANTITIES:

Flush spill area with water.

LARGE QUANTITIES:

Absorb in earth, sand or absorbent material.

Dig up heavily contaminated soil.

Collect in containers for disposal.

Refer to section 7 for types of containers.

Flush residues with small quantities of water.

Minimize use of water to prevent environmental contamination.

Refer to section 13 for disposal of spilled material.

7. HANDLING AND STORAGE

Handling

Good industrial practice in housekeeping and personal hygiene should be followed.

Avoid contact with skin and eyes.

When using do not eat, drink or smoke.

Wash hands thoroughly after handling or contact.

Thoroughly clean equipment after use.

Emptied containers retain vapour and product residue.

Observe all labelled safeguards until container is cleaned, reconditioned or destroyed.

Emptied containers retain vapour and product residue.

FOLLOW LABELED WARNINGS EVEN AFTER CONTAINER IS EMPTIED.

Storage

Compatible materials for storage: stainless steel, aluminium, plastic, fiberglass, glass

lining

Incompatible materials for storage: galvanized steel, unlined mild steel, see section 10.

Keep out of reach of children.

Keep away from food, drink and animal feed.

Keep only in the original container.

Partial crystallization may occur on prolonged storage below the minimum storage

temperature.

If frozen, place in warm room and shake frequently to put back into solution.

Minimum shelf life: 5 years.

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25janO 1 .htm	10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 5 of 12

8.	EXPOSURE CONTROLS/PERSONAL PROTECTION
Airborne Exposure Limits

Components		Exposure  Guidelines

Isopropylamine   salt   of  glyphosate	No   specific  occupational	expos

Surfactant	No   specific  occupational	expos

Water	No   specific  occupational	expos

Engineering controls

Have eye wash facilities immediately available at locations where eye contact can occur. Eye protection

If there is potential for contact: Wear chemical goggles.

Skin protection If repeated or prolonged contact:

Wear chemical resistant gloves. Respiratory protection

No special requirement when used as recommended.

When recommended, consult manufacturer of personal protective equipment for the appropriate type of equipment for a given application.

9.	PHYSICAL AND CHEMICAL PROPERTIES

These physical data are typical values based on material tested but may vary from sample to sample. Typical values should not be construed as a guaranteed analysis of any specific lot or as specifications for the product.

Form:	Liquid

Odour:	Slight

Flash point:	Does not flash.

Specific gravity:	1.1655 @ 20°C/15.6°C

pH:	4.4-5.0

Partition  coefficient	(log   Pow):      <   0.000    (active   ingredient)

10.	STABILITY AND REACTIVITY
Stability

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25janO 1 .htm	10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 6 of 12

Stable under normal conditions of handling and storage. Hazardous decomposition

Thermal decomposition: No data.

Hazardous products of combustion: see section 5.

Materials to avoid/Reactivity

Reacts with bases to liberate heat.

Reacts with galvanized steel or unlined mild steel to produce hydrogen, a highly

flammable gas that could explode.

Hazardous polymerization

Does not occur. 11. TOXICOLOGICAL INFORMATION

This section is intended for use by toxicologists and other health professionals. Data obtained on product or on similar products are summarized below. Data obtained on active ingredient are summarized below.

Acute oral toxicity Rat, LD50 (limit test): > 5,000 mg/kg body weight

Other effects: breathing difficulty, decreased activity, soft stools Practically non-toxic. FIFRA category IV. No mortality.

Acute dermal toxicity

Rat, LD50 (limit test): > 5,000 mg/kg body weight Target organs/systems : None. Other effects: None. Practically non-toxic. FIFRA category IV. No mortality.

Acute inhalation toxicity Rat, LC50, 4 hours, aerosol: 2.6 mg/L Target organs/systems : None.

Other effects: breathing difficulty, decreased activity, local effects Practically non-toxic. FIFRA category IV.

Skin irritation Rabbit 6 animals, OECD 404 test

Days to heal: 1

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25janO 1 .htm	10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 7 of 12

?

Primary Irritation Index (PII): 0.4/8.0 Other effects: None. Essentially non irritating. FIFRA category IV.

Eye irritation Rabbit 6 animals , OECD 405 test

Days to heal: 10 Moderate irritation. FIFRA category II.

Skin sensitization Guinea pig, Buehler test Positive incidence: 0 %

EXPERIENCE WITH HUMAN EXPOSURE

Ingcstion. short term, case report(s); Gastro-intestinal effects: irritation, nausea/vomiting, diarrhea

Ingestion. short term:

Respiratory effects: increased fluid in lungs (lung/pulmonary oedema) Cardiovascular effects: decreased blood pressure (hypotension)

Similar formulation Acute oral toxicity

Rat, LD50: 5,000 mg/kg body weight Slightly toxic. FIFRA category III.

Acute dermal toxicity

Rabbit LD50 (limit test): > 5,000 nig/kg body weight

Practically non-toxic. FIFRA category IV. No mortality.

Skin irritation

Rabbit 6 animals, OECD 404 test

Days to heal: 3

Primary Irritation Index (PII): 1.0/8.0

Slight irritation.

FIFRA category IV.

Eye irritation

Rabbit 6 animals , OECD 405 test

Days to heal: > 21

Other effects: pannus, tissue destruction in eye (necrosis of conjunctivae)

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25jan01 .htm	10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 8 of 12

Severe irritation. FIFRA category I. Acute inhalation toxicity

Rat, LC50, 4 hours, aerosol: 3.28 mg/L Practically non-toxic. FIFRA category IV.

Skin sensitization

Guinea pig, 9-induction Buehler test

Positive incidence: 0 "/o

EXPERIENCE WITH HUMAN EXPOSURE

Ingestion, excessive, intentional misuse:

Respiratory effects: pneumonitis (aspiration)

Gastro-intestinal effects: nausea/vomiting, diarrhea, abdominal pain, bloody vomiting (haematemesis)

Cardiovascular effects: abnormal heart rhythm (cardiac dysrhythmia), decreased heart output (myocardial depression)

General/systemic effects: disturbances of fluid and electrolyte regulation, abnormally decreased blood volume (hypovolaemia), elevated serum amylase, fluid loss (haemoconcentration), no cholinesterase inhibition

Laboratory effects -blood chemistry: elevated serum transaminases, mild acidosis

Eve contact short term, epidemiological;

Note: No cases of irreversible eye effects could be attributed to glyphosate formulations in an extensive epidemiological survey of reported accidental eye contact with these formulations.

N-(phosphonomethyl)glycine

Mutagenicity In vitro and in vivo mutagenicity test(s):

Not mutagenic.

Repeated dose toxicity

Rabbit dermal, 21 days: NOAEL toxicity: > 5,000 mg/kg body weight/day

Target organs/systems : None.

Other effects: None. Rat, oral, 3 months: NOAEL toxicity: > 20,000 mg/kg diet

Target organs/systems : None.

Other effects: None.

Carcinogenicity

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25jan01 .htm	10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25janO 1	Page 9 of 12

r

Mouse, oral, 24 months:

NOEL tumour. > 30,000 mg/kg diet

NOAEL toxicity: - 5,000 mg/kg diet

Tumours: None.

Target organs/systems : liver

Other effects: decrease of body weight gain, histopathologic effects

Rat, oral, 24 months:

NOEL tumour. > 20,000 mg/kg diet

NOAEL toxicity: - 8,000 mg/kg diet

Tumours: None.

Target organs/systems: eyes

Other effects: decrease of body weight gain, histopathologic effects

Toxicity to reproduction/fertility

Rat, oral, 3 generations:

NOAEL toxicity: > 30 mg/kg body weight NOAEL reproduction: > 30 mg/kg body weight Target organs/systems in parents: None. Other effects in parents: None. Target organs/systems in pups: None. Other effects in pups: None.

Developmental toxicity/teratogenicity

Rat, oral, 6-19 days of gestation:

NOAEL toxicity: 1,000 mg/kg body weight

NOAEL development: 1,000 mg/kg body weight

Other effects in mother animal: decrease of body weight gain, decrease of survival

Developmental effects: weight loss, post-implantation loss, delayed ossification

Effects on offspring only observed with maternal toxicity.

Rabbit oral, 6-27 days of gestation:

NOAEL toxicity: 175 mg/kg body weight NOAEL development: 175 mg/kg body weight Target organs/systems in mother animal: None. Other effects in mother animal: decrease of survival Developmental effects: None.

12. ECOLOGICAL INFORMATION

This section is intended for use by ecotoxicologists and other environmental specialists.

Monsanto has not conducted environmental studies on this product. Data obtained on similar products are summarized below.

Similar formulation

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25janO 1 .htm	10/23/2008

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 10 of 1

> •

Aquatic toxicity, fish Bluegill sun fish (Lepomis macrochirus):

Acute toxicity, 96 hours, flowthrough, LC50: 5.8 mg/L moderately toxic Rainbow trout (Oncorhynchus mykiss):

Acute toxicity, 96 hours, flowthrough, LC50: 8.2 mg/L moderately toxic

Aquatic toxicity. invertebrates Water flea (Daphnia magna):

Acute toxicity, 48 hours, static, EC50: 12.9 mg/L slightly toxic

Aquatic toxicity. algae/aquatic plants Green algae (Selenastrum capricornutum):

Acute toxicity, 96 hours, static, EC50: 2.6 mg/L moderately toxic

Avian toxicity Bobwhite quail (Colinus virginianus):

Dietary toxicity, 5 days, LC50: > 5,620 mg/kg diet practically nontoxic Mallard duck (Anas platyrhynchos):

Dietary toxicity, 5 days, LC50: > 5,620 mg/kg diet practically non-toxic

Arthropod toxicity Honey bee (Apis mellifera):

Oral/contact, 48 hours, LD50: > 100 uig/bee practically non-toxic

Soil organism toxicity, invertebrates Earthworm (Eisenia foetida):

Acute toxicity, 14 days, LC50: > 5,000 mg/kg dry soil practically non-toxic

N-(phosphomethyl)glycine: (glyphosate)

Bioaccumulation Bluegill sunfish (Lepomis macrochirus):

Whole fish: BCF: < 1

No significant bioaccumulation is expected.

Dissipation Soil, field:

Half life: 2-174 days Koc: 884 - 60,000 L/kg Binds strongly to soil. Water, aerobic:

Half life: < 7 days

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25janO 1 .htm	10/23/200

Monsanto's Roundup Herbicide MSDS Effective 25jan01	Page 12 of 12

* f

The information given here is not necessarily exhaustive but is representative of relevant, reliable data. For more information refer to product label. Please consult Monsanto if further information is needed. Follow all local/regional/national regulations. In this document the British spelling was applied. All trademarks herein are trademarks of Monsanto Company or its subsidiaries.

Full denomination of most frequently used acronyms. BCF (Bioconcentration Factor), BOD (Biochemical Oxygen Demand), COD

(Chemical Oxygen Demand), EC50 (50°/ effect concentration), ED50 (50°/ effect dose), I.M. (intramuscular), I.P. (intraperitoneal), TV. (intravenous), Koc (Soil adsorption coefficient), LC50 (50°/ lethality concentration), LD50 (50°/ lethality dose), LDLo (Lower limit of lethal dosage), LEE (Lower Explosion Limit), LOAEC (Lowest Observed Adverse Effect Concentration), LOAEL (Lowest Observed Adverse Effect Level), LOEC (Lowest Observed Effect Concentration), LOLL (Lowest Observed Effect Level), MEL (Maximum Exposure limit), MTD (Maximum Tolerated Dose), NOAEC (No Observed Adverse Effect Concentration), NOAEL (No Observed Adverse Effect Level), NOEC (No Observed Effect Concentration), NOEL (No Observed Effect Level), OEL (Occupational Exposure Limit), PEL (Permissible Exposure Limit), PIT (Primary Irritation Index), Pow (Partition coefficient n-octanol/water), S.C. (subcutaneous), STEL (Shot-Term Exposure Limit), TLV-C (Threshold Limit Value-Ceiling), TLV-TWA (Threshold Limit Value - Time Weighted Average), UEL (Upper Explosion Limit)

This Material Safety Data Sheet (MSDS) serves different purposes than and DOES NOT REPLACE OR MODIFY THE EPA-APPROVED PRODUCT LABELING (attached to and accompanying the product container). This MSDS provides important health, safety, and environmental information for employers, employees, emergency responders and others handling large quantities of the product in activities generally other than product use, while the labeling provides that information specifically for product use in the ordinary course. Use, storage and disposal of pesticide products are regulated by the EPA under the authority of the Federal Insecticide, Fungicide, and Rodenticide Act (FIFRA) through the product labeling, and all necessary and appropriate precautionary, use, storage, and disposal information is set forth on that labeling. It is a violation of federal law to use a pesticide product in an manner not prescribed on the EPA-approved label.

Although the information and recommendations set forth herein (hereinafter "Information") are presented in good faith and believed to be correct as of the date hereof, MONSANTO Company makes no representations as to the completeness or accuracy thereof. Information is supplied upon the condition that the persons receiving same will make their own determination as to its suitability for the purposes prior to use. In no event will MONSANTO Company be responsible for damages of any nature whatsoever resulting from the use of or reliance upon information. NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE OR OF ANY OTHER NATURE ARE MADE HEREUNDER WITH RESPECT TO INFORMATION OR TO THE PRODUCT TO WHICH INFORMATION REFERS.

If you have come to this page from an outside location click here to get back to mindfully.org

http://www.mindfully.org/Pesticide/Monsanto-Roundup-MSDS25janO 1 .htm	10/23/2008