MATERIAL SAFETY DATA SHEET

P.O. Drawer  1410    -     1600 E. Hill St., Long Beach, CA 90801 Contact:  Safety Department    -    Telephone: (213) 427-5471

WARNING STATEMENT

February 1989

DANGER! Extremely Flammable.    Keep away from heat, sparks and open flame.

Vapor reduces oxygen available for breathing and may cause suffocation in confined spaces.    Use only with adequate ventilation.    Odor may not provide adequate warning of potentially hazardous concentrations.    Vapor is heavier than air and collects in low levels. Liquid may cause freeze burn similar to frostbite.

.. .;•;:•••	-:

Product Name:    Petrolane Propane

Chemical Name:    Propane

Synonyms: LP-Gas, Bottled Gas

Chemical Family: Paraffinic Hydrocarbon

Chemical Formula: C3HB

DOT Proper Shipping Name: Liquefied Petroleum Gas

DOT Hazard Class: Flammable Gas

DOT LIT Number: UNT075

Transportation Emergency Telephone: 800-424-9300    (CHEMTREC)

NFPA Classification:

Health	1    Slightly Toxic

Fire	4    Extremely Flammable

Reactivity    0    Stable

iii^^

Component

Ethane

Propane

Propylene

Butane

CAS Number

74-84-0

74-98-6

115-07-1

106-97-8

%

0-6

87-97

0-5

0-2.5

OSHA PEL

None established 1000 ppm <8hr) None established None established

ACGIH TLV

Simple asphyxiant Simple asphyxiant Simple asphyxiant 800 ppm (8 hr)

Data

Boiling Point:    -44°F

% Volatile by Volume:    100% Solubility in Water:    Insoluble Evaporation Rate(Bu Ac=1):    N/A Gas Volume @ Atm. Pressure & 60 °F (Cu. ft. gas/gal, liquid): 36.4

Melting Point:    -309°F

Vapor Pressure:    208 psig (max.) @  100°F

Vapor Density (Air=1):    1.5

Specific Gravity (H20=1): 0.504

Appearance and Odor:    Colorless, odorless in pure form.

Propane contains a foul smelling, skunk-like warning agent(odorant).    The odorant is effective, in most instances, but not everyone can smell the odor.    The ability of people to detect odors varies widely.    Also, certain chemical reactions with material  in the propane system can reduce the propane odor level.    No odorant will be 100% effective in all circumstances.    If odor level appears to be weak, notify propane supplier immediately.

Data;::.|

Flash Point (Method Used):    -156°F (estimated) Flammable Limits (% Volume in Air):    Lower 2.1%

Upper 9.5%

Extinguishing Media:    Dry chemical, foam or C02 for small fires.    Stop flow of gas first.

Special Fire Fighting Procedures and Precautions:    Elimimate sources of ignition.    Evacuate area.    Notify fire department.    Allow only trained, properly protected personnel in area.    Shut off source of gas, if possible.   Allow fire to burn itself out after gas flow is shut off.    High volume water supply can be used to cool heat-exposed pressure containers and nearby equipment.    Approach a flame enveloped container from the side, never the head ends.   Use extreme caution when applying water to a container which has been exposed to heat or flame for more than a short time.   Shock of cool water on hot metal could cause container rupture.     For uncontrollable fires and when flame is impinging on container, withdraw all personnel and evacuate surrounding vicinity immediately.

Unusual Fire and Explosion Hazards:    Products of combustion may yield carbon monoxide, a toxic gas. Uncontrolled vapors spread rapidly, are heavier than air and are extremely flammable.

sReael	Data

S-775

Stability:    Stable	Conditions to Avoid:    High heat, sparks, open flame

Materials to Avoid:    Strong oxidizing agents

Hazardous Decomposition Products:    Incomplete combustion can cause carbon monoxide, a toxic gas.

Hazardous Polymerization:    Will not occur	Conditions to Avoid:    None

IVI;      Heal t h Hazard Da ta

Product is not listed as carcinogenic by NTP, IARC or OSHA. Product may contain a trace, but detectable amount of benzene, a chemical listed by the State of California and known to cause cancer or reproductive toxicity.

Routes of Entry / Acute Effects of Overexposure:

Inhalation:    Exposure to high concentrations of the vapor causes dizziness, drowsiness, nausea or unconsciousness due to anesthetic properties.

Skin Contact:    Liquid can cause freeze burns similar to frostbite if contact with skin occurs. No skin absorption is expected.

Eye Contact:    Liquid can cause freeze burns if contact with eyes occurs.

Ingestion:    Ingestion is not expected to occur in normal use.

Chronic Effects of Qverexposure:    No abnormal reactions reported following exposure to  1000 ppm

for 8 hours per day, 5 days per week, for 2 weeks.

Medical Conditions Generally Aggravated by Exposure: Persons with chronic respiratory diseases should avoid exposure.

IVII.        Emergency and f \ rs t A S d P rocedures 1

Eye Contact: Flush with water.    Obtain medical assistance if contact with liquid has occurred.

Skin Contact:    If freeze burn occurs, remove contaminated clothes, shoes and jewelry.    Immerse burned area in warm (not hot) water.    Keep immersed.    Call for medical assistance.

jnhalation:    Remove victim from further exposure and into fresh air.    Provide oxygen if breathing is labored.    If victim is unconscious, seek immediate medical attention.    If breathing has stopped, give artificial respiration.

Ingestion:    Not expected to occur in normal use.

y 1 1 I .;;•:-: ; : Soil :1: ;:or : Leak

Product is extremely flammable.    Vapor is heavier than air and may collect at lower levels. Flammable connentrations  may he present below nose  level.     If there  is a  leak but no fire,  do not light the escaped gas.    Eliminate all  ignition sources.    Do not smoke, use a nearby phone or actuate electrical  switches.    Evacuate the area.    If possible, remove  leaking container to safe area.    Stop flow of gas or allow vapor to disperse in a safe area.    Water spray can be used to help dilute vapor concentration in air.

Dispose of gas only by controlled burning in compliance with local laws and regulations.

a-Xm  Hand 1 i ng and Storage Preeaut Ions SI

Store in an authorized location (outside, detached storage is preferred) with adequate ventilation. Keep away from heat and ignition sources.    Inspect cylinders frequently for leaks, dents, gouges and corrosion with emphasis on bottom of cylinder.    Store cylinders in upright position or with pressure relief valves in vapor space.    Do not drop or abuse cylinders.    Keep container valve closed and plugged when not in use.    Install protective caps when cylinders are not connected for use.    Empty containers retain some residue, so they should be treated as if they are full.

Personal Protect ion informal ton Tl Ventilation:    Use adequate ventilation to maintain exposures below recommended limits.

Respiratory Protection:    Use a NIOSH-approved respirator if area is thought to contain unknown concentration of gas.

Eye Protection:    Use safety goggles or safety glasses with side shields.

Protective Clothing:    No special garments are necessary, but avoid skin contact with liquid because of possibility of freeze burn.    Propane resistant gloves are recommended.

IXIa Gommuh i cat ion wl ith Employees and Pur-chasers ; ail

This Material Safety Data Sheet (MSDS) alerts the reader to the potential safety and health hazards of propane.    It also contains valuable reference material relating 10 the safe use and handling of the product.    Make sure that this information is shared with all employees and purchasers wno use or handle the product.    It is an important part of the OSHA hazard communication program.

This information is believed to be accurate as of the date of issue, but is offered without guarantee.  Conditions of use and suitability for use are beyond Company control, therefore, all risks of use of the product are assumed by the user.  COMPANY EXPRESSLY DISCLAIMS ALL WARRANTIES OF EVERY KIND INCLUDING WARRANTIES OF MERCHANTIBILITY AND FITNESS FOR ANY PARTICULAR PURPOSE.  Company assumes no responsibility for any injuries or damages caused by the product even if safety procedures are followed as outlined herein.  Nothing herein is intended to be construed as permission or recommendation for use of the product in any manner which might infringe existing patents.