VALUE

P.O. 80X2235 Smyrna, Georgia 30081

MATERIAL SAFETY DATA SHEET Meeting OSHA Standard 29 CFR §1910,1 200 (g)

EFFECTIVE DATE:

Nov. 25, 1985

24 Hour Emergency telephone number: 404-588-4400

SECTION I — PRODUCT IDENTIFICATION

PRODUCT TRADE NAME: Mop-up

CHEMICAL NAME AND SYNONYMS: Disodium Oclaborate Tetrahydrate

CHEMICAL FAMILY: Sodium borate
HAZARD RATING: NFPA: Health	0

Fiammability   0

Reactivity	0

CAS NO: 12008-41-2 FORMULA: Na?BBo.,,4H2Q     -

SECTION II — HAZARDOUS INGREDIENTS

Material or Component None

Material or Component

SECTION III — PHYSICAL DATA

Approx. Formula weight 412 White, odorless powder (light blue tint) Bulk density 25-35 Ibs/ft3 Particle Size: 149-44 microns pH@23°C(73.4°F)    1% 8.5

1O% 7.6

15% 7.3

Solubility in water:

20°C    (68°F) 9.5%

94°C(201 °F) 45.3%

SECTION IV — HEALTH HAZARD INFORMATION

EFFECTS OF EXPOSURE (SYMPTOMS) INGESTION: Nausea, vomiting.

ACTUE ORAL LDM 2.0 g/kg body weight (rats). EYE; (16CFR §1500.42) May be slightly irritating. Reversible, SKIN: Not an irritant.

ACUTE DERMAL LDSO(16CFR §1 500.40) Unknown PRIMARY SKIN IRRITATION INDEX: (1 6CF §1 500.41) Less than 1 CORROSIVE

INHALATION May be irritating to nose and throat. ACUTE LCSO Unknown.

NON CORROSIVE; X per49CFR§24O

EFFECTS OF OVEREXPOSURE

ACUTE OVEREXPOSURE/METHOD OF EXPOSURE/SYMPTOMS

Eyes — may be irritating. Reversible.
Skin — None expected.        :	:

Mouth/Nose — May be slightly irritating. Ingestion — Nausea/vomitirg,

POLYBQR'" 1 1/1/85

HRONIC OVEREXPOSURE/METHOD OF EXPOSURE/SYMPTOMS

Eyes: May be irritating.

Skin: May be irritating on prolonged exposure on wet skin.

Mouth/Nose: May be irritating.

Ingestions: Nausea, vomiting, diarrhea, skin rash.