938 P12   MfiR 27 '02 13:57

DE-1

MATERIAL SAFETY DATA SHEET

Radiator Specialty Company

1900 WILKINSON BLVD. CHARLOTTE, NC 28208 (704) 377-6555

POISON INFORMATION & EMERGENCY: 303-623-5716

MATERIAL SAFETY DATA SHEET

May be used to comply with OSHA's Hazard Communication Standard 29 CFR 1910.1200. Standard must be consulted for specific requirements.

US DEPARTMENT OF LABOR

Occupational Safety and Health Administration. (Non-Mandatory Form) Form Approved OMB No. 1218-0072

SECTION I GENERAL INFORMATION

PRODUCT NAME PART NUMBER

DE-ICER DE-1

NOTE: Blank spaces are not permitted. If any item is not applicable or no information is available, the space must be marked to indicate that.

SECTION II HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

Components not identified are non-hazardous according to 29 CFR 1910.1200 SECTION III PHYSICAL/CHEMICAL CHARACTERISTICS

| COMPONENT WT'/p | C.A.S. NO. | TLV (ACGIH-X OSHA ) |
| --- | --- | --- |
| Methanol* 45-55 Ethylene Glycol * + 3-7 Carbon Dioxide 3-5 | 67-56-1 107-21-1 124-38-9 | 200 ppm 100 ppm 5000 ppm |
| 'THESE CHEMICALS ARE SUBJECT TO REPORTING REQUIREMENTS + THIS PRODUCT CONTAINS CHEMICALS KNOWN TO THE STATE' OF BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM. Comments |  | OF SARA TITLE III , 40 CFR PART 372. CALIFORNIA TO CAUSE CANCER AND |

Specific Gravity (HjO-1)	0.792

Solubility in Water	Infinite

Flash Point (Method) - F°         54 (TCC)

Melting Point-F°	-144T

Vapor Pressure (mmHg)	N/A

Evaporation Rate (Butyl Acetate=1)   5.9

Appearance and Odor	Clear liquid with alcohol ocior,

pH         N/A

Solubility in Solvent N/A
% Volatile* By Wt. >92
Boiling Point - F°	148°F

Vapor Density (Air=1)  1.11

SECTION IV FIRE AND EXPLOSION HAZARD DATA

EXTINGUISHING MEDIA:
Water Fog

(LEL:6.0      UEL36.5)

Foam   X	CO2      X

Dry Chemical     X

SPECIAL FIRE FIGHTING PROCEDURES, Wear self-contained, positive pressure breathing apparatus and
protective clothes.   Cool containers exposed to heat with water fog.

UNUSUAL FIRE AND EXPLOSION HAZARDS   Aerosol containers are under pressure and may burst wren heatec

SECTION V REACTIVITY DATA

93B P13   MflR 27 '02 13:58

Stable       X

Unstable

Corrosive      NO         Hazardous Polymerization*? Yes	No    X

^compatibilities  Strong oxidizing agents/ will attack some forms of plastics and rubber.

Decomposition or Byproducts   Carbon monoxide and carbon dioxide may be formed during combustion,

SECTION VI HEALTH HAZARD INFORMATION

| Recommended TLV | of Product |  | 100 | ppm |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EYE CONTACT May be slightly irritating | to eyes |  |  |  | SKIN | CONTACT | Slight irritant dermatitis. | Prolonged | exposure | may cause |  |
| INHALATION Avoid chronic exposure | INGESTION |  |  |  |  |  | HARMFUL May | OR FATAL IF SWALLOWED! cause diarrhea , nausea, and |  |  | cramps, |
| OTHER Methanol is a repeated and prolonged | poisonous, narcotic chemical and if ingested can cause blindness and death. Reports have over exposure to solvents with permanent brain and nervous system damage,. |  |  |  |  |  |  |  |  | associated |  |

SECTION VII EMERGENCY AND FIRST AID PROCEDURES

| EYE CONTACT Flush eyes with water for 15 minutes while lifting eyelids. Consult a physician immediately. |  |
| --- | --- |
| SKIN CONTACT Wash with soap and water |  |
| INHALATION Move to fresh air. If breathing has stopped give artificial respiration or oxygen as required. Get medical attention. |  |
| INGESTION If swallowed contact a physician immediately. Only induce vomiting at the instruction of a physician, Aspiration lungs can cause chemical pneurnonitis, which can be fatal. | into |

SECTION VIII SPECIAL PROTECTION INFORMATION

|  | CONSUMER | BULK HANDLING | (Prolonged Exposure) |
| --- | --- | --- | --- |
| RESPIRATORY I PROTECTION | N/A | TLV is exceeded, wear organic vapor respirator |  |
| VENTILATION | Use with adequate ventilation | General |  |
| EYE PROTECTION | N/A | Goggles |  |
| PROTECTIVE CLOTHING | N/A | Rubber gloves and | gpron |

SECTION IX PRECAUTIONS FOR SAFE HANDLING AND USE

| SPILL OR LEAK PROCEDURE Remove ignition sources, ventilate area, confine spill, wipe up with absorbent material, waste drum. | transfer to |
| --- | --- |
| WASTE DISPOSAL METHOD Dispose of in accordance with all applicable government laws and regulations. |  |
| STORAGE AND HANDLING PRECAUTION Store and use away from all heat and ignition sources. |  |
| OTHER PRECAUTIONS KEEP AWAY FROM CHILDREN AND ANIMALS! AVOID EYE AND SKIN EXPOSURE! AVOID BREATHING VAPORS! |  |

HAZARD INFORMATION LABEL DATA
HAZARD CODE	y\

Supersedes   NEW

4-Ex«nm*     ««««/    \
3 = High	/\ 3    /\R£ACTVW

1-Bllghit

OSHA Revised MAY 1999 Title R. GEER • CHEMIST

WMa flatfafor Sp»p*'(y Company belies f'S Sa'a'« accurate as onns fgwsion aa;». ** me** no warrant/ *i:n rasps;' (5 We 2S."4 snd ws c/prtSSl's ti'S&Birn 6-'l /isO/My /to? rel/anes rnarfoi  TH6 dS'S is offeva solely <°<~ mrorww, invsa.-'sSKJA