MATERIAL SAFETY DATA SHEET

AMOFUEL NO. 2 DIESEL MSDS NO:  02000909

MANUFACTURER/SUPPLIER: Amoco Oil Company 200 East Randolph Drive Chicago, Illinois 60601

EMERGENCY HEALTH INFORMATION:  (800) 447-8735 EMERGENCY SPILL INFORMATION:  (800) 424-9300

CHEMTREC, U.S.A. OTHER PRODUCT SAFETY INFORMATION:  (312) 856-3907

IMPORTANT COMPONENTS:  Petroleum distillate (CAS 68476-30-2).

No exposure limit established.

Naphthalene (CAS 91-20-3) ACGIH TLV 10 ppm, STEL 15 ppm; OSHA PEL 10 ppm, STEL 15 ppm.

Xylene (CAS 1330-20-7) ACGIH TLV 100 ppm, STEL 150 ppm; OSHA PEL 100 ppm, STEL 150 ppm.

WARNING STATEMENT:  Warning!  Combustible.  Harmful if swallowed and/or aspirated into

the lungs.  Can cause skin irritation on prolonged or repeated contact.

HMIS/NFPA CODES:(HEALTH;!)(FLAMMABILITY;2)(REACTIVITY;0)

"

APPEARANCE AND ODOR:  Clear, bright liquid.

HEALTH HAZARD INFORMATION

EYE

EFFECT:     No significant irritation expected.

FIRST AID:  Flush eyes with plenty of water.

PROTECTION: None required, however, use of safety glasses is good industrial practice.

SKIN

EFFECT:     Can cause skin irritation upon prolonged or repeated contact.  See Toxicology Section.

FIRST AID:  Wash exposed skin with soap and water.  Remove contaminated clothing, including shoes, and thoroughly clean and dry before reuse.

PROTECTION:  Avoid prolonged or repeated skin contact.  Wear protective gloves if prolonged or repeated contact is likely.

INHALATION

EFFECT:     No significant health hazards identified for the liquid fuel. Toxicology Section.

FIRST AID:  If adverse effects occur, remove to uncontaminated area. PROTECTION:  Use with adequate ventilation.

AMOFUEL   NO.   2   DIESEL MSDS NO:     02000909

PAGE   02   OF   04

HEALTH HAZARD INFORMATION - CONTINUED

INGESTI ON

EFFECT:     Low viscosity product. Harmful or fatal if swallowed and/or aspirated into lungs.  See Toxicology Section.

FIRST AID:  If swallowed, do NOT induce vomiting, get immediate medical attention.

FIRE AND EXPLOSION INFORMATION

FLASHPOINT:      120°F TO    180°F, (TCC) FLAMMABLE LIMITS:  UPPER:  7.5%  LOWER:  0.6%

EXTINGUISHING MEDIA:  Agents approved for Class B hazards (e.g., dry chemical, carbon

dioxide, halogenated agents, foam, steam) or water fog.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Not applicable.

PRECAUTIONS: Keep away from ignition sources (e.g., heat and open flames).

REACTIVITY INFORMATION

STABILITY:  Stable.

CHEMICAL AND PHYSICAL PROPERTIES

BOILING POINT:      340°F TO     675°F, Range approx. SOLUBILITY IN WATER: Negligible, below 0.1%. SPECIFIC GRAVITY (WATER =1):      0.88 VISCOSITY:  1.8-3.6 cS @ 100°F

STORAGE AND ENVIRONMENTAL PROTECTION

STORAGE REQUIREMENTS:  Store away from heat, ignition sources, and open flame, in

accordance with applicable regulations.

SPILLS AND LEAKS:  Remove mechanically or contain on absorbent material.

WASTE DISPOSAL:  Disposal must be in accordance with applicable federal, state, or

local regulations.  Enclosed-controlled incineration unless directed otherwise by applicable ordinances.

SPECIAL PRECAUTIONS:  Avoid strong oxidizers.

TOXICOLOGICAL INFORMATION

EYE:  Similar products have produced maximum eye irritation scores ranging from 0.33 to 1.0/110.0; 24 hours (rabbits).

SKIN: Similar products have produced primary skin irritation scores ranging from 0.67 to 3.83/8.0 (rabbits).  Dermal LD50 for similar products was greater than 2g/kg; practically non-toxic for acute exposures by this route.

AMOFUEL NO. 2 DIESEL MSDS NO:  02000909

PAGE 03 OF 04

TOXICOLOGICAL INFORMATION - CONTINUED

INGESTION: For a similar product oral LD50 was greater than 5g/kg; practically non-toxic for acute exposures by this route.

A chronic skin-painting study has shown that this product is capable of producing skin cancer in laboratory animals.  Ten out of the original 50 test animals developed on-site tumors following repeated and prolonged skin applications of furnace oil over their lifetime.  The mean latency period was 90.0 weeks.

Occasional skin contact with this product is not expected to have serious effects, but good personal hygiene should be practiced; prolonged and unnces-sary skin contact avoided. This product can also produce moderate skin irritation upon prolonged or repeated skin contact.  Personal hygiene measures taken to prevent skin irritation are expected to be adequate to prevent undue risk of skin cancer.

This product has a sufficiently low vapor pressure to prevent a hazardous buildup of vapors unless the product is heated, used in a confined space with inadequate ventilation or misted.  Inhalation of mist or high concentrations of vapors can produce dizziness, headache, and nausea and possibly irritation of the eye, nose and throat.

Aspiration of this product into the lungs can cause chemical pneumonia and can be fatal. Aspiration into the lungs can occur while vomiting after ingestion of this product.  Do not siphon by mouth.

NIOSH has recommended that whole diesel exhaust be regarded as a potential occupational, carcinogen, based on findings of carcinogenic responses in laboratory animals exposed to whole diesel exhaust.  The excess cancer risk for workers exposed to diesel exhaust has not been calculated; however, exposure should be minimized to reduce potential risk.

No component of this product present at levels greater than 0.1% is identified as a carcinogen by NTP, IARC or OSHA.

REGULATORY INFORMATION

CERCLA REPORTABLE QUANTITY:

This product is exempt from the CERCLA reporting requirements under 40 CFR Part 302.4. However, if spilled into waters of the United States, it may be reportable under 40 CFR Part 153 if it produces a sheen.

DOT PROPER SHIPPING NAME:  Fuel Oil, Diesel, Combustible Liquid, NA1993. OSHA HAZARD COMMUNICATION STANDARD:  Combustible liquid.

RCRA STATUS:

This product is subject to the 40 CFR Part 268.30 land ban on the disposal of certain hazardous wastes because it contains the following substance(s):

COMPONENT/CAS NUMBER Xylene (1330-20-7)

AMOFUEL   N-"'      ">   TESEI MSDS NO:     02000909

PAGE   04   OF   04

REGULATORY INFORMATION - CONTINUED

SARA STATUS:

This product is regulated under the following section(s) of SARA Title III, 42 USC 9601. Spills or releases of the product may be reportable as determined by the information given below:

SECTIONS 311 AND 312 OF SARA AND 40 CFR PART 370:

This product is defined as hazardous by OSHA under 29 CFR Part 1910.1200(d).

SECTION 313 OF SARA AND 40 CFR PART 372:

This product contains the following substances, which are on the Toxic Chemicals

List in 40 CFR Part 372:

COMPONENT/CAS NUMBER

WEIGHT PERCENT

Naphthalene (91-20-3) Xylene (1330-20-7)

TSCA STATUS:  All of the components of this product are listed on the TSCA Inventory.

ISSUE INFORMATION

BY:

Richard A. Symuleski Director, Product Safety

ISSUED: SUPERSEDES:

July 25, 1989 July 18, 1989

This material safety data sheet and the information it contains is offered to you in good faith as accurate. We have reviewed any information contained in this data sheet which we received from sources outside our company.  We believe that information to be correct but cannot guarantee its accuracy or completeness.  Health and safety precautions in this data sheet may not be adequate for all individuals and/or situations.  It is the user's obligation to evaluate and use this product safely and to comply with all applicable laws and regulations.  No statement made in this data sheet shall be construed as a permission or recommendation for the use of any product in a manner that might infringe existing patents.  No warranty is made, either express or implied.

See