MATERIAL SAFETY DATA SHEET: SELECTROL, O/G

Page:    1

(000000-000000-	-0476	)

DATE   OF   ISSUE 8/31/1998

SUPERSEDES 8/26/1998

SECTION   I   -   GENERAL   INFORMATION

^ Chemical Name £ Synonyms	Trade Name S Synonyms

S 'A	SELECTROL, O/G

Chemical Family:	Formula Mixture --> X

ARSENATE

Manufacturer's Name:

MANTEK, DIVISION OF NCH CORP.

Address:

BOX 152170

IRVING,         TEXAS 75015

Prepared By:	Product Code Number	Emergency Phone Number

G ZIMMERMAN/CHEMIST	0476	800-424-9300

SECTION II - HAZARDOUS INGREDIENTS
	THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (Ingredients)	Hazard        TLV	PEL	STEL	CAS #

CALCIUM ACID METKANEARSONATE	IRRITANT       N/E  1.         N/E  2.	5902-95-4

SECTION III - PHYSICAL DATA

Boiling Point (F):	216	Specific Gravity	(H20=l):	1.12

Vapor Pressure (MM HG) :	18	Color:	COLORLESS

Vapor Density (Air=l) :	0.6	Odor:	ODORLESS

PH @ 100% :	4,0	Clarity:	TRANSPARENT

% Volatile by Volume:	87	Evaporation Rate	(BU A/C=1) :	<1

H20 Solubility:	COMPLETE	Viscosity:	NON-VISCOUS

---------.	_        _	„__„

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

NON-FLAM / T.C.C.	N/A	N/A	N/A

Extinguishing Media

X <—Foam	X <—Alcohol Foam        X <—CO2        X <—Dry Chemical         X <—Water Spray      <--Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF

THE SURROUNDING FIRE.

Unusual Fire and Explosion Hazards:

WHILE THE MATERAIL WILL NOT BURN, IT WILL RELEASE HIGHLY TOXIC ARSENIC IF HEATED TO DECOMPOSITION.  USE WATER SPRAY TO COOL FIRE EXPOSED CONTAINERS

TO PREVENT BURSTING.

Aerosol Level (NFPA 30B):    N/A

NFPA 704 Hazard Rating       (0=Insignifleant l=Slight  2=Moderate  3=High  4=Extreme) 3  <—Health       1  <—Flammability  0  <—Instability      <—Special

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value: NOT ESTABLISHED FOR MIXTURE.   SEE SECTION II.

Effects of Overexposure:

-Acute (Short Term Exposure)

SKIN: MAY CAUSE IRRITATION SEEN AS ITCHING OR REDNESS. EYES: MAY CAUSE IRRITATION SEEN AS ITCHING OR REDNESS.  INHALATION: MATERIAL DOES NOT CREATE FUMES; INHALATION OF A MIST MAY CAUSE COUGHING, SNEE3ING, OR A BURNING SENSATION IN THE NOSE OR THROAT- INGESTION: MAY CAUSE NAUSEA, VOMITING OR DIARRHEA.

-Chronic (Long Term Exposure)

MEDICAL CONDITIONS AGGRAVATED: NONE KNOWN- TARGET ORGANS: NONE KNOWN. THERE IS NO PRIMARY ROUTE OF ENTRY INTO THE BODY. THE PRIMARY ROUTE OF EXPOSURE IS SKIN AND EYE CONTACT.

Primary Routes of Entry:       <—Inhalation       <—Ingestion        <—Absorption

gency and First Aid Procedures:

-Inhalation: b^MOVE THE VICTIM TO FRESH AIR. APPLY ARTIFICIAL RESPIRATION, PREFERABLY MOUTH TO MOUTH, IF INDICATED.

-Eye Contact: FLUSH WITH PLENTY OF WATER. GET MEDICAL ATTENTION IF IRRITAION PERSISTS.

-Skin Contact: WASH WITH PLENTY OF WATER. GET MEDICAL ATTENTION IF 1..RITATION PERSISTS.

-Ingestion:

CALL A PHYSICIAN IMMEDIATLY. DRINK ONE OR TWO GLASSES OF WATER AND INDUCE VOMITING BY TOUCHING THE BACK OF THE THROAT WITH A FINGER- DO NOT INDUCE VOMITING OR GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON.

-Notes to Physician: A GARLIC ODOR OF THE BREATH IS CHARACTERISTIC OF INGESTION OF A LARGE AMOUNT.     BAL (DIMERCAPROL)  IS A SPECIFIC ANTIDOTE.

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> No         NTP—> NO	OSHA—> No	ACGIH—> No	OTHER—> No

CALCIUM METHANEARSONATE

ORL-RAT LD50:6000MG/KG  3.

VOC INFORMATION: THIS PRODUCT CONTAINS 0 GRAMS/LITER VOC.

SECTION VII - REACTIVITY DATA

Stability:	X <—Stable      <—Unstable

Conditions to Avoid: N/A

Incompatibility (Materials to Avoid):

STRONG REDUCING AGENTS, STRONG BASES, HEAVY METAL COMPOUNDS,

Hazardous Decomposition Products: OXIDES OF CARBON, ACRID SMOKE, ARSINE.

Hazardous Polymerization:	<—May Occur      X <—Will Not Occur

Conditions to Avoid: N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

DIKE AND CONTAIN SPILL IF POSSIBLE. AVOID RUNOFF INTO SEWERS OF BODIES OF WATER.  IF POSSIBLE PICK UP UNCONTAMINATED MATERIAL FOR USE ACCORDING TO

LABEL DIRECTIONS. ABSORB ANY UNUSABLE MATERAIL WITH AN INERT MATERIAL AND STORE IN A LABELED CONTAINER,

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS -

Neutralizing Agent: N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

Required Ventilation:

GENERAL VENTILATION IS NORMALLY ADEQUATE.   LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE MISTS.

Respiratory Protection:

NONE REQUIRED UNDER NORMAL CONDITIONS OF USE.    A NIOSH/MSHA APPROVED RESPIRATOR IF APPLIED IN A WAY THAT MAY RESULT IN MIST BEING BLOWN INTO THE

FACE.

Glove Protec tion:

NEOPRENE RUBBER OR NITRILE GLOVES SHOULD BE WORN.

Eye Protection:

SAFETY GLASSES WITH SIDE SHIELDS IF THE METHOD OF USE PRESENTS THE LIKELIHOOD OF EYE CONTACT.

Other Protection:

WEAR PROTECTIVE CLOTHING WHEN HANDLING.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:    Indoors--> X	Outdoors-->	Heated—>	Retrigerated—>

Minimum Temperature:  35 F.  Maximum Temperature: 120 F.

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. DO NOT STORE NEAR FbKD OR FOODSTUFFS.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS. DO NOT APPLY TO FOOD CROPS OR ANIMAL FKK1)

CROPS. DO NOT ALLOW SPRAY OR RUNOFF TO CONTAMINATE WATER USED BY WILDLIFE, AQUATIC LIFE, DOMESTIC ANIMALS, OR USED FOR IRRIGATION.

SECTION XI - REGULATORY INFORMATION

Chemical Name	CAS Number	Upper % Limit

ARSENIC COMPOUNDS	N/A	5

Those ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of California.

MATERIAL SAFETY DATA SHEET: SELECTRGL, O/G	Page: 3

SECTION XI - REGULATORY INFORMATION (Continued)

SECTION XII - REFERENCES

1. THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND

BIOLOGICAL EXPOSURE INDICES, ACGIH, 1997.

OSHA PEL.

'?

REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFODlSC, 1998. u OF THE COMPONENTS WITHIN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

IRR: IRRITANT , FLAM/FLAMM: FLAMMABLE, COMB: COMBUSTIBLE, CORR: CORROSIVE CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC:CLEVELAND OPEN CUP, PMCC: PENSKY-MARTIN CLOSED CUP, TCC:TAGLIABUE CLOSED CUP, LEL:LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTPrNATIONAL TOXICOLOGY PROGRAM, OSHA:OCCUPATIONAL SAFETY S HEALTH ADMINISTRATION, ACHGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV:THRESHOLD LIMIT VALUE, PEL:PERMISSIBLE EXPOSURE LEVEL, STEL: SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD: MODERATE, SEViSEVERE, MUT:MUTAGENIC, AS PHYX: ASPHYXIANT

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF-

MANTEK, DIVISION OF NCH CORP- assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label.  Users assume all risks associated with such unrecommerided use, storage, or disposal of the produc t.