AP8833

flinn scientific, inc.

Material Safety Data Sheet (MSDS)	MSDS #: 639 °°

Revision Date: March 14,2001

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Potassium Iodide Solution

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 — COMPOSITION, INFORMATION ON INGREDIENTS

Potassium Iodide Solution (7681-11-0) 2-17%, and Water (7732-18-5) 83-98%. A 0.2% solution of proprionic acid is used as a preservative in the 0.1 Molar solution. CAS#: None Established

SECTION 3 — HAZARDS IDENTIFICATION

Colorless solution. Odorless.	FLINN AT-A-GLANCE

Substance not considered hazardous. However, not all health aspects of this substance have been	Health-0

thoroughly investigated.	Flammability-0

React! vity-0 Exposure-0 Storage-0

0 is low hazard, 3 is high hazard

SECTION 4 — FIRST AID MEASURES

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Non-combustible liquid.	NFPA CODE

None Established

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Restrict unprotected personnel from area and ventilate area. Contain spill with sand or absorbent material; deposit in sealed bag or container. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Inorganic #2. Store with acetates, halides, sulfates, sulfites, thiosulfates and phosphates.

SECTION 8 — EXPOSURE CONTROLS, PERSONAL PROTECTION

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

1999 Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

flinn scientific, inc.

Material Safety Data Sheet     Potassium Iodide Solution	MSDS#:    639.00

i	Revision Date:   March 14,2001

SECTION 9 — PHYSICAL AND CHEMICAL PROPERTIES

Colorless, odorless solution. Concentration: 0.1 to 1.0 Molar

SECTION 10 — STABILITY AND REACTIVITY

Shelf life: Indefinite.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: N.A.	ORL-RAT LD50: N.A.

Chronic effects: N.A.	IHL-RAT LC50: N.A.

Target organs: N.A.	SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Data not yet available.

SECTION 13 — DISPOSAL CONSIDERATIONS

Please consult with state and local regulations.

Flinn Suggested Disposal Method #26b is one option.

SECTION 14 — TRANSPORT INFORMATION

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION

Not listed.

SECTION 16 — OTHER INFORMATION

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual for additional information about laboratory chemicals

© 1999 Flinn Scientific, Inc. All Rights Reserved.	PAGE 2 OF 2