SAFETY DATA SHEET

PortionPac® Chemical Corporation

400 N. Ashland Avenue, Chicago; IL 60622-6382 Voice: 312/226-0400 Fax; 312/226-5400

inernet: www:portionpaccorp.com

24 HOUR EMERGENCY

Response Phone

1 800-535-5053

SECTION 01 IDENTIFICATION MSDS-No;: SFS20  DATE: June 2000

Trade Name:  SFSPac Germicidal Detergent No. SFS20

Note:  CAS Registry numbers are not applicable to formulated products

SECTION 02 – PHYSICAL & HEALTH HAZARDOUS INGREDIENTS

Hazardous Material as defined by 29 CFR. 1910.1200 Reportable' under CERCLA or  SARA TITLE III Sec. 304 'Regulations

Common Names:		ethanolamine MEA monoethanolamine				CAS# 141-43-5

Chemical Names:		monoethanolamine, 2-aminoethanol

Hazard % in formula:		in concentrate:  7.95%	  in working/use solution: 0.031%

Hazard Reference:		3 ppm ACGIH 85/86 Ref (1) Sex. IX LD50 rats 10.2g/Kg  J IND HYG TOXICOL 23 259 (1941)

Common Name:		butyl cellosolve Poly Solv EB					CAS# 111-76-2

Chemical Name:	2-butoxyethanol

Hazard % in formula	in concentrate:  2.0%	   in working/use solution: 0.008%

Hazard Reference:	50 ppm OSHA PEL     25 ppm ACGIH TLV IDLH 700ppm

SECTION 02A OTHER INGREDIENTS NOT CONSIDERED HAZARDOUS IN FORMULATION

Water									CAS# 7732-18-5

n-alkyl dimethyl benzyl ammonium chloride				CAS# 68424-85-1

nonylphenoxy poly(ethyleneaxy) ethanol					CAS# 9016-45-9

tetrapotassium pyrophosphae						CAS# 7320-34-5

EDTA									CAS# 64-02-5

Trace colorant

SECTION 03 PHYSICAL & CHEMICAL CHARACTERISTICS

Boiling Point:			190 deg. F

Vapor Pressure:		19mm Hg

Vapor Density (Air=1)		Not determined

Water solubility			Complete

Melting/Freezing point:		<0.0 deg. F.

Appearance:			Red liquid slightly more viscous than water

Specific Grav. (water=1)	1.056

Percentage volatiles in con.	Approx.65%  due to water in formulation

Evaporation rate:		much slower than 1  (n-butyl acetate = 1)

pH:				in concentrate: 11.9 (+ -) .2	in working/use solution: 8.9 ( + - ) .3

Odor:		Fresh, antiseptic odor.

SECTION 04 PHYSICAL HAZARD DATA

Flash point:			214 deg. F.   Closed cup.

Flammable limits:			Not determined

Fire fighting media:			Water spray, CO2, dry chemcial - treat primary cause of fire.

Special fire fighting procedures:		N/A

Fire/Explosion Hazards:		No unusual hazards known.

SECTION 05 REACTIVITY DATA

Stability:			Stable

Hazard Polymerization:		Will not occur.

Conditions to Avoid:	Long exposure to materials containing copper aluminum and strong oxidinzing agents may cause decoloration

Incompatible Materials:	Strong oxidizing or reducing agents

Hazardous Decomposition products:	If heated to decomposition, CO, CO2, NOx and ammonia may be produced

SECTION 06  HEALTH HAZARD DATA

Oral Toxicity:	Not determined for formulation

Carcinogencity:	None of the individual materials in this formulation are listed as carcinogens in NTP, IARC, Monographs or are OSHA regulated carcinogens

SECTION 07  SYMPTOMS OF OVEREXPOSURE

Symptoms of Ingestion:	When concentrate is ingested, immediate burning pain in the mouth, throat, abdomen and possible severe swelling of the larynx.  Possible skeletal muscle paralysis affecting the ability to breathe with circulatory shock if large quantities  are ingested.  Possible convulsions.  May cause red blood cell hemolysis and possible liver and kidney injury.  May be fatal.

Symptoms of inhalation:	If misted in concentration form, which is improper use, can cause irritation of mucous membrane, nose, eye and throat.

Symptoms of Skin Contact:	May cause dermatitis or irritation in some individuals upon prolonged contact.  Localized skin defatting can be expected from concentrated material on long contact.  Harmful if absorbed through the skin.

Symptoms of Eye Contact:	Causes painful stinging or burning of eyes and lids, watering of eye conjunctivits and, in concentrated undiluted form may cause opaqueness of cornea, possible leading to loss of sight.  Corrosive.

SECTION 08  EMERGENCY FIRST AID PROCEDURES

For Ingestions:	DO NOT induce vomiting.  Immediately give 3-4 glasses of milk, egg whites or gelatin solution.  (If none, give water for dilution).  If vomiting occurs, give fluids again.  NEVER give anything to an unconscious person.  Call a physician or your local Poison Control Center.  Probable mucosal damage may contraindicate the use of gastic lavage.  Measures against circulatory shock as well as wxygen and measures to support breathing manually or mechanically may be neded.  If persistent, convulsions may be controlled by the cautious intravenous infection of a short acting barbiturate drug.

For Skin:	As for all foreign materials, wash off with copious amounts of water.  Remove clothing that has been saturated by concentrate.  Thoroughly wash affected clothing and shoes.

For Eyes:	PROMPTLY flush with large amount of water occasionally lifting the lower and upper lids.  Call  a physician if irritation persists.

Medical Conditions Aggravated by Exposure:	No data found.

SECTION 09   OCCUPATIONAL CONTROL PROCEDURES

Ventilation:	Use with adequate ventilation.  Working solution should not present any specific hazard.  If misted or aerosol generated local or mechanical exhaust recommended to maintain vapor concentration below TLV.  This level should not be reached under normal working conditions.

Respiratory Protection:	Not required under normal working/use conditions.

Eye Protection:	EPA recommends use of goggles to handle germicidal products.

Skin Protection:	EPA recommends use of gloves to handle germicidal products.

Personal Hygiene:	As in handling any germicidal detergent, wash thoroughly after using.

SECTION 10  PRECAUTIONS FOR SAFE HANDLING STORAGE AND USE

Precautionary Measures:	Avoid contact with eyes and prolonged contact of concentrate with skin.  Use with adequate ventilation.  Do not store at elevated temperatures.

Spills Clean-up Procedures:	Concentrated materials are packed in unit-dosed bags limiting any spills to very small quantities.  Paper toweling or mopping is usually sufficient.

Disposal Method:	Bags may be disposed of in normal waste.

Food Contamination:	Foods contaminated by germicides should be discarded and utensils, etc.  should be rinsed with potable water before use.

GENERAL NOTE ABOUT SFSPac  PRODUCTS

SFSPac Germicidal Detergent formulation is not substantially different form any other commercially available germicides.  The unique packaging of these materials in unit dosed bags limits the amount of exposure of the concentrate to very small amounts.  These can be cleaned up with paper toweling or plain mopping.  These are in fact products for mopping as well as other maintenance chores requiring disinfectants.  We know of no serious hazards associated with the proper use and handling of this product.  Portion PacChemical Corporation makes no warranty, expressed or implied, as to the accuracy, completeness or reliability of this information, except that such information is to the best of our knowledge and belief, accurate as of the date indicated.