MATERIAL SAFETY DATA SHEET

Revised: 09/05/00

HYDROGEN PEROXIDE, 3%	Replaces: None

CAROLINA BIOLOGICAL	Printed: 04/08/06

PRODUCT IDENTIFICATION

PRODUCT NAME:    HYDROGEN PEROXIDE, 3%

COMMON SYNONYMS: N/A

CHEMICAL FAMILY: AQUEOUS SOLUTIONS

FORMULA:	H202

FORMULA WT.:      34.01

CAS NO.:	7722-84-1

NIOSH/RTECS NO.: MX0890000

PRODUCT USE:     LABORATORY REAGENT

PRODUCT CODES:   2180,5102,2182

CHEMTREC # (800) 424-9300 NATIONAL RESPONSE CENTER # (800) 424-8802 J.T.BAKER INC.  222 RED SCHOOL LANE  PHILLIPSBURG, NJ  08865 24-HOUR EMERGENCY TELEPHONE -- (201) 859-2151 EFFECTIVE:  11/09/93    ISSUED: 04/08/95

REVISION #05

PRECAUTIONARY LABELING BAKER SAF-T-DATA* SYSTEM

HEALTH        -   0   NONE FLAMMABILITY  -   0   NONE REACTIVITY    -   1   SLIGHT CONTACT       -   1   SLIGHT

LABORATORY PROTECTIVE EQUIPMENT GOGGLES; LAB COAT

U.S. PRECAUTIONARY LABELING

DURING USE AVOID CONTACT WITH EYES, SKIN, CLOTHING.  WASH THOROUGHLY AFTER HANDLING.  WHEN NOT IN USE KEEP IN TIGHTLY CLOSED CONTAINER.

INTERNATIONAL LABELING

AVOID CONTACT WITH EYES.  AFTER CONTACT WITH SKIN, WASH IMMEDIATELY WITH PLENTY OF WATER.  KEEP CONTAINER TIGHTLY CLOSED.

SAF-T-DATA* STORAGE COLOR CODE: ORANGE (GENERAL STORAGE)

COMPONENTS

COMPONENT	CAS NO.     WEIGHT %   OSHA/PEL

ACGIH/TLV

HYDROGEN PEROXIDE	7722-84-1   2-4         1    PPM   1

PPM

WATER	7732-18-5.  96-98      N/E        N/E

CONTAINS A TRACE OF	ACETOPHENETIDIN AS PRESERVATIVE CAS NO. 62-44-2

PHYSICAL DATA

BOILING POINT: N/A	VAPOR PRESSURE (MMHG): N/A

MELTING POINT: N/A	VAPOR DENSITY (AIR=1): N/A

SPECIFIC GRAVITY: 1.01	EVAPORATION RATE: N/A

(H20=l)

SOLUBILITY(H20) : COMPLETE (100%)	% VOLATILES BY VOLUME: 100

(21 C) PH: N/A

ODOR THRESHOLD (P.P.M.): N/A	PHYSICAL STATE: LIQUID

Page	1 of 5

MATERIAL SAFETY DATA SHEET

Revised: 09/05/00

HYDROGEN PEROXIDE, 3%	Replaces: None

CAROLINA BIOLOGICAL	Printed: 04/08/06

COEFFICIENT WATER/OIL DISTRIBUTION: N/A

APPEARANCE & ODOR: CLEAR, COLORLESS LIQUID. SLIGHTLY ACRID ODOR.

FIRE AND EXPLOSION HAZARD DATA

FLASH POINT (CLOSED CUP) : N/A

AUTOIGNITION TEMPERATURE: N/A

FLAMMABLE LIMITS:   UPPER - N/A        LOWER - N/A

FIRE EXTINGUISHING MEDIA

USE EXTINGUISHING MEDIA APPROPRIATE FOR SURROUNDING FIRE. SPECIAL FIRE-FIGHTING PROCEDURES

FIREFIGHTERS SHOULD WEAR PROPER PROTECTIVE EQUIPMENT AND SELF-CONTAINED

BREATHING APPARATUS WITH FULL FACEPIECE OPERATED IN POSITIVE PRESSURE

MODE. UNUSUAL FIRE & EXPLOSION HAZARDS

NONE IDENTIFIED. TOXIC GASES PRODUCED

NONE IDENTIFIED EXPLOSION DATA-SENSITIVITY TO MECHANICAL IMPACT

NONE IDENTIFIED. EXPLOSION DATA-SENSITIVITY TO STATIC DISCHARGE

NONE IDENTIFIED.

HEALTH HAZARD DATA

THRESHOLD LIMIT VALUE (TLV/TWA):  1.4 MG/M      (1 PPM) TLV IS FOR HYDROGEN PEROXIDE.

SHORT-TERM EXPOSURE LIMIT (STEL): NOT ESTABLISHED PERMISSIBLE EXPOSURE LIMIT (PEL): 1.4 MG/M      (1 PPM) PEL IS FOR HYDROGEN PEROXIDE.

TOXICITY OF COMPONENTS

ORAL RAT LD50 FOR HYDROGEN PEROXIDE	1518

MG/KG

INTRAPERITONEAL MOUSE LD50 FOR WATER	190

G/KG

INTRAVENOUS MOUSE LD50 FOR WATER	25

G/KG

CARCINOGENICITY:   NTP: NO    IARC: YES     Z LIST: YES  OSHA REG: YES

CARCINOGENICITY

THIS PRODUCT CONTAINS A TRACE OF PHENACETIN (CAS NO. 62-44-2) , WHICH IS

LISTED BY IARC,  GROUP 2A, AS PROBABLY CARCINOGENIC TO HUMANS. ACCORDING

TO NTP, THERE IS LIMITED EVIDENCE FOR CARCINOGENICITY OF PHENACETIN IN

HUMANS. PHENACETIN IS MUTAGENIC TO SALMONELLA TYPHIMURIUM  IN THE PRESENCE

OF A HAMSTER LIVER MICROSOME PREPARATION, AND IT PRODUCES CHROMOSOME

ABERRATIONS IN CHINESE HAMSTER FIBROBLASTS IN VITRO. REPRODUCTIVE EFFECTS

NONE IDENTIFIED. EFFECTS OF OVEREXPOSURE

INHALATION:       NONE IDENTIFIED

Page 2 of 5

MATERIAL SAFETY DATA SHEET

HYDROGEN PEROXIDE, 3% CAROLINA BIOLOGICAL

Revised: 09/05/00 Replaces: None Printed: 04/08/06

SKIN CONTACT:

NONE IDENTIFIED NONE IDENTIFIED NONE IDENTIFIED NONE IDENTIFIED IDENTIFIED

EYE CONTACT:

SKIN ABSORPTION:

INGESTION:

CHRONIC EFFECTS: TARGET ORGANS

NONE

EYES, SKIN, RESPIRATORY SYSTEM, LUNGS MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE

NONE IDENTIFIED PRIMARY ROUTES OF ENTRY

NONE INDICATED EMERGENCY AND FIRST AID PROCEDURES

INGESTION: GIVE

INHALATION: EXPOSED

SKIN CONTACT:

OF

EYE CONTACT:

OF

IF SWALLOWED AND THE PERSON IS CONSCIOUS, IMMEDIATELY

LARGE AMOUNTS OF WATER.  GET MEDICAL ATTENTION. IF A PERSON BREATHES IN LARGE AMOUNTS, MOVE THE

PERSON TO FRESH AIR.

IN CASE OF CONTACT, IMMEDIATELY WASH SKIN WITH PLENTY

SOAP AND WATER FOR AT LEAST 15 MINUTES.

IN CASE OF EYE CONTACT, IMMEDIATELY FLUSH WITH PLENTY

WATER FOR AT LEAST 15 MINUTES.

SARA/TITLE III HAZARD CATEGORIES AND LISTS YES CHRONIC: YES FLAMMABILITY: NO  PRESSURE: NO  REACTIVITY: NO

ACUTE:

YES  CONTAINS HYDROGEN PEROXIDE (RQ = 1

EXTREMELY HAZARDOUS SUBSTANCE: LB, TPQ

CERCLA HAZARDOUS SUBSTANCE: (45 .4

= 1,000 LBS) YES  CONTAINS PHENACETIN (RQ = 100 LBS

KG)

SARA 313 TOXIC CHEMICALS:       NO

TSCA INVENTORY:	YES

STATE LISTS: FOR PRODUCTS SOLD IN THE STATE OF CALIFORNIA, THE STATE

REQUIRES

THAT WE PROVIDE TO USERS AND THEIR EMPLOYEES THE FOLLOWING MESSAGE:

WARNING:

THIS PRODUCT CONTAINS A CHEMICAL KNOWN TO THE STATE OF CALIFORNIA TO

CAUSE

CANCER.

REACTIVITY DATA

HAZARDOUS POLYMERIZATION: WILL NOT

STABILITY: STABLE

OCCUR

CONDITIONS TO AVOID:

LIGHT, HEAT

AMMONIA, CARBONATES, ORGANIC MATERIALS DECOMPOSITION PRODUCTS: NONE IDENTIFIED

SPILL & DISPOSAL PROCEDURES

STEPS TO BE TAKEN IN THE EVENT OF A SPILL OR DISCHARGE

WEAR SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE CLOTHING. STOP

Page 3 of 5

INCOMPATIBLES:

MATERIAL SAFETY DATA SHEET

HYDROGEN PEROXIDE, 3% CAROLINA BIOLOGICAL

Revised: 09/05/00 Replaces: None Printed: 04/08/06

LEAK IF YOU CAN DO SO WITHOUT RISK.  USE WATER SPRAY TO REDUCE VAPORS.

TAKE UP WITH SAND OR OTHER NON-COMBUSTIBLE ABSORBENT MATERIAL AND PLACE

INTO CONTAINER FOR LATER DISPOSAL.  FLUSH SPILL AREA WITH WATER. DISPOSAL PROCEDURE

DISPOSE IN ACCORDANCE WITH ALL APPLICABLE FEDERAL, STATE, AND LOCAL

ENVIRONMENTAL REGULATIONS.

INDUSTRIAL PROTECTIVE EQUIPMENT

VENTILATION:	USE GENERAL OR LOCAL EXHAUST VENTILATION TO MEET

TLV

REQUIREMENTS. RESPIRATORY PROTECTION: RESPIRATORY PROTECTION REQUIRED IF AIRBORNE

CONCENTRATION EXCEEDS TLV.  AT CONCENTRATIONS UP TO 1

PPM, A DUST/MIST RESPIRATOR IS RECOMMENDED.

THIS LEVEL, A SELF-CONTAINED BREATHING APPARATUS

ADVISED.

SAFETY GOGGLES, UNIFORM, APRON, PROPER GLOVES

ABOVE IS

EYE/SKIN PROTECTION: ARE

RECOMMENDED .

STORAGE AND HANDLING PRECAUTIONS

SAF-T-DATA* STORAGE COLOR CODE: ORANGE (GENERAL STORAGE) STORAGE REQUIREMENTS

KEEP CONTAINER TIGHTLY CLOSED.  SUITABLE FOR ANY GENERAL CHEMICAL STORAGE

AREA.

TRANSPORTATION DATA AND ADDITIONAL INFORMATION

DOMESTIC (D.O.T.)

PROPER SHIPPING NAME:  CHEMICALS, N.O.S. (NON-REGULATED)

INTERNATIONAL (I.M.O.)

PROPER SHIPPING NAME:  CHEMICALS, N.O.S.  (NON-REGULATED) MARINE POLLUTANTS: NO

AIR (I.C.A.O.)

PROPER SHIPPING NAME:

CHEMICALS, N.O.S.  (NON-REGULATED)

U.S. CUSTOMS HARMONIZATION NUMBER: 28470000003

NOTE:  WHEN HANDLING LIQUID PRODUCTS,  SECONDARY PROTECTIVE CONTAINERS MUST BE USED FOR CARRYING. N/A = NOT APPLICABLE, OR NOT AVAILABLE N/E = NOT ESTABLISHED.   THE INFORMATION IN THIS MATERIAL SAFETY DATA SHEET MEETS THE REQUIREMENTS O THE UNITED STATES OCCUPATIONAL SAFETY AND HEALTH ACT AND REGULATIONS PROMULGATED THEREUNDER (29 CFR 1910.1200 ET. SEQ.) AND THE CANADIAN WORKPLACE HAZARDOUS MATERIALS INFORMATION SYSTEM. THIS DOCUMENT IS INTENDED ONLY AS A GUIDE TO THE APPROPRIATE PRECAUTIONARY HANDLING OF THE MATERIAL BY A PERSON TRAINED IN, OR SUPERVISED BY A PERSON TRAINED IN, CHEMICAL HANDLING.  THE USE IS RESPONSIBLE FOR DETERMINING THE PRECAUTIONS AND DANGERS OF THIS CHEMICAL FOR HIS OR HER PARTICULAR APPLICATION. DEPENDING ON USAGE, PROTECTIVE

Page 4 of 5

MATERIAL SAFETY DATA SHEET

Revised: 09/05/00

HYDROGEN PEROXIDE, 3%	Replaces: None

CAROLINA BIOLOGICAL .	Printed: 04/08/06

clothin including eye and face guards and respirators must be used to avoid contact with material or breathing chemical vapors/fumes. exposure to this product may have serious adverse health effects. this chemical may interact with other substances. since the potential uses are so varied, baker cannot warn of all of the potential dangers of use or interaction with other chemicals or materials. baker warrants that the chemical meets the specifications set forth on the label.  baker disclaims any other warranties, expressed or implied, with regard to the product supplied hereunder, its merchantability or its fitness for a particular purpose.  the user should recognize that this product can cause severe injury and eve death, especially if improperly handled or the known dangers of use are not heeded. read all precautionary information. as new documented general safety information becomes available, baker will periodically revise this material safety data sheet.  note: chemtrec, canutec, and national response center 'emergency telephone numbers are to be used only in the event of chemical emergencies involving a spill, leak, fire, exposure, or accident involving chemicals. all non-emergency questions should be directed to customer service (1-800-jtbaker assistance. copyright 1995 j.t.baker inc. * trademarks of j.t.baker inc. approved by quality assurance department.