MATERIAL SAFETY DATA SHEET: MB-215/ANCOCIDE 4070

Section I - General Information

(000000-000000-   -OT3Z)

Date of Issue:

11/9/2005 12:00:00 AM

Chemical Name & Synonyms:

N/A

Chemical Family:

AQUEOUS ISOTHIAZOLINE SOLUTION

Manufacturer Name:

CHEM-AQUA, INC

Manufacturer Address:

BOX 152170 IRVING, TEXAS 75015

Prepared By:

LBoynton/Chemist

Product Code Number:

C537

Supercedes:

1/10/2005 12:00:00 AM

Trade Name & Synonyms:

MB-215/ANCOCIDE 4070

Formula is a mixture: [V]

Emergency Phone Number:

800-424-9300

Section II - Hazardous Ingredients

THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

Chemical Name (Ingredients)

5-CHLORO-2-METHYL-4-ISOTHIAZOLIN-3-ONE 2-METHYL-4-ISOTHIAZOLIN-3-ONE MAGNESIUM NITRATE MAGNESIUM CHLORIDE

Hazard

CORROSIVE CORROSIVE IRRITANT IRRITANT

STEL       CAS #

TLV

PEL

N/E 1	N/E2	N/E	26172-55-4

N/E 1	N/E 2	N/E	2682-20-4

N/E1	N/E 2	N/E	10377-60-3

N/E 1	N/E 2	N/E	7786-30-3

Section III - Physical Data

Boiling Point (°F):	212°

Vapor Pressure (mm Hg):	16.8

Vapor Density (Air=1):	0.6

pH @ 100% :	2-4

% Volatile by Volume:	97.5

H20 Solubility:	COMPLETE

Specific Gravity (H20=1):	1.02-1.04

Color:	PALE GREEN

Odor:	MILD ORGANIC

Clarity:	TRANSPARENT

Evaporation Rate (BuAc=1):	0.58

Viscosity:	NON-VISCOUS

Section IV - Fire and Explosion Hazard

Flash Point: >200°F	Method Used: N/A

Flammable Limits: HYDROGEN GAS	DEL: 75%

LEL: 4%	Aerosol Level (NFPA SOB): N/A

Extinguishing Media:

[V] Foam	[V] Alcohol Foam         [V] C02

[V] Dry Chemical         [V] Water Spray	[ ] Other

NFPA 704 Hazard Rating:

4-Extreme 3-High 2-Moderate 1-Slight 0-lnsignificant

Health: 3 Flammability: 1 Instability: 0 Special:

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF THE SURROUNDING FIRE. COOL FIRE-EXPOSED CONTAINERS WITH WATER SPRAY TO PREVENT BURSTING.

Unusual Fire and Explosion Hazards:

1 of 4

MATERIAL SAFETY DATA SHEET: MB-215/ANCOCIDE 4070

PROLONGED CONTACT WITH REACTIVE METALS, SUCH AS ALUMINUM, COPPER, BRASS, BRONZE, CHROMIUM, MAGNESIUM, TIN, ZINC, AND ALLOYS, CAN CAUSE THE FORMATION OF FLAMMABLE HYDROGEN GAS WHICH CAN FORM AN EXPLOSIVE MIXTURE WITH AIR. DO NOT ALLOW SOLUTION TO EVAPORATE TO DRYNESS. DRY NITRATES IN COMBINATION WITH ORGANIC MATERIAL CAN BE EXPLOSIVE. THE USE OF WATER SPRAY (FOG) WHILE EFFECTIVE, MAY CAUSE FROTHING AND FOAMING. NEVER USE A WATER JET AS THIS WILL JUST SPREAD THE FIRE. USE CARE AS SPILLS MAY BE SLIPPERY.

Section V - Health and Hazard Data

Threshold Limit Value:

NOT ESTABLISHED.

Effects of Overexposure:

Acute: (Short Term Exposure)

EYE CONTACT: CORROSIVE. CAUSES BURNS, CORNEAL DAMAGE, AND POSSIBLE BLINDNESS. SKIN CONTACT: CORROSIVE. CAUSES BURNS AND POSSIBLE DEEP ULCERATIONS OR SCARRING. MAY CAUSE ALLERGIC SKIN REACTION SEEN AS DELAYED SKIN RASH WHICH MAY BE FOLLOWED BY BLISTERING, SCALING, AND OTHER SKIN EFFECTS. INHALATION: CAUSES BURNS TO THE RESPIRATORY TRACT, NOSE, MOUTH, AND THROAT WITH DISCOMFORT, NASAL DISCHARGE, SNEEZING, COUGHING, RAPID HEARTBEAT, AND CHEST PAIN. INHALATION MAY CAUSE CHEMICAL PNEUMONITIS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL. INGESTION: CORROSIVE. CAUSES BURNS TO THE MOUTH, THROAT, ESOPHAGUS, AND STOMACH WITH NAUSEA AND PAIN. SYMPTOMS MAY INCLUDE VOMITING OF BLOOD. BLOOD LOSS THROUGH DAMAGED TISSUE CAN LEAD TO LOW BLOOD PRESSURE AND SHOCK, AND MAY BE FATAL.

Chronic: (Long Term Exposure)

MAY CAUSE SKIN SENSITIZATION IN SOME INDIVIDUALS. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA, AND DERMATITIS. TARGET ORGANS: BLOOD-FORMING ORGANS. THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

Primary Routes of Entry	1

[] Inhalation	[ ] Ingestion	[V] Absorption	I

Emergency First Aid Procedures:

Inhalation:

MOVE PERSON TO FRESH AIR. IF PERSON IS NOT BREATHING, CALL 911 OR AN AMBULANCE, THEN GIVE ARTIFICIAL RESPIRATION, PREFERABLY MOUTH-TO-MOUTH, IF POSSIBLE. CALL A POISON CONTROL CENTER OR DOCTOR FOR FURTHER TREATMENT ADVICE.

Eye Contact:

HOLD EYE OPEN AND RINSE SLOWLY AND GENTLY WITH WATER FOR 15-20 MINUTES. REMOVE CONTACT LENSES, IF PRESENT, AFTER THE FIRST 5 MINUTES, THEN CONTINUE RINSING, ETC. CALL A POISON CONTROL CENTER OR DOCTOR FOR TREATMENT ADVICE.

Skin Contact:

TAKE OFF CONTAMINATED CLOTHING. RINSE SKIN IMMEDIATELY WITH PLENTY OF WATER FOR 15-20 MINUTES. CALL A POISON CONTROL CENTER OR DOCTOR FOR TREATMENT ADVICE.

Ingestion:

CALL A POISON CONTROL CENTER OR DOCTOR IMMEDIATELY FOR TREATMENT ADVICE. HAVE PERSON SIP A GLASS OF WATER IF ABLE TO SWALLOW. DO NOT INDUCE VOMITING UNLESS TOLD TO DO SO BY A POISON CONTROL CENTER OR DOCTOR. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS PERSON.

Notes to Physician:

PROBABLE MUCOSAL DAMAGE MAY CONTRAINDICATE THE USE OF GASTRIC LAVAGE. MEASURES AGAINST CIRCULATORY SHOCK, RESPIRATORY DEPRESSION, AND CONVULSIONS MAY BE NEEDED. IN SEVERE CASES OF AN ALLERGIC REACTION, ANAPHYLACTIC SHOCK MAY OCCUR. HAVE THE PERSON LIE DOWN WITH THEIR LEGS ABOVE THEIR CHEST TO INCREASE BLOOD FLOW TO THE HEART

Section VI - Toxicity Information

r- Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

[ ] IARC	[ ] NTP	[ ] OSHA	[ ] ACGIH	[ ] Other

VOC CONTENT: 0% BY WEIGHT, 0% BY VOLUME, 0 G/L

PRODUCT MIXTURE ORL-RAT LD50(FEMALE): 3310 MG/KG 3. ORL-RAT LD50(MALE): >5000 MG/KG 3. SKN-RBT IRRITATION: CORROSIVE 3. EYE-RBT IRRITATION: CORROSIVE 3. SKN-RBT: SENSITIZER 3.

5-CHLORO-2-METHYL-4-ISOTHIAZOLIN-3-ONE NO TOXICITY DATA AVAILABLE

MATERIAL SAFETY DATA SHEET: MB-215/ANCOCIDE 4070

~

2-METHYL-4-ISOTHIAZOLIN-3-ONE NO TOXICITY DATA AVAILABLE

MAGNESIUM NITRATE

NO TOXICITY DATA AVAILABLE

MAGNESIUM CHLORIDE ORL-RAT LD50: 2800 MG/KG 4.

Section VII - Reactivity Data

Stability

[V] Stable	[ ] Unstable

Conditions to Avoid:

AVOID HEAT, HOT SURFACES, SPARKS, AND OPEN FLAMES.

Hazardous Polymerization

[V] Will not occur	[ ] May occur

Conditions to Avoid:

N/A

Incompatibility (Materials to Avoid):

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; REDUCING AGENTS SUCH AS THIOSULFATE, AMINES, AND MERCAPTANS. PROLONGED CONTACT WITH REACTIVE METALS, SUCH AS ALUMINUM, COPPER, BRASS, BRONZE, CHROMIUM, MAGNESIUM, TIN, ZINC, AND ALLOYS, CAN CAUSE THE FORMATION OF FLAMMABLE HYDROGEN GAS WHICH CAN FORM AN EXPLOSIVE MIXTURE WITH AIR.

Hazardous Decomposition Products:

OXIDES OF CARBON, NITROGEN, AND SULFUR; HYDROGEN CHLORIDE.

Section VIII - Spill Or Leak Procedures

r

Steps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY. SHUT OFF SOURCE OF LEAK. DIKE AND CONTAIN SPILL. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. THIS PRODUCT IS TOXIC TO FISH. PREVENT PRODUCT FROM CONTAMINATING SOIL OR FROM ENTERING SEWAGE AND DRAINAGE SYSTEMS AND BODIES OF WATER. FLUSH AREA WITH WATER.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS. PESTICIDE WASTES ARE ACUTELY HAZARDOUS. IMPROPER DISPOSAL OF EXCESS PESTICIDE OR RINSATE IS A VIOLATION OF FEDERAL LAW. IF THESE WASTES CANNOT BE DISPOSED OF BY USE ACCORDING TO LABEL INSTRUCTIONS, CONTACT YOUR STATE PESTICIDE OR ENVIRONMENTAL CONTROL AGENCY.

Neutralizing Agent:

NEUTRALIZE SPILL AREA WITH FRESHLY PREPARED SOLUTION OF 5% SODIUM BICARBONATE AND 5% SODIUM HYPOCHLORITE IN WATER. APPLY SOLUTION TO THE SPILL AREA AT A RATIO OF 10 VOLUMES NEUTRALIZATION SOLUTION PER ESTIMATED VOLUME OF RESIDUAL SPILL. LET STAND 30 MINUTES BEFORE REMOVAL.

Section IX - Special Protection Information

Required Ventilation:

MISTS. LOCAL VENTILATION IS PREFERRED, BECAUSE IT PREVENTS DISPERSION INTO WORK AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (Z88.2-1992). FOR CONCENTRATIONS ABOVE THE TLV AND/OR PEL BUT LESS THAN 10 TIMES THESE LIMITS, A NIOSH APPROVED HALF-FACEPIECE RESPIRATOR EQUIPPED WITH APPROPRIATE CHEMICAL CARTRIDGES MAY BE USED. FOR CONCENTRATIONS GREATER THAN 10 TIMES THE TLV AND/OR PEL, CONSULT THE NIOSH RESPIRATOR DECISION LOGIC FOUND IN PUBLICATION NO. 87-116 OR ANSI Z88.2-1992.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES SHOULD BE WORN. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR HAND PROTECTION, 29 CFR 1910.138.

Eye Protection:

CHEMICAL GOGGLES AND A FACE SHIELD SHOULD BE WORN WHEN HANDLING. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE PROTECTION, 29 CFR 1910.133.

Other Protection:

WEAR PROTECTIVE CLOTHING WHEN HANDLING. A SAFETY SHOWER AND AN EYEWASH STATION SHOULD BE AVAILABLE.

3 of 4

MATERIAL SAFETY DATA SHEET: MB-215/ANCOCIDE 4070 Section X - Storage and Handling Information

r Storage Conditions	—	-	—	-	-	—			—	-

I [V] Indoors	[ ] Outdoors	[ ] Heated	[ ] Refrigerated

r

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. DO NOT STORE IN UNLINED METAL CONTAINERS. PRODUCT IS CORROSIVE TO MILD STEEL. DO NOT CONTAMINATE FOOD, FEED, OR NATURAL BODIES OF WATER. KEEP FROM FREEZING. IF PRODUCT FREEZES ALLOW IT TO SLOWLY WARM TO ROOM TEMPERATURE AND STIR THOROUGHLY BEFORE USING.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

Section XI - Regulatory Information

Chemical Name	CAS Number	Upper % Limit

NITRATE COMPOUNDS	N/A	5

Those Ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you area California customer. ThisMSDS is not intended for users in the state of

California.

Section XII - References

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 2005.

- OSHA PEL. 3. VENDOR'S MSDS. 4. REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFOWeb, 2005. ALL THE
COMPONENTS OF THIS PRODUCT ARE IN COMPLIANCE WITH THE TOXIC SUBSTANCES CONTROL ACT (TSCA) AND ARE EITHER LISTED
ON THE TSCA INVENTORY OR OTHERWISE EXEMPTED FROM LISTING. IRR:IRRITANT, FLAM/FLAMM:FLAMMABLE, COMB:COMBUSTIBLE,
CORR:CORROSIVE, CARC:CARCINOGENIC, TOXTOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC:CLEVELAND OPEN CUP,
PMCC:PENSKY-MARTIN CLOSED CUP, TCC:TAGLIABUE CLOSED CUP, LELLOWER EXPLOSION LIMIT, UELiUPPER EXPLOSION LIMIT,
NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC:INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL
TOXICOLOGY PROGRAM, OSHA:OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION, ACGIHiAMERICAN CONFERENCE OF
GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLVTHRESHOLD LIMIT VALUE, PELPERMISSIBLE EXPOSURE LIMIT, STEL:SHORT-TERM
EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX:ASPHYXIANT, PNOS:PARTICLES (INSOLUBLE) NOT
OTHERWISE SPECIFIED, SDT:STANDARD DRAIZE TEST, ORLORAL, IHLINHALATION, HMN:HUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

CHEM-AQUA, INC assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label. Users assume all risks associated with such unrecommended use, storage, or disposal of the product.

©2006 NCH Corporation All rights reserved.

•~