MATERIAL SAFETY DATA SHEET

(SIMILAR TO OSHA FORM 174)

N.A.=Not Applicable Not Est.=Not Established Prop.=Proprietary

SECTION 1-MANUFACTURER/DISTRIBUTOR INFORMATION

Product Name: Common Scents #4 HD Tile & Grout Cleaner

Supplier's Name: Seven Oaks Supply Co.

Supplier's Address:  120 East 3rd Street • Oakboro, NC 28129	Emergency Phone #: (800) 230-7460

HMIS Hazard Code: Health:2   Flammability:0   Reactivity:0   Other:0	Chemical Name: Alkaline Detergent

Chemical Family: Detergent Blend	Formula: N.A. Blended Product

D.O.T. Shipping Name: CORROSIVE LIQUID n.o.s. (contains potassium hydroxide)

Hazard Class: 8	D.O.T. ID#: UN1760

SECTION 2-HAZARDOUS INGREDIENTS

CHEMICAL NAME

Polyethylene Mono (Nonylphenyl) Ether Glycols Potassium Hydroxide 2-Butoxy Ethanol Dodecyl-Benzenesuifonic Acid, Sodium Salt

CAS NO.

9016-45-9

1310-58-3

111-76-2

25155-30-0

% Concentrate Range

PROP. PROP. PROP. PROP.

Hazard Ratings (TLV/PEL)

Not Est. 2 mg/m3 25 ppm Not Est.

| SECTION 3-PHYSICAL DATA |  | SECTION 4-FIRE AND EXPLOSION DATA |
| --- | --- | --- |
| Boiling Point (F): 212° Vapor Pressure: = To Water Vapor Density: = To Water Solubility In Water: Complete Appearance: Clear, Thin, Green Liquid Odor: Buttery Citrus | Specific Gravity: 1 .044 Evaporation Rate: = To Water | Flash Point (F): None to boiling (TOC) Flammable Limits: N.A. Unusual Fire & Explosion Hazards: None |

SECTION 5-HEALTH HAZARD DATA

Route(s) of Entry:  lnhalation:No   Skin:Yes   lngestion:Yes	Threshold Limit Value:  Not Est.

Effects of Overexposure: Causes skin and eye irritation. Harmful if swallowed. May cause severe mucosal damage and gastric

distress. May cause systemic intoxication.

Emergency & First Aid Procedures:  For Eyes:  Flush with water for at least fifteen minutes

Get medical aid. For Skin: Thoroughly Do not induce vomiting. Give milk, egg

wash with water and apply emollient creme. If irritation persists get medical aid.   If Ingested:

white, gelatin, or water. Get immediate medical aid.

Conditions Aggravated by Exposure:   May aggravate existing skin conditions.

Listed Carcinogen, Mutagen, Teratogen (Ingredient, Source, Amount): None

| SECTION 6-REACTIVITY DATA | SECTION 7-SPILL OR LEAK PROCEDURES |
| --- | --- |
| Stability: Stable Conditions to Avoid: None Incompatibility: Acids and Oxides Hazardous Decomposition Products: Carbon Oxidizers and Amines Hazardous Polymerization: Will not occur. | Steps to be taken if material is released or spilled: Recontainerize by mopping, wet vacuum, or suitable absorbent. Rinse remaining material to drains. Be cautious of slippery floors. Waste disposal method: Follow local, state, and federal . regulations. |
| SECTION 8-SPECIAL PROTECTION INFORMATION | SECTION 9-SPECIAL PRECAUTIONS |
| Respiratory Protection: Generally not Required. Ventilation: General (in order to maintain air-borne concentrations below OSHA limits) Protective Gloves: Rubber or latex. Eye Protection: Goggles or safety glasses with side shields Other Protective Equipment: Rubber boots should be worn if working in standing solution. | Precautions to be taken in handling and storage: Store between 40° and 1 20° F. Other Precautions: Keep out of reach of children. For sale to, use and storage by service personnel only. |

The information contained in this MSDS was obtained from current and reliable sources, however the data is provided without any warranty, expressed
or implied, regarding its correctness or accuracy. Since the conditions or handling, storage and disposal of this product are beyond the control of this
company, it is not responsible for loss, injury and expense arising out of the product's improper use. No warranty, expressed or inferred, regarding the
product described in this MSDS shall be created or inferred by any statement in this MSDS. Various government agencies may have specific regula
tions regarding the transportation, handling, storage, use or disposal of this product which may not be covered by this MSDS. The user is responsible
for full compliance.

DATE PREPARED-10/5/00   By: G.C.