Material Safety Data Sheet

QUICK IDENTIFIER (In FUal Coiamon Nun.|

HAN-I-SIZE

| Name' 'momar, incorporated | Emergency Phone No. | 404-355-4580 |
| --- | --- | --- |
| Address 1830 ELLSWORTH INDUSTRIAL DRIVE ATLANTA GA 30318 | Informition Call | 404-355-4580 |
| te£^£rf£«ium /^,tu^ .Xfasrt&G | Date Prepared | 9/6/89 |

SECTION 1 - IDENTITY

Common Name	HAN-I-SIZE

Not applicable

Chemical Family Alcohol/Surfactants Blend

Formula:  Aqueous mixture  - waterless instant   hand  and arm sanitizer

SECTION 2 - HAZARDOUS INGREDIENTS

Hm/dom Comporv«nU>l

CAS No.

>\  liy   Weight

Thrtlhold Limit V«lu« [\uuUI

1)   Ethyl Alcohol

64-17-5

60.0

(ACGIH-TON)   1000 pprn

(LD50 oral   rats«=7060 mp/kg,

dermal   rabbits-20

'LC50   inhalation   rats

20,000 ppm/lOH)

SECTION 3 - PHYSICAL & CHEMICAL CHARACTERISTICS (Fire & Explosion Data)

Boiling Pont 165 -  173'F

Specific Gravity

0.900

Vapor Pressure Not applicable

Percent Volatile      99.5

Complete

Solubility in Water

Vapor Density in air Not Applicable

Evaporation Rate  Slower

Reactivity in Water  None

Appearance and Odor

Colorless, clear chick liquid with lemon odor

Flash Point  7 2 ° F

Specific Gravity  0.900

Vapor Pressure  Not Applicable.
Special Fire Fighting Procedures	'

Use water spray to keep fire exposed containers cool, to disperse vapors, and to flush spills from exposures, Firefighters wear self-contained breathing apparatus with full face piece in pressure demand or other positive pressure mode.

Unusual Fire and explosion hazards

In case of spillage, flush with water to avoid slipperiness.

han-i-size

SECTION 4 - PHYSICAL HAZARDS

Stability, Conditions to Avoid:  Excessive heat, sparks, open flame or welding arcs or torches.

Incompatability:  Strong oxidizing agents.

Hazardous decomposition Products:  Thermal decomposition gives off oxides of carbon

Hazardous Polymerization.:  Conditions to Avoid:  None

SECTION 5 - HEALTH HAZARDS

Threshold Limit Value:  1000 ppm (ACCGIH-TWA for Ethyl Alcohol)

Signs and symptoms of Exposure, Acute Overexposure:  May Cause eye irritation

Chronic Overexposure:  None anticipated

Chemical Listed as Carcinogen or Potential Carcinogen: None

Osha Permissible Exposure Limit:  1000 ppm

ACHIH Threshold Limit Value  1000 ppm

Other Exposure Limit Used:  Not established

Emergency and First Aid Procedures;  Inhalation, no emergency care anticipated

Eyes:  Flush with plenty of water for at least 15 minutes.  If irritation persisits, get medical attention.

Skin:  No emergency care anticipated.  Should user show a sensitivity to this product, treat symptomatically.

Ingestion:  No emergency care anticipated.  Treat symptomatically

SECTION 6 - SPECIAL PROTECTION INFORMATION

Respriatory Protection Required	None   required

Vemitlatin:  Local exhaust is adequate, Mechanical or General not required

Protective Equipment:  Gloves not applicable

Eye Protection:  Not required

Other Protective Clothing or Equipment:  Not Required

SECTION 7 - SPECIAL PRECAUTIONS AND SPILL/LEAK PROCEDURES

Precautions to be Taken in Handling and Storage:  Do not contact eyes.

Other Precautions:  Do not store near excessive heat, spark, or open flames.

Steps to be Taken in Case Material is Released or Spilled:  Absorb on mineral clay absorbent material.

Waste Disposal Methods:  To dispose of, check with federal, state, and local regulations.