Blue Circle Cement Inc.

One South Executive Park 6060 St. Albans Street, Suite 502 Charlotte, North Carolina 28287 Telephone: (704) 553-7101 Facsimile: (704) 552-6

MATERIAL SAFETY DATA SHEET SECTION I

1.	Manufacturer's name and address:  Blue Circle Cement Inc.

2520 Paul Avenue, N. W. Atlanta, GA  30318

- Emergency telephone:  (404) 792-2361

- Chemical name and synonyms :  Portland and Masonry Cements

- Trade name and synonyms:  Magnolia

- Chemical family:  Calcium

- Formulae:  3CaO Si02	(CAS #12168-85-3)

2CaO Si02	(CAS #10034-77-2)

3CaO A1203	(CAS #12042-78-3)

4CaO A1203 Fe203	(CAS #12068-35-8)

CaS04 2H20	(CAS #13397-24-5)

plus traces of CaO,	MgO , K2SO4, and Na2SO4

SECTION II - HAZARDOUS INGREDIENTS

7 .   N.A.

SECTION III - PHYSICAL DATA

- Boiling point:  N.A.

- Vapor pressure:  N.A.

- Vapor density:  N.A.

- Solubility in water:  Slightly (0.1 - 1%)

- Specific gravity:  3.15

- Percentage volatile by volume:  N.A.

- Evaporation rate:  N.A.

- Appearance and odor:  Gray (or white) powder; odorless

SECTION IV - FIRE AND EXPLOSION HAZARD DATA

- Flash point:  N.A.

- Flammable or explosive limits:  N.A.

- Extinguishing media:  N.A.

- Special firefighting procedures:  N.A.

- Unusual fire and explosion hazards:  None

SECTION V - HEALTH HAZARD DATA

21.	Threshold limit value:

5 mg m (respirable dust) 10 mg m (total dust) 30 million particles per cubic foot

22.	Effects of overexposure:

Acute:  Wet cement, especially as an ingredient in plastic (unhardened) concrete, can dry the skin and cause alkali burns.  Cement dust can irritate the eyes and upper respiratory system.

Chronic:  Cement dust can cause inflammation of the lining tissue of the interior of the nose and inflammation of the cornea.  Hypersensitive individuals may develop an allergic dermatitis.  (Cement may contain traces of hexavelent chromium.)

23.	Emergency and first-air procedures:  Irrigate eyes with water.

Wash affected areas of the body with soap and water.

SECTION VI - REACTIVITY DATA

24.	Stability:  Product is stable.  Although no hazardous reaction

will occur; Portland cement should be kept dry until used.

- Incompatibility:  None

- Hazardous decomposition products:  None

- Hazardous polymerization:  Will not occur