MATERIAL SAFETY DATA SHEET

AND SAFE HANDLING AND DISPOSAL INFORMATION         pARE   1   Qf.  ...

zep manufacturing company                    DATE                 :   04/21/89      ZEP   TILE   AND   GROUT   CLEANER
first in maintenance products	SUPERSEDES-.    02/08/87     PRODUCT   NUMBERS    1046

SECTION   I   -   E   M   E   R   G   E   N   C   Y        C   0   N   T   A   C   T   S

ZEP MANUFACTURING COMPANY	NON-OFFICE HOURS, WEEKENDS, AND HOLIDAYS:AREA CODE 4O4

P.O. BOX 2015	435-2973, 996-0899, 252-1587, 351-2952, 445-9226

ATLANTA, GEORGIA 303O1	LOCAL POISON CONTROL CENTER

TELEPHONE (404)352-1680	TRANSPORTATION EMERGENCY

BETWEEN 8500A.M.-5500P.M.	CHEMTREC: TOLL FREE 1-800-424-9300 ALL CALLS RECORDED

(EASTERN TIME ZONE)	DISTRICT OF COLUMBIA <202)483-7616 ALL CALLS RECORDED

SECTION   II   -   H   A   2   A   R   D   0   U   S         INGREDIENT   S

TLV   EFFECTS       7. IN

DESIGNATIONS	(PPM) (SEE REVERSE) PROD.

@*# ETHYLENE GLYCOL MONOBUTYL ETHER #* 2-BUTOXYETHAN-   25    TOX IRR CBL   < 5 OL; BUTYL CELLOSOLVE; CAS# 111-76-2; RTECStt KJ8575000! OSHA PEL (SKIN)-50 PPM.

** TETRASODIUM ETHYLENEDIAMINE TETRAACETATE **         N/D   IRR	5-10

ETHYLENEDINITRILO TETRA-ACETIC ACID; EDTA; CAS# 64-02-8? RTECS# AH40250005 OSHA PEL-N/D.

** SODIUM OODECYLBENZENE SULFONATE ** LINEAR ALKYL     N/D   IRR	< 5

ARYL SODIUM SULFONATE; CAS# 25155-30-0; RTECS# DB6825000; OSHA PEL N/D

e IDENTIFIES CHEMICALS LISTED UNDER SARA-SECTION 313 FOR RELEASE REPORTING.

SPECIAL NOTE! ADVERSE HEALTH EFFECTS WOULD NOT BE EXPECTED UNDER RECOMMENDED CONDITIONS OF USE SO LONG AS PRESCRIBED SAFETY PRECAUTIONS ARE PRACTICED.

SECTION   III-HEALTH        HAZARD        DATA

ACUTE EFFECTS OF OVEREXPOSUREs

PRODUCT IN CONCENTRATED FORM IS A SEVERE EYE IRRITANT. OVER-EXPOSURE MAY LEAD TO EYE TISSUE DAMAGE WHICH CAN BE PERMANENT. SKIN CONTACT MAY PRODUCE IRRITATION. OVER-EXPOSURE BY INHALATION MAY CAUSE RESPIRATORY IRRITATION.

SECTION   III   -HEALTH        HAZARD        DATA   (CONTINUED)

CHRONIC EFFECTS OF OVEREXPOSUREs

REPEATED OR PROLONGED SKIN CONTACT MAY PRODUCE CHRONIC INFLAMMATION OR DERMATITIS, CHARACTERIZED BY REDNESS, SCALING, OR ITCHING. REPEATED EYE EXPOSURE MAY PRODUCE CHRONIC INFLAMMATION OF THE EYE OR CORNEAL DAMAGE.

ANIMAL STUDIES INDICATE A POTENTIAL FOR LIVER, KIDNEY, OR RED BLOOD CELL DAMAGE. RELEVANCE OF THESE STUDIES OR EXPOSURE LEVELS WHICH MIGHT PRODUCE THESE EFFECTS IN HUMANS HAS NOT BEEN ESTABLISHED. NONE OF THE HAZARDOUS INGREDIENTS ARE LISTED AS CARCINOGENS BY IARC, NTP, & OSHA

est-'d pel/tlv: not established   primary routes of entry: inh, skin, ing. hmis codes: health 2?flam. o5 react. 0?pers. protect. b $ chronic haz. yes

first aid procedures:

skin : immediately flush contaminated skin with plenty of water for at least 15 minutes. get medical attention if irritation develops.

eyes = immediately flush eyes with plenty of water for at least 1!::; minutes, occasionally lifting upper and lower lids. get medical attention at once.

inhale: move exposed person to fresh air. if irritation persists, get medical attention promptly.

ingest: if this product is swallowed, do not induce vomiting. if victim is

CONSCIOUS GIVE PLENTY OF WATER TO DRINK. GET MEDICAL ATTENTION AT ONCE. SECTION IV ~ S P E C I A L   PROTECTION   I N F 0 R M A T I 0 N

PROTECTIVE CLOTHING   : WEAR NITRILE GLOVES OR USE GLOVES WITH DEMONSTRATED

RESISTANCE TO THE INGREDIENTS IN THIS PRODUCT.

EYE PROTECTION         : WEAR TIGHT-FITTING SPLASH-PROOF SAFETY GLASSES

ESPECIALLY IF CONTACT LENSES ARE WORN.

RESPIRATORY PROTECTION: KEEP FACE AWAY FROM SPRAY MIST AND DO NOT BREATHE

VAPORS.

VENTILATION	s VENTILATION SHOULD BE EQUIVALENT TO OUTDOORS. USE EX-

HAUST FANS AND OPEN WINDOWS IN ENCLOSED SPACES.

SECTION V - P H Y S I C A L   DATA

BOILING POINT (F)   : APPROX. 220F   SPECIFIC GRAVITY	: 1.1

VAPOR PRESSURE(MMHG>: N/A	PERCENT VOLATILE BY VOLUME	<'/.)      : 83.6

VAPOR DENSITY(AIR=1): N/A	EVAPORATION RATE(WATER	=1): 1

SOLUBILITY IN WATER : COMPLETE       PH(CONCENTRATE)	: 12.8

PH(USE DILUTION OF	): N/A

APPEARANCE AND ODOR :A CLEAR, RED-VIOLET LIQUID WITH A PLEASANT	FRAGRANCE

SECTION   VI   -   F   I   R  E        AND        EXPLOSION        DATA

FLASH POINT(F) (METHOD USED): NONE	(TCC )

FLAMMABLE LIMITS     LEL N/A    UEL N/A EXTINGUISHING MEDIA  : N/A SPECIAL FIRE FIGHTING: NONE UNUSUAL FIRE HAZARDS : NONE

SECTION VII - R E A C T I V I T Y   DATA

STABILITY	:	STABLE

INCOMPATIBILITY(AVOID) :	ACIDS

POLYMERIZATION         :	WILL NOT OCCUR.

HAZARDOUS DECOMPOSITION:	C02, HEAT

SECTION VIII - S P I L L   AND   D I S P 0 SAL   P R 0 C E D U R E S

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED!

ABSORB SPILL WITH AN INERT ABSORBENT MATERIAL. (E.G. ZEP-0-ZORB)! PICK UP AND DEPOSIT IN A SEALABLE, D.O.T. SPECIFICATION CONTAINER FOR DISPOSAL AS A HAZARDOUS WASTE.  THOROUGHLY CLEAN AREA WITH A DETERGENT SOLUTION.  RINSE AREA THOROUGHLY WITH CLEAN WATER.

WASTE DISPOSAL METHOD:

UNUSED PRODUCT MAY HAVE TO BE ABSORBED ON AN INERT MATERIAL <ZEP-0-ZORB) AND DISPOSED OF AS A HAZARDOUS WASTE.  IN MOST STATES, QUANTITIES TOTALING LESS THAN 220 LBS. OF HAZARDOUS MATERIALS PER MONTH CAN BE DISPOSED OF IN A CHEMICAL LANDFILL WITHOUT BEING DECLARED AS A HAZARDOUS WASTE.  SINCE REGULATIONS VARY CONCERNING THE DISPOSAL OF THESE CHEMICALS, CONSULT LOCAL, STATE AND FEDERAL AGENCIES FOR PROPER DISPOSAL PROCEDURES IN YOUR AREA.

RCRA HAZ. WASTE NOS.s D002

SECTION IX - S P E C I A L   P R E C A U T I 0 N S

PRECAUTIONS TO BE TAKEN WHEN HANDLING AND STORING:

STORE TIGHTLY CLOSED CONTAINER IN A DRY AREA AT TEMPS. BETWEEN 40-120 DEGREES F.

STORE AWAY FROM STRONG ACIDS AND OXIDIZING COMPOUNDS.

KEEP PRODUCT AWAY FROM SKIN AND EYES.

DO NOT BREATHE SPRAY MISTS OR VAPORS.

KEEP AWAY FROM FOOD AND FOOD PRODUCTS.

KEEP OUT OF THE REACH OF CHILDREN.

SECTION X - T R A N S P 0 R T A T I 0 N   DATA

DOT PROPER SHIPPING NAME

NONE

DOT HAZARD CLASS: N/A

DOT I.D. NUMBER : N/A      DOT LABEL/PLACARD: NONE EPA TSCA CHEMICAL INVENTORY - ALL INGREDIENTS ARE LISTED

EPA CWA 40CFR PART 117 SUBSTANCE<RQ IN A SINGLE CONTAINER): SODIUM DODECYL-BENZENE SULFONATE, 1000#