MATERIAL SAFETY DATA SHEET: M/M PENETRATING THREADLOCKER	Page: l

(OOOOOO'-OOOOOO-    -4964    )	DATE OF ISSUE        SUPERSEDES

2/01/2000	N/A

SECTION I - GENERAL INFORMATION

Chemical Name   &  Synonyms

Trade Name   &   Synonyms

M/M  PENETRATING  THREADLOCKER

lemical  Family: METHACRYLATE

Formula Mixture   —> X

Manufacturer' s Name:

MANTEK,   DIVISION  OF  NCH  CORP.

Address:

BOX 152170

IRVING,	TEXAS   75015

Prepared By:

G  ZIMMERMAN/CHEMIST

Product  Code Number 4964

Emergency  Phone Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

" cal Name   (Ingredients) POLYGLYCOL  METHACRYLATE CUMENE HYDROPEROXIDE SACCHARIN N, N-DIAKYLTOLUIDINE

Haza

IRRITANT IRRITANT CARC. 5. IRRITANT

TLV

NOT EST	1

NOT EST.	1

NOT EST.	1

NOT EST.	1

PEL

NOT	EST.	2

NOT	EST.	2

NOT	EST-	2

NOT	EST.	2

STEL NOT EST. NOT EST. NOT EST. NOT EST.

CAS It

25852-47-5

30-15-9

31-07-2

613-18-9

SECTION III - PHYSICAL DATA

Boiling Point   (F) :	>300	Specific   Gravity   (H20=l) :	1.07

Vapor  Pressure   (MM  HG) :	<5	Color:	GREEN

Vapor Density   (Air=l) :	N/E	Odor:	SWEET

PH  8   100%    :	N/A	Clarity:	OPAQUE

Volatile   by  Volume:	12.6	Evaporation  Rate    (BU A/C=1) :	<1

.^0   Solubility:	SLIGHT	Viscosity:	NON-VISCOUS

SECTION IV - FIRE AND EXPLOSION HAZARD

Flash Point	Flammable Limits	LEL	UEL

>200 F.  / T.C.C.	N/E	N/E	N/E

Extinguishing Media

X <—Foam	<—Alcohol Foam        X <--CO2       X <—Dry Chemical	<—Water Spray      <—Other

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR.

Unusual Fire and Explosion Hazards:

FUMES FORMED WHEN PRODUCT IS HEATED ARE VERY IRRITATING. IF POSSIBLE USE A WATER SPRAY TO COOL FIRE EXPOSED CONTAINERS TO PREVENT BURSTING OR

MELTING.

Aerosol Level (NFPA 30B) :    N/A

NFPA 704 Hazard Rating       (0=Insignificant l=Slight  2=Moderate  3=High  4=Extreme) 1  <—Health      1  <—Flammability  1  <--Instability      <---Special

SECTION V - HEALTH HAZARD DATA

Threshold Lim.it Value:

NOT ESTABLISHED FOR MIXTURE.

Effects of Overexposure:

-Acute (Short Term Exposure)

SKIN CONTACT: MAY CAUSE IRRITATION SEEN AS ITCHING AND REDNESS. MAY CAUSE AN ALLERGIC REACTION IN SOME INDIVIDUALS. EYE CONTACT: MAY CAUSE IRRITATION SEEN AS TEARING AND REDNESS. INGESTION: MAY CAUSE IRRITATION WITH POSSIBLE NAUSEA, VOMITING AND DIARR- HEA. INHALATION: MAY CAUSE RESPIRATORY IRRITATION SEEN AS COUGHING AND SNEEZING.

-Chronic (Long Term Exposure)

TARGET ORGANS: BLOOD-FORMING ORGANS MAY CAUSE SKIN SENSITIZATION IN SOME INDIVIDUALS. SENSITIZED PERSONS MAY HAVE ONLY A MILD REACTION AFTER THE INITIAL EXPOSURE, BUT MAY EXIBIT A MUCH MORE PRONOUNCED REACTION UPON SUBSEQUENT EXPOSURE. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING —*=IN CONDITIONS SUCH AS DERMATITIS.

i.mary Routes of Entry:       <--Inhalation       <--Ingestion      X <--Absorption

Emergency and First Aid Procedures:

-Inhalation: REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact: RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WI^-H PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF

MATERIAL SAFETY DATA SHEET: M/M PENETRATING THREAD LOCKER   Page: 2 _ SECTION V - HEALTH HAZARD DATA (Continued) _ '

PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

".""" ...............       .....	..... ""     """ .......	^

PER, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS.

Lan:

DOTE. TREAT THE PATIENT SYMPTOMAT ICALLY.

_ SECTION VI - TOXICITY INFORMATION _

3 Listed as Carcinogen or Potential Carcinogen By:
Yes	OSHA — > No	ACGIH — > No	OTHER — > No

3LE

LE

3  3.

»ffiS HAVE CAUSED CNS EFFECTS (INTOXICATION) IN HEMICAL REACTION, THIS MATERIAL IS CONSUMED SS AND RELEASES NO FUMES-  4.

/KG 4. •AG/KG 4 .

tfN THAT SACCHARIN CAN PRODUCE CANCER IN RATS FOR EXTENDED PERIODS. 3.   INGESTION IS NOT IS USED AS DIRECTED.

_ SECTION VII - REACTIVITY DATA

table      <- -Unstable RAGE.

s to Avoid) :

WHILE IN THE UNCURED STATE.

roduc t s : MOKE.

<--May Occmr      X < — Will Not Occur

SECTION VIII - SPILL OR LEAK PROCEDURES

rial is Released or Spilled:

PACKAGING, A LARGE SPILL IS UNLIKELY, PICK UP SPILLED MATERIAL WITH A CLOTH OR SUITABLE ABSORBENT AND DISPOSE OF PROPERLY.

WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

SECTION IX - SPECIAL PROTECTION INFORMATION

RMALLY ADEQUATE.

L CONDITIONS OF USE.

R GLOVES SHOULD BE WORN.

HOD OF USE PRESENTS THE LIKELIHOOD OF EYE CONTACT.

ONS OF USE.

SECTION X - STORAGE AND HANDLING INFORMATION

ndoors--> X	Outdoors-->	Heated-->	Refrigerated-->

Maximum Temperature: 100 F

n Handling and Storing:

REA. KEEP CONTAINER CLOSED WHEN NOT IN USE.

DREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS. DO NOT CONTAMINATE PRODUCT BY RETURNING IGIONAL CONTAINER.

MATERIAL SAFETY DATA SHEET: M/M PENETRATING THREAD LOCKER SECTION X - STORAGE AND HANDLING INFORMATION (Continued)

SECTION XI - REGULATORY INFORMATION

CAS Number	Upper % Limit

ENE  HYDROPEROXIDE	80-15-9	5

Those ingredients  listed above  are  subject   to  the  reporting requirements   of   313   of  Title   III   of   the   Superfund Amendments   and Reauthorization Act   o 1986  and  40   CFR part   372.

Please call   1-800-527-9919   for  additional   information if  you  are  a  California  customer. This MSDS   is  not intended for users   in   the   state  of California.

SECTION  XII   -   REFERENCES

1.   THRESHOLD   LQilT  VALUES   FOR  CHEMICAL   SUBSTANCES  AND   PHYSICAL  AGENTS   AND BIOLOGICAL  EXPOSURE   INDICES,   ACGIH,    1999.       2.      OSHA  PEL.

- REGISTRY   OF  TOXIC  EFFECTS   OF   CHEMICAL   SUBSTANCES,    CCINFODisc,    1999-

- VENDOR'S  MSDS.	5.    IARC.

ALL COMPONENTS IN THIS PRODUCT CAN BE FOUND IN THE CURRENT TSCA INVENTORY.

IRR: IRRITANT, FLAM/FLAMM: FLAMMABLE, COMB: COMBUST IBLE , CORK: CORROSIVE CARC:CARCINOGENIC, TOX:TOXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC:CLEVELAND OPEN CUP, PMCC: PENSKY-MARTIN CLOSED CUP, TCC: TAGLIABUE CLOSEDCUP, LELiLOWER EXPLOSION LIMIT, UELzUPPER EXPLOSION LIMIT, NFPA:NATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTPiNATIONAL TOXICOLOGY PROGRAM, OSHA: OCCUPATIONAL SAFETY £ HEALTH ADMINISTRATION, ACHGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV: THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LEVEL, STEL: SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD: MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX:ASPHYXIANT, PNOC: PARTICULATES NOT OTHERWISE CLASSIFIED

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR LMPLI REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

MANTEK, DIVISION OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the prod in a manner not recommended on the product label.  Users assume all risks associated with such unrecommended use, storage, or disposal of the product