ComStar International, Inc.

MATERIAL SAFETY DATA SHEET

Supplies:  I.P.C. Div., ComStar Interanational, Inc.	Phone:  718 445 7900

Addressl  20-45 123th Street, College Point, N.Y.  11356

Trade Name:	Mule Kick Drain Cleaner-Caustic Soda

Chemical Name and Synonyms:	Sodium Hydroxide

Chemical Family:	Inorganic Alkali		Formula:NaOH

SECTION I	PHYSICAL DATA:

Boiling Range:  390 C	API Gravity:  na

Specific Gravity:  2.13	Pounds/Gal:  na

Vapor Pressure:  L.T. 0.01	Vapor Density:

Solubility:  42g/100cc 0 C	Solubility in Acid:  Reacts violently with heat forming Na2SO4 Sodium Sulfate

Evaporation Rate:  0	Per cent Volatile by Volume:  na

Appearance:  White, solid, flake or granular	Odor:  Practically odorless

SECTION II	HAZARDOUS INGREDIENTS:

| HAZARDOUS INGREDIENTS | VOLUME % | OSHAPEL | TLV UNITS |  |
| --- | --- | --- | --- | --- |
| Sodium Hydroxide Beads | 100 |  | 2 MG/M3 |  |

SECTION III	FIRE AND EXPLOSION HAZARD DATA:

Lower Flammable Limit: na				DOT Classification:  Corrosive

Flash Point:  None					Flammable Classification:  na

Extinguishing Media:  Does not burn or support combustion

Special Fire Fighting Procedures:  None

Unusual Fire and Explosion Hazards:

SECTION IV	HEALTH HAZARD DATA:

Threshold Limit Value:  2ug/m3

Effects of Overexposure:  Immediate destruction of tissue

Emergency and First Aid Procedures:  Skin or Eye Contact – irrigate affected area with copious amounts of water for at least 15 minutes.  Get medical attention.

SECTION V   REACTIVITY DATA:

Stability:  Stable

Incompatibility:  Aluminum, tin, lead, zinc and their alloys.  All acids.

Hazardous Polymerization:  Will not occur

SECTION VI     SPILL OR LEAK PROCEDURES:

Steps to be taken if material is released or spilled:  Shovel up all possible material and recontainer.  Flush with large quantities of water.  Neutralize with dilute acid.  Dispose of in accordance  with federal, state, and local regulations.

Waste disposal method:  Follow state and local regulations.

SECTION VII	SPECIAL PROTECTION INFORMATION:

Respiratory Protection:  US Bureau of Mines-approved for dust

Protective Gloves:  Rubber or Neoprene, Chemical safety goggles

Other Protective Equipment:  Rubber boots and aprons

Precautionary Labeling:  D.O.T. “Corrosive” label in addition to commodity label.

Other handling and storage precautions and conditions:  Considerable heat is generated when water and caustic soda (NaOH) is mixed; boiling and spattering of hot caustic solution may result.