Chem-Tex Laboratories, Inc

P.O. Box 5228

Concord, NC 28027

(704) 795-9322

MATERIAL SAFETY DATA SHEET

I.	CHEMICAL PRODUCT/COMPANY IDENTIFICATION

PRODUCT NAME:	Bac-Shield

EPA Reg #:	81446-1

IL	COMPOSITION: INFORMATION ON INGREDIENTS

CHEMICAL FAMILY:	Natural Polysaccharide

CAS REGISTRY NO:

CHEM. TREC:	Not Registered

CHEMICAL NAME:	Copolymer of poly n-acetylglucosamine and poly D-glucosamine

III.	PHYSICAL DATA

APPEARANCE:	Transparent Liquid

SOLUBILITY:	Soluble in water

SPECIFIC GRAVITY:	0.97 g/cm3

FLAMMABILITY:	Non-Flammable

FLASH POINT:	No flash point

BOILING POINT:	> 100 C (1% Cone.)

ODOR:	Faint "sea-side" odor

FORM:	Liquid

COLOR:	White to Yellow

PH:	4.5 + /- .5

SOLIDS: (ASTM)	8.25 + / - .25

IV.	HAZARDS IDENTIFICATION

ROUTE(S) OF ENTRY:	Eyes, skin

EFFECTS OF OVEREXPOSURE:    Material is mildly irritating to eyes and skin

V.	FIRST AID MEASURES

EYES:	Immediately flush eyes with plenty of water for at least

15 minutes. Call a physician.
SKIN:	In case of skin contact, the compound is not hazardous

but cleansing the skin after use is advisable.

INGESTION:	If swallowed, no specific intervention is indicated as the

compound is not likely to be hazardous by ingestion. However, consult a physician if necessary.

INHALATION:	If large amounts are inhaled, remove to fresh air. Call a

physician.

VI.

FIRE AND EXPLOSION DATA

FIRE AND EXPLOSION HAZARDS:

EXTINGUISHING MEDIA:

SPECIAL FIRE FIGHTING INSTRUCTIONS:

No fire or explosive hazards. Spray water or chemical foam None

VII.	ACCIDENTAL RELEASE MEASURES

SPILL OR LEAK PROCEDURES:

Utilize protective clothing. Follow normal disposal procedure for non-hazardous liquids.

VIII.

HANDLING AND STORAGE

STORAGE TEMPERATURE:

SHELF LIFE:

SPECIAL SENSITIVITY:

HANDLING/STORAGE PRECAUTIONS:

0-50°C recommended (avoid over 280°C)

2 Years

None known

Store in dry place. Keep containers covered

to prevent contamination.

IX.

EXPOSURE CONTROL/PERSONAL PROTECTION

EYE PROTECTION REQUIREMENTS:

SKIN PROTECTION REQUIREMENTS:

RESPIRATOR REQUIREMENTS:

LOCAL EXHAUST:

MECHANICAL:

ADDITIONAL PROTECTIVE MEASURES:

Safety glasses are recommended

Rubber gloves are recommended

N/A

N/A

Mechanical exhaust not required

Emergency showers and eye wash stations

are suggested. Employees should wash their

hands before eating or drinking.

X.

STABILITY AND REACTIVITY

STABILITY:

HAZARDOUS POLYMERIZATION:

INCOMPATIBILITIES:

DECOMPOSITION: INSTABILITY CONDITIONS:

DECOMPOSITION TEMPERATURE: DECOMPOSITION PRODUCTS:

Stable

Polymerization will not occur.

Incompatible with strong oxidants and

strong bases.

Decomposition will not occur

Stable at normal temperatures and storage

conditions.

None

N/A

^

XI.	TOXICOLOGICAL INFORMATION

r>

ANIMAL DATA

ACUTE TOXICITY:

Acute Oral LD 50 (Rat):	> 5000 mg/kg (No Mortality)

Acute Dermal LD50 (Rat):	>5000 mg/kg (No Mortality)

Acute Inhalation LC50 (Rat):	2.07 mg/1 (No Mortality)

Skin Irritation Study (Rabbit):	PDII = 0.7 (Slightly Turning)

Primary Eye Irritation Study (Rabbit):    PII = 9.3 (Unrinsed), 7.3 (Rinsed) (Mildly Irritating)

Dermal Sensitization (G. Pig):	Not A sensitizer

XII.	ECOLOGICAL INFORMATION

Material is biodegradable

XIII.	DISPOSAL CONSIDERATIONS

Treatment, Storage, Transportation and Disposal must be in accordance with applicable Federal, State/Provincial, and Local regulations.

CONTAINER DISPOSAL:    Wash container with water

XIV.	TRANSPORTATION INFORMATION

D.O.T. SHIPPING NAME:	Aqueous Bac-Shield

TECHNICAL SHIPPING NAME:	Bac-Shield

D.O.T. HAZARD CLASS:	None

PRODUCT RQ. (LBS):	None

D.O.T. LABEL:	None

D.O.T. PLACARD:	None

XV.	REGULATORY INFORMATION

OSHA STATUS:	Bac-Shield is not hazardous under the criteria of the Federal

OSHA Hazard.

TSCA STATUS:	Bac-Shield is on TSCA inventory.

RCRA:	If discarded in its purchased form, this product would not be a

hazardous waste. Under RCRA, it is the responsibility of the product user to determine if the product or a derived form of the product should be classified as a hazardous waste.

XVI.	OTHER INFORMATION

N/A

XVII.	APPROVAL

PREPARED BY:	David M. Bilbro

APPROVAL DATE:	November 28, 2006

Disclaimer

This material safety data sheet was prepared solely for the purpose of providing employees with information regarding potential hazards presented by listed substance(s). It was prepared in the context of reasonable foreseeable exposure situations, which may arise give the nature and the expected use of the product. The substance(s) may present greater or lesser hazard under other circumstances. The data and recommendations presented herein are based upon our research and the research of others but Chem-Tex Laboratories, Inc. makes no warranty of any kind expressed or implied, except that the product supplied will be of standard quality of Chem-Tex Laboratories, Inc and the user assumes all risk and liability resulting from reliance of the information contained on this Material Safety Data Sheet and the use of the product, whether used singularly or in combination with other substances. This disclaimer does not, however apply to an independent statutory or contractual obligations, which may otherwise exist between Chem-Tex Laboratories, Inc and any other person.