FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"	msds #    29Q QQ

Material Safety Data Sheet (MSDS)	Revision Date: March 14,2001

Section 1 — Chemical Product and Company Identification

Cyclohexane

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Cyclohexane

CAS#: 110-82-7

Section 3 — Hazards Identification

Clear colorless liquid. Pungent odor; rubber cement-like.	flinn Ai-A-GuANCE

Irritant to body tissues. Mildly toxic by ingestion, inhalation, and skin absorption. Avoid contact with	Health-1

body tissues.	Flammability-3

Dangerous fire risk; extremely flammable.	Reactivity-0

Exposure-1 Storage-3

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Rinse out mouth with water. Give a mixture of 2 Tbs. of carbon in 8 oz. of water to drink. Call a physician or poison

control at once.

Section 5 — Fire Fighting Measures

Class IB flammable liquid.	NFPA code

Flash Point: 0 F   Upper: 8%   Lower: 1.3%   Autoignition Temperature: 500 F	H-l

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE         F-3
and SCBA with full facepiece operated in positive pressure mode.	R-0

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill site after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic #3. Store with hydrocarbons, esters and aldehydes. Store in a dedicated flammables cabinet. If a flammables cabinet is not available, store in Flinn Saf-Stor can. Use and dispense in a hood.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 300 ppm (OSHA)

© 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Cyclohexane

MSDS #:    290.00

Revision Date:   March 14, 2001

Section 9 — Physical and Chemical Properties

Clear colorless liquid. Pungent odor; rubber cement-like. Solubility: Insoluble in water. Soluble in alcohol. Formula: C6H12 Formula Weight: 84.18

Specific Gravity: 0.78 Boiling Point: 81 C

Section 10 — Stability and Reactivity

Avoid contact with heat, open flame, any source of ignition and strong oxidizers. Shelf life: Indefinite

Section 11 —lexicological Information

Acute effects: Irritant, lung irritation, chest pain and edema which

ORL-RAT LD50: 12705 mg/kg IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

may be fatal, gastrointestinal disturbances

Chronic effects: N.A.

Target organs: Lungs and nerves

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #18a is one option.

Section 14 — Transport Information

Shipping Name: Cyclohexane Hazard Class: 3, Flammable liquid UN Number: UN 1145

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (203-806-2), RCRA codeU056.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Flinn Chemical Packaging Prevents Accidents

flinn@flinnsci.coin   www.flinnsci.com

P.O. Box 219   BatavialL 60510

(800) 452-1261 Fax (866) 452-1436

) 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2