| Chemsico Division of United Industries P.O. Box 142642 St. Louis, MO 63114-0642 Material Safety Data Sh Complies with OSHA's Hazard Communication Standard, 29 C | Hazardous Material Identification System - (HMIS) |
| --- | --- |
|  | HEALTH -1 REACTIVITY -0 |
|  | lt?t?l FLAMMABILITY-0 PERSONAL - :fr 1910.1200 |
| I Trade Name: Green Thumb Wasp & Hornet Killer |  |
| Product Type: Aerosol insecticide |  |
| Product Item Number: 596692 | Formula Code Number: 21-0688/21-0689 |
| EPA Registration Number Manufacturer Emergency Telephone Numbers |  |
| Chemsico For Chemical 1-800-633-2873 0/cio . , „ Division Of United Industries Corp. Emergency For 1-800-332-5553 9688-1 17 8494 Chapin Industrial Drive "^motion: c. a. Duckworth St. Louis, MO 631 4 ^Cred: May ,2002 |  |
| II Hazards Ingredient/Identity Information | III Physical and Chemical Characteristics |
| Chemical % OSHA PEL ACGIH TLV Mineral Spirits 4.00 100 ppm 100 ppm CAS #8052-4 1-3 Butyl Proposal 6.00 None None CAS#5131-66-8 Tralomethrin 0.01 NA NE CAS# 66841-25-6 d-trans Allethrin 0.05 NE NE CAS# 6 1009-26-5 Propane 4.50 1000 ppm NE CAS# 74-98-6 | Appearance i Odor: Jet spray and a glycol ether odor. Boiling Point: NA Melting Point: NA Vapor Pressure: 1 10 psig at 54° C/1300 F Specific Gravity: 0.97 (H2O = 1 ) Vapor Density: Greater than 1 (Air= 1) % Volatile (by vol.): 95% Solubility in Water: Greater than 87% Evaporation Rate: Less than 1 (Butyl Acetate = 1) |
| IV Fire and Explosive Hazards Data | V Reactivity Data |
| Flash Point: 75° F (TCC) (liquid phase) Flame Extension: 0-inches (Level 1 Aerosol) Flammable Limits: NA Autoignition Temperature: NA Fire Extinguishing Media: Water fog, Carbon dioxide, Dry chemical Decomposition Temperature: NA Special Fire-Fighting Keep cans cool. Use equipment or shielding to Procedures: protect personnel against bursting, rupturing or venting cans. Unusual Fire & At elevated temperatures (over 54° C/130° F), Explosion Hazards: cans may vent, rupture or burst. See Section V. | Stability: Stable Polymerization: Will not occur Conditions to Avoid: Temperatures over 1 30° F Incompatible Materials: NA Hazardous Decomposition or Byproducts: Carbon dioxide, carbon monoxide |
| VI Health Hazard Data | VII Precautions for Safe Handling and Use |
| Eye Contact: Causes eye irritation. Avoid contract with eyes. First Aid: Flush eyes with plenty of water. Call a physician if irritation persists. Special Notes: Avoid contamination of food and drinking water. Health conditions Aggravated by None known Exposure: Ingredients listed by NTP, OSHA, or IARC as Carcinogens or Potential Carcinogens: None | Steps to be Taken in Case Material is Released or Spilled: Avoid breathing vapors. Avoid contact with liquid. Remove ignition sources. Soak up spills with absorbent material. Waste Disposal: Do not puncture or incinerate containers. Give empty, leaking or full containers to a facility qualified to dispose of pressurized containers. Handling & Storage Precautions: Do not store where temperatures can exceed 54° C/1300 F. |
| VIII Control Measures | IX Transportation Data |
| Read and follow label directions. They are your best guide to using this product effectively, and give necessary safety precautions to protect your health. | DOT Shipping Name: Consumer Commodity DOT Hazard Class: ORM-D |

The information and statements herein are believed to be reliable but are not to be construed as warranty or representation for which we assume legal responsibility. Users should undertake sufficient verification and testing to determine the suitability for their own particular purpose of any information or products referred to herein. NO WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE IS MADE.