PETROLANE

MATERIAL SAFETY DATA SHEET

P.O.  Drawer   1410     -     1600  E. Contact:  Safety Department

WARNING STATEMENT

Hill   St.,  Long  Beach,   CA  90801 Telephone:  (213)  427-5471

February  1989

DANGER! Extremely Flammable.    Keep away from  heat, sparks and open flame.

Vapor reduces oxygen available for breathing and may cause suffocation in confined spaces.    Use only with adequate ventilation.    Odor may  not  provide  adequate warning of  potentially hazardous concentrations.    Vapor is heavier than air and collects  in  low  levels. Liquid may cause freeze  burn  similar to frostbite.

vPfrpduct:.''	'•'•?• icat.ioitsijj

Product Name:    Petrolane Propane	'«'['.

Chemical Name:    Propane

Synonyms: LP-Gas, Bottled Gas

Chemical Family:  Paraffinic Hydrocarbon

Chemical Formula: C3H8

DOT Proper Shipping Name: Liquefied Petroleum Gas

DOT Hazard Class: Flammable Gas

DOT I.D. Number: UN 1075

Transportation Emergency Telephone: 800-424-9300     (CHEMTREC)

NFPA Classification:

Health	1    Slightly Toxic

Fire	4    Extremely Flammable

Reactivity     0    Stable

|  | ; HaZarclaus I ngr-ed i erits :;sj |
| --- | --- |
|  | Component CAS Number |
|  | Ethane 74-84-0 Propane 74-98-6 Propylene 115-07-1 Butane 106-97-8 |
| III ; Physical Data i ¦ |  |

0-6 87-97 0-5 0-2.5

OSHA PEL

None  established 1000 ppm  <8hr> None established None established

ACGIH TLV

Simple asphyxiant Simple asphyxiant Simple asphyxiant 800 ppm (8 hr)

Boiling Point:     -44°F

% Volatile by Volume:     100% Solubility  in Water:    Insoluble Evaporation Rate(Bu Ac=1):    N/A Gas Volume @ Atm. Pressure & 60 °F (Cu. ft. gas/gal,  liquid): 36.4

Melting Point:    -309°F

Vapor Pressure:    208 psig  (max.)  @   100°F

Vapor Density (Air=1):     1.5

Specific Gravity (H20=1):  0.504

Appearance and Odor:    Colorless, odorless  in pure form.

Propane contains a foul  smelling,  skunk-like warning agent(odorant).    The odorant is effective, in most instances,  but not everyone can smell  the odor.    The ability of people to detect odors varies widely.    Also, certain chemical  reactions  with  material   in the propane  system can reduce the propane odor level.    No  odorant w,ill  be  100% effective  in all  circumstances.    If odor level  appears to be weak,  notify propane  supplier immediately.

F ire • arid Exp 1os i on  Data

Flash Point (Method Used):    -156"F (estimated) Flammable Limits (% Volume in Air):    Lower 2.1%

Upper 9.5%

Extinguishing Media:    Dry chemical, foam or C02  for small fires.    Stop flow of gas first.

Special Fire Fighting Procedures and Precautions:    Elimimate sources of ignition.    Evacuate area.    Notify fire department.    Allow only trained, properly protected personnel in area.    Shut off source of gas, if possible.    Allow fire to burn itself out after gas flow is shut off.    High volume water supply can be used to cool heat-exposed pressure containers and nearby equipment.    Approach a flame enveloped container from the side, never the head ends.    Use extreme caution when applying water to a container which has been exposed to heat or flame for more than a short time.    Shock of cool water on hot metal could cause container rupture.      For uncontrollable fires and when flame  is  impinging on container, withdraw all personnel and evacuate surrounding vicinity immpr^aipiv

•nioiistion  may yield carbon monoxide, a toxic gas. a\_i  and are  extremely flammable.

Unusual Fire and Explosion. Hazards.    Products  -Uncontrolled vapors  spread rapidly,  are _heavi_Rj.;hsj

Stability:	Stable	Conditions to  Avrii

Materials	to Avoid:     Strong oxidizing  agent?.

Hazardous	Decomposition Products:

Hazardous	Pnlvmnnzation:    Wi!!  no'

heat,  sparks,  open flame

stion  nan cause carbon monoxide,  a toxic gas. nrfilions   'c   Avoid:     None

listed as carcinogenic by NTP, IARC or OSHA. Product may contain a trace, but ount of benzene, a chemical listed by the State of California and known to cause oductive toxicity.

•y  /  Acute  Effects  of Overcxposure:

;_   Exposure to high  concentrations  of  (he vapor causes  dizziness,  drowsiness,  nausea or usness  due to  anesthetic  properties.

:act:    Liquid can cause freeze burns  similar  to  frostbite .if contact with skin occurs, absorption  is expected.

3£li.   Liquid can cause freeze burns  if contact with eyes occurs.   ' Ingestion  is not expected to  occur in normal  use.

Effects of -QverexpQsure:    No abnormal  reactions reported following exposure to  1000 ppm irs per day,  5 days per week,  for  ?  weeks.

tions Generally Afjn,rav3tnd  hv E:xpdstife:     Persons wi'h  chronic respiratory diseases jxposure. andrpTfst "Aid "PFocitilures~i

:lush with water.    Obtain  medical  assistance  if contact with  liquid has occurred.

If freeze  burn  occurs,  remove  contaminated clothes,  shoes  and jewelry.     Immerse i warm  (not hot)  water.    Keep  immersed.    Call  for medical  assistance.

3move victim from further exposure and  into fresh air.    Provide oxygen  if breathing : victim  is unconscious,  seek  immediate  medical  attention.    If breathing has stopped, respiration.

4 expected to occur in normal  use.

aak  Procedures I

remely flammable.    Vapor  is heavier than air and may collect at  lower levels.

ncentrations  may he present  below nose  level.     If  there  is  a  leak but no fire,  do not

ped gas.    Eliminate all  iqnition  sources.    Do not smoke, use  a nearby phone or

cal  switches.    Evacuate tne area.    If possible,  remove  leaking container to safe

ow of gas or allow vapor to disperse  in  3 safe area.    Water spray can be used to help

oncentration  in air.

s only by controlled burning  in compliance'with  local  laws and regulations. "storageT'reca'ut i ons    i

uthorized  location  (outside,  detached  storage  is preferred)  with adequate ventilation. Dm heat and  ignition sources.    Inspect cylinders frequently for leaks,  dents, gouges and i emphasis on bottom of cylinder.    Store  cylinders  in upright position or with pressure in vapor space.    Do not drop or abuse cylinders.    Keep container valve closed and not  in use.     Install  protective  caps  when  cylinders  are  not connected for use.    Empty 3in some residue, so they should be  treated  as  if they are full.

tection Information    |	i'.

Jse adequate ventilation to  maintain  exposures  below recommended  limits.

otection:    Use a NIOSH-approved respirator  if  area  is thought to contain unknown of gas.

i:    Use safety goggles or safety glasses with  side  shields.

thing:    No special  garments are necessary, but avoid skin contact with  liquid because of freeze burn.    Propane resistant gloves  are recommended.

^Wft'h Employees raV^yurchasers "71

Safety Data Sheet  (MSDS)  alerts  the reader to the  potential  safety and health hazards t also contains valuable reference' material; relating  10 the safe use and handling of Make sure  thai   this  infnrma'ion   is1 shar'H  >.-,,;th  afl .employees  and purchasers who use product:    '-   is  ar>  imputtan!   ria'l        5h*   OSHA  hazaK  communication program.

1 is believed to be accurate as of the date of issue, but is offered without ditions of use and suitability for use are beyond Company control, therefore, 3 of the product are assumed by the user.  COMPANY EXPRESSLY DISCLAIMS ALL /ERY KIND INCLUDING WARRANTIES OF MERCHANTIBILITY AND FITNESS FOR ANY 3SE.  Company assumes no responsibility for any injuries or damages ~oduct even if safety procedures are r oil-owed as outlined herein.  Nothing Jed to be construed as permission or recommendation for use of the product lich mitjht infringe e* i s t" i nc; patent^.