MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

LUGOL SOLUTION	Replaces: 07/10/07

CAROLINA BIOLOGICAL	Printed: 05/20/08

1.	PRODUCT DESCRIPTION

Product Name:      Lugol Solution

Product Code(s):   87-2793, 87-2795

Size:	100 ML,  500 ML

Chemical Name:     Product is mixture

CAS Number:        See section 2

Formula:	See section 2

Synonyms:	Donaldson's Amoeba Stain 2

Distributor:       Carolina Biological Supply Company

2700 York Road

Burlington, NC 27215

Chemical Information:  800-227-1150 (8am--5pm (ET) M-F) Chemtrec (Transportation Spill Response 24 hours): 800-424-9300

2.	COMPOSITION/INFORMATION ON INGREDIENTS

Principal Hazardous Components:

Potassium Iodide(CAS# 7681-11-0)10%

Iodine(CAS# 7553-56-2)5% TLV and PEL units:

Potassium Iodide:  No information found
Iodine:	ACGIH-TLV Ceiling 0.1 ppm

OSHA-PEL  Ceiling 0.1 ppm, Ceiling 1 mg/m3

3.	HAZARD IDENTIFICATION

Emergency Overview:  Harmful if swallowed, inhaled or absorbed through

the skin.  May cause irritation to eyes, skin and mucous membranes.

Symptoms of exposure may include burning sensation, coughing, wheezing,

laryngitis,shortness of breath, headache, nausea and vomiting.

Potential Health Effects:

Eyes:  May cause irritation.

Skin:  May cause irritation.

Ingestion:  May cause gastrointestinal discomfort.

Inhalation:  May cause irritation to respiratory tract.

4.	FIRST AID MEASURES

Emergency and First Aid Procedures:

Eyes - Flush with water for at least 15 minutes, raising and lowering

eyelids occasionally.  Get medical attention if irritation persists.

Skin - Thoroughly wash exposed area for at least 15 minutes.  Remove

contaminated clothing.  Launder contaminated clothing before reuse.  Get

medical attention if irritation persists.

Ingestion - If swallowed, if conscious, give plenty of water to dilute,

and get medical attention immediately.  Never give anything by mouth to

an unconscious person.

Inhalation - Remove to fresh air.  Give oxygen if breathing is

difficult; give artificial respiration if breathing has stopped.  Keep

person warm and quiet, and get medical attention.

5.	FIREFIGHTING PROCEDURES

Flash Point(Method Used):  No information available. NFPA Rating:  None established Extinguisher Media:

Use dry chemical, CO2 or appropriate foam.

Flammable Limits in Air % by Volume:  No information available Autoignition Temperature:  No information available.

Page 1 of 4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

LUGOL SOLUTION	Replaces: 07/10/07

CAROLINA BIOLOGICAL	Printed: 05/20/08

Special Firefighting Procedures:

Firefighters should wear full protective equipment and NIOSH approved

self-contained breathing apparatus.

Unusual Fire and Explosion Hazards:  When heated to excessive

temperatures may emit toxic and corrosive fumes of iodine.

6.	SPILL OR LEAK PROCEDURES

Steps to be Taken in Case Material is Released or Spilled:

Ventilate area of spill.  Eliminate all sources of ignition. Remove all non-essential personnel from area.  Clean-up personnel should wear proper protective equipment and clothing.  Absorb material with suitable absorbent and containerize for disposal.

7.	SPECIAL PRECAUTIONS

Precautions to be Taken in Handling or Storing:  Store tightly closed in a cool, dry, well ventilated area away from incompatible materials. Suitable for storage in any general chemical storage area.

8.	SPECIAL PROTECTION INFORMATION

Respiratory Protection(Specify Type):

A NIOSH/MSHA chemical cartridge respirator should be worn if PEL or TLV is exceeded. Ventilation:

Local Exhaust:         Preferred
Mechanical(General):   Acceptable
Special:	No

Other:	No

Protective Gloves:

Rubber, neoprene, PVC, or equivalent. Eye Protection:

Splash proof chemical safety goggles should be worn at all times. Other Protective Clothing or Equipment:

Lab apron, eye wash, and safety shower.

9.	PHYSICAL DATA

Molecular Weight:	No information available

Melting Point:	No information available

Boiling Point:	Approximately 100C

Vapor Pressure:	Approximately 17.535 ® 20C

Vapor Density(Air=l):	No information available

Specific Gravity(H2O=1):	1.0

Percent Volatile by Volume:	98%

Evaporation Rate(H2O=l):	1

Solubility in Water:	Complete, product is aqueous solution

Appearance and Odor:	Deep amber liquid with characteristic odor

of iodine.

10.	REACTIVITY DATA

Stability:  Stable

Conditions to Avoid:  High temperatures or excessive heat.

Incompatibility(Materials to Avoid):  Contact with ammonia fumes may

cause formation of explosive nitroiodide.

Hazardous Decomposition Products:  Free Iodine

Hazardous Polymerization:  Will not occur

11.TOXICITYDATA

Page  2   of   4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

LUGOL SOLUTION	Replaces: 07/10/07

CAROLINA BIOLOGICAL	Printed: 05/20/08

Toxicity Data:   Mixture has not been thoroughly evaluated.  Data is

listed for individual components.

Potassium Iodide:  orl-rnus LCLo:  1862 mg/kg

Iodine:  orl-rat LD50:  14g/kg

Effects of Overexposure :

Acute:  See section 3

Chronic:

Potassium Iodide and Iodine:  Mutation data cited.  Reproductive data cited. Not listed as causing by IARC, NTP, OR OSHA

Conditions Aggravated by Overexposure:  Preexisting conditions of the eyes, skin, nose and throat. Target Organs:  No information available.

Primary Route(s) of Entry:  Inhalation, ingestion, irritation in eyes or skin contact.

12.	ECOLOGICAL DATA

EPA-Waste Numbers:  None

13.	DISPOSAL INFORMATION

Waste Disposal Methods:  Dispose in accordance with all applicable

Federal, State and Local regulations.

Always contact a permitted waste disposer (TSD) to assure compliance.

14.	TRANSPORT INFORMATION

Non-regulated

15.	REGULATORY INFORMATION

EPA TSCA Status:  On TSCA Inventory

Hazard Category for SARA Section 311/312 Reporting:  Acute

|  |  | SARA | Sec. 313 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | SARA EHS | Chemicals |  | CERCLA | RCRA |
| Product or | Sec. 302 | Name | Chemical | Sec. 103 | Sec . |
| Components | TPQ | List | Category | RQ Ibs. | 261.33 |
| Potassium Iodide Iodine | No No | No No | No No | No No | No No |

16. ADDITIONAL INFORMATION

The information provided in this Material Safety Data Sheet represents a compilation of data drawn directly from various sources available to us. Carolina Biological Supply makes no representation or guarantee as to the suitability of this information to a particular application of the substance covered in the Material Safety Data Sheet.  Any employer must carefully assess the applicability of any information contained herein in regards to the particular use to which the employer puts the material. Glossary

ACGIH	American Conference of Governmental Industrial Hygienists

CAS Number..Chemical Abstracts Service Number

CERCLA	Comprehensive Environmental Response, Compensation, and

Liability Act

DOT	U.S. Department of Transportation

IARC	International Agency of Research on Cancer

mppcf	million particles per cubic foot

Page 3 of 4

MATERIAL SAFETY DATA SHEET

Revised: 05/15/08

LUGOL SOLUTION	Replaces: 07/10/07

CAROLINA BIOLOGICAL	Printed: 05/20/08

N/A	Not Available

NTP	National Toxicology Program

OSHA	Occupational Safety and Health Administration

PEL	Permissible Exposure Limit

ppm	parts per million

RCRA	Resource Conservation and Recovery Act

SARA	Superfund Amendments and Reauthorization Act

TLV	Threshold Limit Value

TSCA	Toxic Substances Control Act