-

938 P02

MflR 27   '02

13:54 T7

MATERIAL SAFETY DATA SHEET

Radiator Specialty Company

1900 WILKINSON BLVD. CHARLOTTE, NC 28208 (704) 377-6555

POISON INFORMATION & EMERGENCY: 303-623-5716

MATERIAL SAFETY DATA SHEET

May be used to comply with QSHA's Hazard Communication Standard 29 CFR 1910.1200-Standard must be consulted for specific requirements.

US DEPARTMENT OF LABOR

Occupational Safety and Health Administration. (Non-Mandatory Form) Form Approved OMB No. 1218-0072

SECTION I GENERAL INFORMATION

PRODUCT NAME PART NUMBER

RED HI-TEMP SILICONE RTV GASKET T7 - 03V

NOTE: Blank spaces are not permitted. If any Item Is not applicable or no Information is available, the space must be marked to Indicate that.

SECTION II HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

Components not identified are non-hazardous according to 29 CFR 1910.1200 SECTION III PHYSICAL/CHEMICAL CHARACTERISTICS

| COMPONENT WT% CAS. NO, | TLV (ACGIH-X— -OSHA- -) |
| --- | --- |
| Siloxanes and >90 63148-62-9 Siiicones | not established |
| Comments: |  |

pH       Neutral

Specific Gravity (H2O=1) Solubility In Water Flash Point (Method) - F° Melting Point - F° Vapor Pressure (mmHg)

c.a. 1.0

Decomposes

> 200°F. (COG)

N/A

N/A

Solubility In Solvent	N/A

% Voiatiles By Wt.	< 10%

Boiling Point - F°	.N/A

Vapor Density (Air=1)	N/A

Evaporation Rate (Butyl Acetate»1)     N/A

Red paste with vinegar odor.

Appearance and Odor

SECTION IV FIRE AND EXPLOSION HAZARD DATA

EXTINGUISHING MEDIA: Water Fog        X

Foam   X

CO,

Dry Chemical       X

SPECIAL FIRE FIGHTING PROCEDURES, Wear self-contained, positive pressure breathing apparatus and
protective clothes.

UNUSUAL FIRE AND EXPLOSION HAZARDS

None

938 P03         MRR 27  '02    13:55

SECTION V REACTIVITY DATA

Stable  X

Unstable

Corrosive    NO

Hazardous Polymerization? Yes

No   X

Incompatibilities

Moi$t air / open containers will damage product

-

Hazardous Decomposition or Byproducts

Fire will produce silica fumes Water will produce acetic acid

SECTION VI HEALTH HAZARD INFORMATION

Recommended TLV of Product       10 ppm

EYE CONTACT Irritant

SKIN CONTACT Irritant

INHALATION      N/A

INGESTlON        Harmful if swallowed.

OTHER

WA

SECTION VII EMERGENCY AND FIRST AID PROCEDURES

EYE CONTACT Flush thoroughly with water for 15 minutes while lifting eyelids. Consult a physician immediately SKIN CONTACT Wash thoroughly with $oap and water. Remove contaminated clothing and launder before re-use.

INHALATION   Move to fresh air immediately. If breathing is difficult, administer oxygen; if breathing has stopped, apply artificial respiration. Keep warm and summon medical assistance.

INGESTlON       Weak solution of Sodium Bicarbonate (baking soda) to neutralize acid. Induce vomiting. Consult physician
immediately.        	 		,.

SECTION VIII SPECIAL PROTECTION INFORMATION

|  | CONSUMER | BULK HANDLING (Prolonged Exposure) |
| --- | --- | --- |
| RESPIRATORY PROTECTION | NA | If TLV is exceeded, use NlOSH approved respirator |
| Use with VENTILATION | adequate ventilation | Local |
| EYE PROTECTION | NA | Safety glasses with side shields |
| PROTECTIVE CLOTHING | NA | Gloves |

SECTION IX PRECAUTIONS FOR SAFE HANDLING AND USE

SPILL OR LEAK PROCEDURE Observing health hazards described above, remove ignition sources, ventilate area, wipe up with rags and transfer to waste drum for disposal.

WASTE DISPOSAL METHOD Dispose of In accordant* with all applicable government laws and regulations.

STORAGE AND HANDLING PRECAUTIONS

Store in a cool, dry place, keeping container tightly dosed.

OTHER PRECAUTIONS WEARING CONTACT LENSES IS INADVISABLE WHILE USING THIS PRODUCT! KEEP AWAY FROM CHILDREN AND ANIMALS!      RESPECT ALL CHEMICALSl

HAZARD INFORMATION IABEL DATA

Supersedes AUGUST 1996

9GCK

HAZARD CODE i

3-Utah

OSHA Rvvlttd JANUARY 1999 Titto R. GEER - CHEMIST

iWto FteOletat Specialty Company txOewy this data is sera/rafts as of the imlalon (We, we /»**» no warranty nflh reapeof tolheOBle ana we ax-p/ws// tSstMm all natality for re/lancet Hmreon. 77» date is offered sohly for Information, /wesffjwfan, and W/reaffon.