SSS05228 MAINTENANCE SUPPLY COMPANY, INC.

POST OFFICE BOX 498 HUNTERSVILLE, NORTH CAROLINA 28070

- MATERIAL SAFETY DATA SHEET -

THESE MATERIAL SAFETY DATA SHEETS ARE SENT TO YOU IN COMPLIANCE WITH 29 CFR 1910.1200 WHICH BECAME EFFECTIVE NOVEMBER 25, 1985.

IDENTITY AND DISTRIBUTOR'S INFORMATION

nfpa rating: Health, 1    Flammability, 3    Reactivity, 0

Special, 0 HMIS RATING: Health, 1    Flammability, 3    Reactivity, 0

Personal Protection, B

MANUFACTURED FOR:  Triple S

2 Executive Park Drive

Bill erica, MA 01862 dot hazard classification: orm-d identity (trade name as used on label): sss autumn spice td air

deodorizer

date prepared:   02/29/2000       prepared by: dl manufacturer's msds number: a00209 ah revision - 4 distributor's msds reference:   21805 information calls: (978) 667-7900 emergency response number : 1 (800) 255-3924 notice: judgment based on indirect test data

section 1 - material identification and information

components	sara     osha      acgih

CHEMICAL AND COMMON NAMES      III List   PEL (ppm)   TLV (ppm) (Hazardous Components 1% or greater; Carcinogens 0.1% or greater) The specific chemical identities of the ingredients of this mixture are considered to be trade secrets and are in accordance with the provisions of section 1910:1200 of Title 29 of the Code of Federal Regulations.

(CAS # ND TSRN 209AH)

SECTION 2 - PHYSICAL/CHEMICAL CHARACTERISTICS

BOILING POINT: N/A

SPECIFIC gravity, (H20=l): Concentrate Only = <1

vapor PRESSURE: PSIG @ 70 deg. F. (Aerosols): 70

vapor pressure (Non-Aerosols) (mm Hg and Temperature): n/a

VAPOR DENSITY (Ai r=l): N/E

EVAPORATION RATE, (BuAC=l): Faster

SOLUBILITY IN WATER: Soluble

WATER REACTIVE: NO

appearance and odor: clear liquid with fragrance.

-1-

SECTION 3 - FIRE AND EXPLOSION HAZARD DATA

FLAMMABILITY as per USA FLAME PROJECTION TEST (Aerosols):

NON-FLAMMABLE

AUTO IGNITION TEMPERATURE:  N/E FLAMMABILITY LIMITS IN AIR BY % IN VOLUME:

% LEL: N/E        % UEL: N/E

FLASH POINT and METHOD USED (non-aerosols): N/A EXTINGUISHER MEDIA: Foam, dry chemical, carbon dioxide, water. SPECIAL FIRE FIGHTING PROCEDURE: Provide shielding for

personnel.  Wear self-contained breathing apparatus.

Cool containers with water fog to prevent rupturing &

spewing.

Page 1

SSS05228 unusual fire and explosion hazards: Do not expose aeros9ls to

temperatures above 130 degrees F. or the container may rupture.

SECTION 4 - REACTIVITY HAZARD DATA

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not OCCUT

INCOMPATIBILITY (MATERIALS TO AVOID): Alkalis, oxidizing

materials, amines. conditions TO AVOID: Open flame, welding arcs, heat.

HAZARDOUS DECOMPOSITION PRODUCTS: CO, CO2.

SECTION 5 - HEALTH HAZARD DATA

primary ROUTES OF ENTRY: Inhalation

ACUTE EFFECTS: Vapor concentrations around 1000 ppm may cause slight transient irritation to the upper respiratory tract, inhalation: Excessive inhalation of vapors can be

harmful and may cause headache, dizziness,

asphyxia, anesthetic effects and possible

unconsciousness. Eye contact: irritation. Skin contact: May cause slight irritation due to

defatting of skin. Ingestion: Possible chemical pneumonitis if aspirated

into lungs.  Nausea.

chronic effects : (Effects due to excessive exposure to the raw materials of this mixture) May cause mucous membrane irritation, overnight headache and general weakness.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: May

aggravate existing eye, skin or upper respiratory conditions. EMERGENCY FIRST AID PROCEDURES:

eye contact: Flush with water for at least 15 minutes,  if

irritated, seek medical attention. skin contact: wash affected areas with soap and water,  if

irritated, seek medical attention.  Remove contaminated

clothing and launder before reuse. inhalation: Remove to fresh air.  Resuscitate if necessary.

Get medical attention. ingestion: do not induce vomiting. Drink two large glasses of

water. Get immediate medical attention. -2-

SECTION 6 - CONTROL AND PROTECTIVE MEASURES

RESPIRATORY PROTECTION (specify type): If vapor concentration

exceeds TLV, use respirator approved by U.S. Bureau of Mines for organic vapor.

protective gloves: Latex, if skin easily irritated.

eye protection: safety glasses recommended.

ventilation requirements: Adequate ventilation to keep vapor concentration below tlv.

OTHER PROTECTIVE CLOTHING AND EQUIPMENT: None

HYGIENIC WORK PRACTICES: wash with soap and water before handling food.

SECTION  7 - PRECAUTIONS FOR SAFE HANDLING AND USE STEPS TO BE TAKEN IF MATERIAL IS SPILLED OR RELEASED:

Absorb spilled liquid with suitable medium. Place in5SS05228

closed drum for proper disposal,  incinerate or landfill according to local, state or federal regulations. DO not flush TO SEWER.  Remove ignition sources.

WASTE DISPOSAL METHODS:  Aerosol cans when vented to atmospheric pressure through normal use, pose no disposal hazard.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE: Do not puncture

or incinerate containers. Do not store at temperatures above 130 degrees F.

OTHER PRECAUTIONS AND/OR SPECIAL HAZARDS: KEEP OUT OF REACH OF CHILDREN. Avoid food contamination. Avoid breathing vapors.

We believe the statements, technical information and recommendations contained herein are reliable, but they are given without warranty or guarantee of any kind.

-3-