MATERIAL SAFETY DATA SHEET

| PRODUCT NAME CO Contact Cleaner (Aerosol) #2015,201 |  |  | 6# MSDS002015 |  |  |
| --- | --- | --- | --- | --- | --- |
| MANUFACTURED BY: CRC CHEMICALS |  |  | Phone (21 5) 674-4300 |  |  |
| 885 LOUIS DRIVE, WARMINSTER, PA. 18974 |  |  |  |  |  |
| 1. INGREDIENTS | CAS# | ACGIH TLV | OSHA PEL | Other Limits | % |
| Tr ichlorotr if luoroe thane | 76-13-1 | 1000 |  |  | 96 |
| Carbon Dioxide | 124-38-9 | 5000 |  |  | 4 |

2. PHYSICAL DATA     (Does  not  include  carbon dioxide propellent)

| Specific Gravity 1.57 | Vapor Pressure 5.5psia 70 F | % Volatile 100 |
| --- | --- | --- |
| Boiling Point 117. 6°P | Evaporation Rate 0.3 (Carbon tet * 1) |  |
| Freezing Point -35. 7°F | Vapor Density 6.5 (air =- 1) |  |
| Appearance and Odor Colorless liduid |  |  |

Solubility     Slicrht  solubility  in water  -   Soluble   in many  organic   liquids

3. FIRE AND EXPLOSION DATA

| Flashooint None I Method TCC ,. TOC | Fiammable Limits |  | None |
| --- | --- | --- | --- |
| Extinguishing Media Not needed | LEL — | UEL |  |

Unusual Hazards     Aerosol  containers  can  explode   if  contents   are heated   to

nnore  than  130°F

4. REACTIVITY AND STABILITY

Stability      Very  stable	„

Hazardous decomposition products     Halogens,   halogen  acids,   possibility  of  phosgene (Ther-

mall

Materials to avoid    Powdered  metals  such  as  aluminum,   magnesium,   calcium,

2inc

5. PROTECTION INFORMATION

Ventilation  Maintain  concentration  below TLV.

Respiratory    Self-contained  breathing  apparatus  above  TLV.

Gloves      Neonrene

Eye & Face

Safety  goggles

Other Protective Equipment

^Protective clothing only if there is repeated contact

with liguid.

6. HEALTH HAZARD DATA

Primary Routes of Entry       Inhalation   and   Skin

Signs and Symptoms of Exposure

- Acute Overexposure      Inhalation  -  Drowsiness ,   lightheadedness  possible  cardiac
arrhythmias  at high  concentration.     Skin  -  defatting  action,   may  cause
dermititus.     Fyes  - may  irritate  eyes.

- Chronic Overexposure     May  cause   skin   irritation with  repeated  skin  contact.

Medical Conditions Generally Aggravated by Exposure     Impaired   cardiovascular   function   such   a:
cardiac  arrhythmias might be  sensitized  by   inhalation.

Chemical Listed as Carcinogen                                                               National Toxicology         Yes Q             IARC                            Yes D	OSHA        Yes Q

or Potential Carcinogen	Program	NoX3C	Monographs	No jgx	No   jjx

Emergency and First Aid Procedures      (If   syriptorns   persist  call   a  physician)

- Inhalation       Remove  to  fresh  air  -  apply  artificial  respiration  if  necessary

- Eyes     plush with  large  amounts  of water.	^

- Skin     wasn wjth water.

- Ingestion      induce  vomiting  -  call  a  physician.

7.	SPiLL OR LEAK PROCEDURES

Precautions to be taken in	.

Handling and storage	Ptore   in  a  cool  dry  area.

Steps to be taken in case	_	,,.,	,	,-..	.      „	.     ,   -

Materials is released or spilled Due to high volatility material quickly evaporates.

Ventilate area, evacuate personnel until concentration is below TLV

or use self-contained breathing apparatus.

Waste Disposal   If possible pump into drums for reclamation.

8.	SPECIAL PRECAUTIONS AND USE DIRECTIONS

Aerosol containers may explode if exposed to temperatures over 130°F.

Vapors are very heavy and will collect in low areas.  Oxvgen would be

displaced and asphyxiation could result.  Good ventilation is a must

for low areas and confined spaces.

PREPARED BY:   A. B, Reed

ORIGINAL DATE OR DATE OF REVISION   Nov. 1985