938 P14

MflR 27   '02     13:58

M43

MATERIAL SAFETY DATA SHEET

Radiator Specialty Company

1900 WILKINSON BLVD. CHARLOTTE, NC 28208 (704) 377-6555

POISON INFORMATION & EMERGENCY: 303-623-5716

MATERIAL SAFETY DATA SHEET

May be used to comply with OSHA's Hazard Communication Standard 29 CFR 1910.1200. Standard must be consulted for specific requirements. SECTION I GENERAL INFORMATION

US DEPARTMENT OF LABOR

Occupational Safety and Health Administration. (Non-Mandatory Form) Form Approved OMB No. 1218-0072

PRODUCT NAME PART NUMBER

DOT 3 SUPER HEAVY DUTY BRAKE FLUID . M43-12, -32,-34,-40,-49

NOTE: Blank spaces are not permitted. If any Item Is not applicable or no information is available, the space must be marked to Indicate that.

SECTION II HAZARDOUS INGREDIENTS/IDENTITY INFORMATION

COMPONENT

Polyglyco! Ethers and Pioyglycols

WT%

92-99

C.A.S. NO.

PROPRIETARY

TLV (ACGIH	OSHA	)

NDA

NOTE: Our suppliers consider product composition to be proprietary. Reportable components have been reported to EPA chemical substance inventory as required by TSCA. This proprietary mixture contains chemicals that are subject to Reporting Requirements of Section 313 Sara Title III. Comments:

Components not identified are non-hazardous according to 29 CFR 1910.1200

SECTION III PHYSICAL/CHEMICAL CHARACTERISTICS

Specific Gravity (H2O=1) Solubility fn Water Flash Point (Method) - F° Melting Point - F° Vapor Pressure (mmHg)

Evaporation Rate (Butyl Acetate=1)    < 0.01

Appearance and Odor	Light, yellow to dark amber liquid with mild odor.

1.039 Soluble

280°F. (PMCC) Does not apply <0.01 mmHg

pH        N/A

Solubility in Solvent   Alcohols

% Volatiles By Wt.       Not known

Boiling Point - F°        Greate'r than 450

Vapor Density (Air=1) 6

SECTION IV FIRE AND EXPLOSION HAZARD DATA

EXTINGUISHING MEDIA:

Water Fog       XX	Foam   XX

CO2     XX

Pry Chemical

XX

SPECIAL FIRE FIGHTING PROCEDURES. Wear self-contained, positive pressure breathing apparatus and
protective clothes. Do not use a solid stream of water or foam into hot, burning pools: this may cause frothing and
increase fire intensity.

UNUSUAL FIRE AND EXPLOSION HAZARDS   During a fire, Oxides of nitrogen may be produced.

SECTION V REACTIVITY QATA

Stable   XX

Unstable

Corrosive NO

Hazardous Polymerization? Yes

No    XX

Incompatibilities Strong oxidizing agents, Strong alkalies, high temperatures in the presence of strong bases and acids

Hazardous Decomposition or Byproducts   Burning can produce the following products: carbon monoxide and /or carbon dioxide,
oxides of nitrogen.

SECTION VI HEALTH HAZARD INFORMATION

| Recommended TLV of Product Not established. |  |
| --- | --- |
| EYE CONTACT Causes severe Irritant | SKIN CONTACT Prolonged or repeated contact may cause local redness, defatting, and drying of the skin. |
| INHALATION Excessive inhalation of mist or fumes may cause Irritation of mucous membranes. | INGESTION HARMFUL IF SWALLOWED! May cause stomach cramps, nausea, diarrhea. |
| OTHFR N/A |  |

SECTION VII EMERGENCY AND FIRST AID PROCEDURES

EYE CONTACT  Flush with water for at least 15 minutes thoroughly whiie lifting eyelids, Consult a physician immediately.

SKIN CONTACT  Wash with soap and water thoroughly. Remove contaminated clothing and launder before re-use.

INHALATION    Move to fresh air

INGESTION Drink two glasses of water. INDUCE VOMITING!  This should be done only by a medical or experienced first-aid
personnel.   Consult a physician immediately.

SECTION VHJ SPECIAL PROTECTION INFORMATION

|  | CONSUMER | BULK HANDLING (Prolonged Exposure) |
| --- | --- | --- |
| RESPIRATORY PROTECTION | N/A | Not required. |
| VENTILATION | Use with adequate ventilation . | General |
| EYE PROTECTION N/A |  | Goggles, |
| PROTECTIVE CLOTHING | N/A | f~ Not required. |

SECTION IX PRECAUTIONS FOR SAFE HANDLING AND USE

SPILL OR LEAK PROCEDURE Observing health hazards described above, ventilate area, use absorbent, shovel or sweep up.
Place waste in a DOT approved container and seal. Flush residual material with water.

WASTE DISPOSAL METHOD Dispose of In accordance with all applicable government laws and regulations.

STORAGE AND HANDLING PRECAUTIONS   Store away from acids, ignition sources and oxidizing agents. See incompatibilities

listed above.

OTHER PRECAUTIONS  WEARING CONTACT LENSES IS INADVISABLE!   KEEP AWAY FROM CHILDREN AND ANIMALS!

HAZARD INFORMATION LABEL DATA

White Rani'stor SpM'sly Company Believes Ms tfatv it sewate as ottha mvition data, we iriaka no warranty vrith nf*pe« to tns elate and tw mptQswy disclaim all liability Icrmllancs (fiaraon. Tne data Is o/favd iole/lf tor Inhumation. ImosHyaSm, and veiUlcaBon,

SPKW.

HAZARD CODE FWVMAHLITY 4-Extreme 3-High