COMPANY. USA

i  DIVISION  O'   EXXOf.   COR»OHATlON

EXXON HEAVY DUTY 15W-40

b mo   is-

DATE ISSUED         7/29/85

MATERIAL SAFETY DATA SHEET

EXXON COMPANY, U.S.A.    P.O. BOX 2180    HOUSTON. TX 77252-2180

| A IDENTIFICATION i | AND EMERGENCY INFORMATION |  |
| --- | --- | --- |
| PRODUCT NAME &l*— «EXXDN" HEAVY DUTY 15W-40 | PRODUCT CODE 212041 - 02041 |  |
| CHEMICAL NAME Petroleum Lubricating 011 | CAS NUMBER Complex Mixture CAS Number not appl | 1 cable |
| PRODUCT APPEARANCE AND ODOR Clear, dark amber liquid Mild, bland petroleum odor |  |  |
| EMERGENCY TELEPHONE NUMBER (713) 656-3424 |  |  |

B.    COMPONENTS AND HAZARD INFORMATION

COMPONENTS

CAS NO. OF   APPROXIMATE COMPONENTS   CONCENTRATION

Lubricating Oil Base Stock

Proprietary additives

64742-54-7

or 64742-65-0

Mixture

Greater than 80«

Less than 20°/=

EXPOSURE LIMIT FOR TOTAL PRODUCT   BASIS

5 mg/m3 for oil rmst in air       OSHA Regulation 29 CFR 1910.100O

C.    EMERGENCY AND FIRST AID PROCEDURES

EYE CONTACT

If splashed into the' eyes, flush with clear water for 15 minutes or until irritation subsides.  If irritation persists, call a physician.

SKIN CONTACT

In case of skin contact, remove any contaminated clothing and wash skin thoroughly with soap and water.

INHALATION

Vapor pressure 1s very low.  Vapor inhalation under ambient conditions 1s normally not a problem.  If overcome by vapor from hot product, immediately remove from exposure and call a physician.  If breathing is irregular or has stopped, start resuscitation; administer oxygen, if available.  If overexposed to oil mist, remove frorr. further exposure until excessive oil mist condition subsides.

945-C277IMWH001)

EXXON   HEAVY   DUTY    15W-40

INGESTION

If Ingested., call e physician Immediately.

D.    FIRE AND EXPLOSION HAZARD INFORMATION

FLASH POINT (MINIMUM)	AUTOIGNITIDN TEMPERATURE

199'C (390'F)	Greater than 260'C (SOO'F)

ASTM D 92. Cleveland Open Cup

FLAMMABLE OR EXPLOSIVE LIMITS (APPROXIMATE PERCENT BY VOLUME IN AIR)

Estimated values: Lower Flammable Limit 0.9/4     Upper Flammable Limit 7%

EXTINGUISHING MEDIA AND FIRE FIGHTING PROCEDURES

Foam, water spray (fog), dry chemical, carbon dioxide and vaporizing liquid type extinguishing agents may all be suitable for extinguishing fires Involving this type of product, depending on Size or potential size of fire and circumstances related to the situation.  Plan f1 re protection and response stretagy through consultation with local f1 re protection authorities or appropriate specialt1sts.

The following procedures for this type of product are based on the recommendations in the National Fire Protection Association's "Fire Protection Guide on Hazardous Materials". Eighth Edition (1984 ) :

Use water spra^, dry chemical, foam, or carbon dioxide.  Water or foam may cause frothing. Use water to keep fire-exposed containers cool.  Water spray may be used to flush spills away from exposures.  Minimize breathing gases, vapor, fumes or decomposition products. Use supplied-air breathing equipment for enclosed or confined spaces or as otherwise needed.

DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS

Fumes, smoke, carbon monoxide, sulfur oxides, aldehydes and other decomposition products, in the case of incomplete combustion.

"EMPTY" CONTAINER WARNING

"Empty" containers retain residue (liquid and/or vapor) and can be dangerous.  DO NOT PRESSURIZE , CUT, WELD, BRAZE. SOLDER. DRILL. GRIND OR EXPOSE SUCH CONTAINERS TO HEAT. FLAME, SPARKS OR OTHER SOURCE? OF IGNITION; THEY MAY EXPLODE AND CAUSE INJURY OR DEATH. Do not attempt to clean since residue is difficult to remove.  "Empty" drums should be completely drained, properly bunged and promptly returned to a drum reconditloner.  All other containers should be disposed of in an environmentally safe manner and 1n accordance with governmental regulations.  For work on tanks refer to Occupational Safety and Health Administration regulations, ANSI Z49.1, and other governmental and Industrial references pertaining to cleaning, repairing, welding, or other contemplated operat ions .

E.    HEALTH AND HAZARD INFORMATION

VARIABILITY AMONG INDIVIDUALS

Health studies have shown that many petroleum hydrocarbons and synthetic lubricants pose potential human health risks which may vary from person to person.  As e precaution, exposure to liquids, vapors, mists or fumes should be minimized.

EFFECTS OF OVEREXPOSURE (Signs and symptoms of exposure)

Prolonged or repeated skin contact may cause skin irritation.

NATURE OF HAZARD

Prolonged or repeated skin contact with this product tends to remove skin oils possibly leading to irritation and dermatitis.

^r~-Product contacting the eyes may cause eye Irritation.

TOXICITY INFORMATION

Product has a low order of acute oral toxidty.

9<5-0277(HWH002)

EXXON HEAVY DUTY 15W-40

F.    PHYSICAL DATA

The following data are approximate or typical values and should not be used for precise design purposes.

BOILING RANGE

IBP Approximately 3O2" C (575'F)

SPECIFIC GRAVITY (1B.6 C/15.6 C) 0.89

MOLECULAR WEIGHT

Not determined

pH

Essential ly neutral

POUR, CONGEALING OR MELTING POINT

-26' C (-15' F )

Pour Point by ASTM D 97

VAPOR PRESSURE

Less than 0.01 mm Hg f 20'C

VAPOR DENSITY (AIR

1)

Greater than 5

PERCENT VOLATILE BY VOLUME

Negligible from open container In A hours f 3B'C (1OO'F)

EVAPORATION RATE P 1 ATM. AND 25 C (77 F) (n-BUTYL ACETATE •= 1)

Less than 0.01

SOLUBILITY IN WATER *> 1 ATM. AND 25 C (77 F)

Negligible; less than 0.1%

VISCOSITY

n .2 cst

100' c

G.    REACTIVITY

This product is stable and will not react violently with water. Hazardous polymerization will not occur. Avoid contact with strong oxidants such as liquid chlorine, concentrated oxygen, sodium hypochlorite or calcium hypochlorite.

H.    SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED

Recover free product.  Add sand, earth, or other suitable absorbent to spill area.  Minimize skin contact.  Keep product out of sewers 'and watercourses by diking or impounding.  Advise authorities 1f product has entered or may enter sewers, watercourses, or extensive land areas. Assure -conform 1ty with applicable governmental regulations.

I.    PROTECTION AND PRECAUTIONS

VENTILATION

Use local exhaust to capture vapor, mists or fumes, if necessary.  Provide greater than 60 f«et per minute hood face velocity for confined spaces.  Provide ventilation sufficient to prevent exceeding recommended exposure limit or buildup of explosive concentrations of vapor 1n air.  Use explosion-proof equipment.  No smoking or open lights.

RESPIRATORY PROTECTION

Normally not needed at ambient temperatures. 1n confined or enclosed spaces, if needed.

Use supplied-air respiratory protection

PROTECTIV£ GLOVES

Use chemical-resistant gloves, if needed, to avoid prolonged or repeated skin contact.

945-0277IMWK002!

EXXON HEAVY DUTY 15W-40

EYE PROTECTION	f

Use splash goggles or face shield when eye contact may occur.

OTHER PROTECTIVE EQUIPMENT

Use chemical-resistant apron or other Impervious clothing, if needed, to avoid contaminating regular clothing which could result in prolonged or repeated skin contact.

WORK PRACTICES / ENGINEERING CONTROLS •Keep containers closed when not in use. or strong oxidantE.

Do not handle or store near heat, sparks, flame

PERSONAL HYGIENE

Minimize breathing vapor, mist or fumes.  Avoid prolonged or repeated contact with skin. Remove contaminated clothing; launder or dry-clean before reuse.  Remove contaminated shoes and thoroughly clean before reuse; discard if oil-soaked.  Cleanse skin thoroughly often contact, before breaks and meals, and at end of work period.  Product is readily removed from skin by waterless hand cleaners followed by washing thoroughly with soap and water.

J.    TRANSPORTATION INFORMATION

TRANSPORTATION INCIDENT INFORMATION

For further information relative to spills resulting from transportation incidents, refer to latest Department of Transportation Emergency Response Guidebook for Hazardous Materials Incidents. DOT F 5800.3.