[Gliddenl    The Glidden Company Cleveland, Ohio 44115

MATERIAL SAFETY DATA SHEET

Pa/nfs World Leader

FIRE AND EXPLOSION HAZARD DATA

Extinguishing media: Dry chemical or foam water fog. Carbon dioxide.

Unusual fire and explosion hazards : Vapors are heavier than air and may travel long distances to a source of ignition and flash back. Closed containers may burst if exposed to extreme heat or fire. May decompose under fire conditions emitting irritant and/or toxic gases. In closed tanks, water or foam may cause frothing or eruption.

Special Ore fighting procedures : Water may be used to cool and protect exposed containers. Firefighters should use full protective clothing, eye protection, and self-contained breathing apparatus. Self-contained breathing apparatus recommended.

HEALTH HAZARD DATA

Primary route(s) of exposure: Inhalation, skin contact, eye contact, ingestion.

Effects of overexposure:

Inhalation: Irritation of respiratory tract, lungs. Prolonged inhalation may lead to mucous

membrane irritation, drowsiness, dizziness and/or lightheadedness, headache, uncoordination, nausea, vomiting, diarrhea, gastro-intestinal disturbances, flu-like symptoms, coughing, choking, sneezing, apathy, central nervous system depression, respiratory problems, anesthetic effect or narcosis, difficulty of breathing, blood abnormalities, fever and chills, abnormal blood pressure, kidney damage, pneumoconiosis, loss of consciousness.

Skin contact: Irritation of skin. Prolonged or repeated contact can cause dermatitis. Skin contact may result in dermal absorption of component(s) of this product which may cause apathy.

Eye contact: Irritation of eyes. Prolonged or repeated contact can cause conjunctivitis, tearing of eyes, redness of eyes, severe eye irritation or burns, corneal injury, blindness.

Ingestion: Ingestion may cause mouth and throat irritation, drowsiness, dizziness and/or

lightheadedness, headache, uncoordination, nausea, vomiting, diarrhea, gastro-intestinal disturbances, severe abdominal pain, abdominal pain, apathy, central nervous system depression, respiratory problems, intoxication, liver damage, kidney damage, reproductive system damage, pulmonary edema, loss of consciousness, acute poisoning, respiratory failure, cardiac failure, brain damage, death.

Supplemental health Information: Other effects of overexposure may include toxicity to liver, kidney, lungs, blood, reproductive system. May be absorbed through skin. Excessive inhalation of fumes may lead to metal fume fever characterized by a metallic taste in mouth, excessive thirst, coughing, weakness, fatigue, muscular pain, nausea, chills and fever. Notice - reports have associated repeated and prolonged occupational overexposure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal. Some laboratory tests results have shown ethylene glycol to be an animal teratogen. Contains crystalline silica which is considered a hazard by inhalation. IARC has classified crystalline silica as probably carcinogenic for humans (2a). This classification is based on the findings of laboratory animal studies that were considered sufficient and data from epidemiological studies that were considered limited for carcinogenicity. Crystalline silica is also a known cause of silicosis, a noncancerous lung disease. NTP has classified crystalline silica a reasonably anticipated human carcinogen. Prolonged ingestion of diethylene glycol monomethyl ether has resulted in fetal development abnormalities in rats and effects on fertility in mice.

Medical conditions aggravated by exposure : Eye, skin, respiratory disorders asthma-like conditions kidney disorders

FIRST AID PROCEDURES

Inhalation: Remove to fresh air. Restore and support continued breathing. Get emergency medical attention. Have trained person give oxygen if necessary. Get medical help for any breathing difficulty. Remove to fresh air if inhalation causes eye watering, headaches, dizziness, or other discomfort. Get medical attention if discomfort or irritation persists.

Skin contact: Flush from skin with water. Then wash thoroughly with soap and water. Remove

contaminated clothing. Wash contaminated clothing before re-use. If irritation occurs, consult a physician.

Eye contact: Flush immediately with large amounts of water, especially under lids for at least 15 minutes. If irritation or other effects persist, obtain medical treatment.

Ingestion : If swallowed, obtain medical treatment immediately.

REACTIVITY DATA

Stability: Stable

Incompatibility : Oxidizers, acids, reducing agents, bases, halogens. Alkalis carbon tetrachloride (at elevated temperatures), aluminum, zinc, hypochlorites, metal compounds, hydrogen fluoride, magnesium, caustics, sodium, potassium.

Conditions to avoid : Elevated temperatures, moisture, contact with oxidizing agent, contact with aluminum or zinc, storage near acids, freezing, sparks, open flame. Ignition sources

Hazardous decomposition products : Carbon monoxide, carbon dioxide, oxides of nitrogen, acrid

fumes, oxygen, toxic gases. Hazardous polymerization: Will not occur

SPILL OR LEAK PROCEDURES

Steps to be taken in case material is released or spilled : Comply with all applicable health and environmental regulations. Eliminate all sources of ignition. Ventilate area. Spills may be collected with absorbent materials. Evacuate all unnecessary personnel. Place collected material in proper container. Complete personal protective equipment must be used during cleanup. Large spills -shut off leak if safe to do so. Dike and contain spill. Pump to storage or salvage vessels. Use absorbent to pick up excess residue. Keep salvageable material and rinse water out of sewers and water courses. Small spills - use absorbent to pick up residue and dispose of properly.

Waste disposal: Dispose in accordance with all applicable regulations. Avoid discharge to natural waters.

SPECIAL PROTECTION INFORMATION

Respiratory protection: Where respiratory protection is required, use only NIOSH/ MSHA approved respirators in accordance with OSHA standard 29 CFR 1910.134.

Ventilation : Provide dilution ventilation or local exhaust to prevent build-up of vapors. Use explosion-proof equipment.

Personal protective equipment: Eye wash, safety shower, safety glasses or goggles. Impervious gloves, impervious clothing, apron.

SPECIAL PRECAUTIONS

Handling and storage : Store below lOOf. Keep away from heat, sparks and open flame. Keep from freezing. Do not store in aluminum containers. Keep container tightly closed in a well-ventilated area.

Other precautions : Use only with adequate ventilation. Do not take internally. Keep out of reach of children. Avoid contact with skin and eyes, and breathing of vapors. Wash hands thoroughly after handling, especially before eating or smoking. Keep containers tightly closed and upright when not in use. Avoid conditions which result in formation of inhalable particles such as spraying or abrading (sanding) painted surfaces. If such conditions cannot be avoided, use appropriate respiratory protection as directed under special protection information. Empty containers may contain hazardous residues.

The information contained herein is based on data available at the time of preparation of this data sheet which The Glidden Company believes to be reliable. However, no warranty is expressed or
implied regarding the accuracy of this data. The Glidden Company shall not be responsible for the use of this information, or of any product, method or apparatus mentioned and you must make your
own determination of its suitability and completeness for your own use, for the protection of the environment, and the health and safety of your employees and the users of this material.
Complies with OSHA hazard communication standard 29CFR1910.1200.	prepared 07/05/95	EMERGENCY TELEPHONE NO. (800)545-2643

Physical Data

| Product Code | Description | Wt/Gal. | voc Er./ltr. | % Volatile bv Volume | Flash Point | Boiling Ranee | HMIS | DOT, proper shipping name |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6600 | pure white | 10.40 | 229.90 | 67.49 | none | 212-477 | *110 | paint1**freezable** |
| 6610 | artisan brown | 8.90 | 246.13 | 71.06 | none | 212-477 | *110 | paint.**freezable** |
| 6611 | liberty red | 9.46 | 247.79 | 70.05 | none | 212-477 | *110 | paint, **freezable** |
| 6612 | green bank | 9.01 | 241.65 | 69.69 | none | 212-477 | *310 | paint,**freezable** |
| 6613 | Stewart house brown | 8.87 | 247.17 | 71.39 | none | 212-477 | *110 | paint,**freezable** |
| 6614 | black | 8.71 | 245.15 | 71.66 | none | 212-477 | *110 | paint,**freezable** |
| 6618 | pastel tint base | 10.30 | 221.87 | 65.97 | none | 212-477 | *110 | paint.**freezable** |
| 6625 | white | 10.35 | 218.31 | 64.40 | none | 212-477 | *110 | paint,**freezable** |
| 6680 | deep tint base | 8.78 | 243.93 | 72.08 | none | 212-477 | *110 | paint,**freezable** |
| 6687 | intermediate tint base | 9.33 | 243.22 | 70.03 | none | 212-477 | *110 | paint,**freezable** |
| 6690 | accent base | 8.46 | 178.58 | 71.52 | none | 212-900 | *310 | paint, **freezable * * |
| 6700 | pure white | 10.31 | 94.21 | 63.45 | none | 212-477 | *110 | paint,**freezable** |
| 6710 | artisan brown | 9.63 | 230.49 | 69.71 | none | 212-477 | *111 | paint,**freezable** |
| 6711 | liberty red | 9.83 | 231.28 | 70.00 | none | 212-477 | *110 | paint,**freezable** |
| 6712 | ereen bank | 9.55 | 223.60 | 68.81 | none | 212-477 | *110 | paint,**freezable** |
| 6713 | Stewart house brown | 9.45 | 233.21 | 70.17 | none | 212-477 | *111 | paint,**freezable** |
| 6718 | pastel tint base | 10.14 | 99.97 | 65.68 | none | 212-477 | *110 | paint,**freezable** |
| 6725 | satin white | 10.32 | 94.47 | 63.71 | none | 212-477 | *110 | paint,**freezable** |
| 6780 | deep tint base | 9.39 | 233.50 | 70.19 | none | 212-477 | *110 | paint,**freezable** |
| 6787 | intermediate tint base | 9.89 | 218.67 | 67.79 | none | 212-477 | *110 | paint,**freezable** |

|  |  |  | ACGIH-TLV |  | OSHA-PEL |  |  |  | S.R. STD. | S 2 | S 3 | C C |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHEMICAL NAME | COMMON NAME | CAS. NO. | 8-HOUR TWA | STEL | 8-HOUR TWA | STEL | C | S |  |  |  |  | N | I | O |
| 1,2-ethanediol | ethylene glycol | 107-21-1 | c 50 ppm | ne | ne | ne | 50ppm | ne | ne |  |  | V | n | n | n |
| poly(oxy-l,2-ethanediyl), .alpha.-(nonylphenyl)-.omega.-hydroxy-branched | nonyl phenol surfactant | 68412-54-4 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| petroleum hydrocarbon, proprietary blend, hvdrophobic silica | defoamer | mixture | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| c.i. pigment green 7 | phthalocyanine green | 1328-53-6 | 10mg/m3 | ne | 15 mg/m3 | ne | ne | ne | ne | n | n | V | n | n | n |
| c.i. pigment yellow 42 | iron oxide yellow | 51274-00-1 | 5mg/m3 | ne | 10 me/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| ceramic materials and wares, chemicals | aluminum silicate | 66402-68-4 | 10mg/m3 | ne | 15 mg/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| kieselguhr, soda ash flux-calcined | silica, diatomaceous earth | 68855-54-9 | 10mg/m3 | ne | 6mg/m3 | ne | ne | ne | ne | n | n | n | n | Y | n |
| iron oxide | ferric oxide | 1332-37-2 | 5mg/m3 | ne | 10 nig/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| ethanol, 2-(2-methoxvethoxv)- | diethylene glycol monomethyl ether | 111-77-3 | ne | ne | ne | ne | ne | ne | 30 ppm |  |  | n | n | n | n |
| ethanol, 2-(2-butoxvethoxvV | diethylene glycol monobutyl ether | 112-34-5 | ne | ne | ne | ne | ne | ne | ne |  |  | n | n | n | n |
| propanoic acid, 2-methyl-, monoester with 2,2,4-trimethyl-l,3-pentanediol | texanol | 25265-77-4 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| cristobalite | crystalline silica, cristobalite | 14464-46-1 | 0.05 mg/m3 | ne | 0.05 mg/m3 | ne | ne | ne | ne | n | n | n | n | y | n |
| aluminum hydroxide | same | 21645-51-2 | 10 mg/m3 | ne | 5 mg/m3 | ne | ne | ,ne | ne | n | n | n | n | n | n |
| titanium oxide | titanium dioxide | 13463-67-7 | 10 mg/m3 | ne | 10 mR/m3 | ne | ne | ne | ne | n | n | n | n | n | n |
| zinc | zinc | 7440-66-6 | ne | ne | ne | ne | ne | ne | ne |  |  | y | n | n | n |
| acrylic copolymer | same | sup. conf. | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| acrylic copolymer | same | sup. conf. | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| acrylic copolymer | same | sup. conf. | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| styrene acrylic copolymer | same | sup. conf. | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| nepheline syenite | nepheline syenite | 37244-96-5 | 5 mg/m3 | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| nepheline syenite | nepheline syenite | 37244-96-5 | 5 mg/m3 | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |
| Water | water | 7732-18-5 | ne | ne | ne | ne | ne | ne | ne | n | n | n | n | n | n |

Footnotes:

C = Ceiling - Concentration that should not be

exceeded, even instantaneously.

S = Skin - Additional exposure, over and above airborne exposure, may result from skin absorption.

S.R.STD. = Supplier Recommended Standard n/a = not applicable ne = not established

ppm = parts per million

mg/m3 = milligrams per cubic meter

S2 = Sara Section 302 EHS

S3 = Sara Section 313 Chemical CC = CERCLA Chemical

Carcinogenicity Listed By:

N = NTP, I = IARC, O = OSHA

y = yes, n = no