FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

MSDS #:    427.00

Material Safety Data Sheet (MSDS)	Revision Date: November 25,2002

Section 1 — Chemical Product and Company Identification

Lactic Acid

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition. Information on Ingredients

Lactic Acid Synonym: acetonic acid

CAS#: 50-21-5

Section 3 — Hazards Identification

Colorless or yellowish, hygroscopic syrupy liquid. Odorless.	flinn Al-A-GLANCE

Corrosive to eyes; irritates skin and respiratory tract. Slightly toxic by ingestion.	Health-1

Flammability-1 Reactivity-1 Exposure-2 Storage-1

0 is low hazard, 3 is high hazard

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

Section 5 — Fire Fighting Measures

Combustible liquid.	NFPACODE

Flash Point: 230 F	None Established

When heated to decomposition, emits toxic acrid smoke and irritating fumes.

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear PPE

and SCBA with full facepiece operated in positive pressure mode.

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area and ventilate area. Contain spill with sand or absorbent material; deposit in sealed bag or container. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic #1. Store with acids, anhydrides and peracids. Store in a cool dry place. Deliquescent, store in Flinn Chem-Saf Bag. Use and dispense in a hood.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use exhaust ventilation to keep airborne concentrations low.

) 2002 Flinn Scientific, Inc. All Rights Reserved.	page 1 OF 2

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

Lactic Acid

MSDS#:    427.00

Revision Date:   November 25, 2002

Section 9 — Physical and Chemical Properties

Colorless or yellowish, hygroscopic syrupy liquid. Odorless. Solubility: Miscible with water and other solvents. Formula: C3H6O3 Formula Weight: 90.09

Specific Gravity: 1.2 Melting Point: 18 C

Section 10 — Stability and Reactivity

Avoid contact with bases, oxidizing agents and reducing agents. Shelf life: Poor; substance hygroscopic.

Section 11 —Toxicoloqical Information

Acute effects: Corrosive Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: 3730 mg/kg IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #24a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

TSCA-listed, EINECS-listed (200-018-0), RCRA code D002.

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Improve Student Lab Results-Use Flinn Chemicals

flinn@flinnsci.com  www.flinnsci.com

P.O. Box 219   BatavialL 60510 (800) 452-1261 Fax (866) 452-1436

> 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2