MATERIAL SAFETY DATA SHEET

(comlies with OSHA Communication Standard 29 CFR 1910.1209 Dept. of Labor)

CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

PRODUCT NAME TM3208

Battery Protector

Revision Date : 4/9/98

MANUFACTURER/SUPPLIER’S NAME: Hi-Port Aerosol, 9150 Valley View Road, Mandedoneia, OH  44056

TELEPHONE NUMBER (330) 487-4195

Product .		Composition		ACGIH TLV		OSHA PEL

CAS REG NO. UNITS	wgt%		TWA      STEL		TWA    STEL

Acetone

67-64-1		30-40		750ppm    1000ppm	1000ppm   NE

Propane-Isobutane-n-Butane

74-98-6		20-30		1000ppm   Asphyxiant        1000ppm  Asphyxiant

1.1.1 Trichloroethane

71-55-6		10-20		350ppm    450ppm	350ppm  450ppm

Toluene

108-88-3		1-10		50ppm skin   NA	200ppm    NA

Ethyl Acetate

141-78-6		1-10		400ppm    NE		400ppm  NE

2-Butoxyethanol

111-76-2		1-10		25ppm skin  NE		50ppm skin  NE

Carbon Black

1333-86-4		0.3 -10		35mg/m3  35mg/m3

Additional Component Information:  1,1,1 Trichloroehane contains an inhibitor package which includes 15% Tept-butly Alcohol (75-65-0), 0.5% 1,2, Butylene Oxide (106-88-7), and 19% Dunethoxymerhane (109-87-5) Trt-butyl Alcohol and 1,2 Buylene Oxide are subject to the reporting requirements of Section 313 of SARA Title III.  Acceptable ceiling concentration for toluene – 300ppm, max peak above ceiling ---8 hr shift --- 500ppm

HEALTH HAZARD DATA (ACUTE AND CHRONIC)

May cause dizziness or narcosis in high vapor concentrations.  Will cause defatting of skin.  Effects are reversible.  Long term exposure (years) to high concentrations of vapor may cause lung, liver or kidney damage.  The solvents listed have been reported to affect the central nervous system.  Aspiration hazard if swallowed.  Eye and skin irritant.  May irritate respiratory tract.  Deliberately concentrating and inhaling the vapor of the contents may be harmful or fatal.

Other:  2-Butoxyehanol may cause corneal injury and blood abnormailities, may be absorbed through the skin with toxic effects, and may cause damage to spleen and testes.  Ethyl Acetate may cause anemia.  Eye irritation may be severe.  May cause cardiac abnormalities.  Overexposure to toluene may cause nasal and brain damage.

Carcinogenicity:  Carbon black is listed as a potential carcinogen per IARC.

First Aid:  Give oxygen.  Do not induce vomiting.  Wash eyes and skin with water for 15 minutes.  Get medical help. Never administer adrenain following overexposure.

Routes of Entry:  Ingestion, inhalation, and skin

Signs and symptoms of Overexposure:  Inhalation – difficulty in breathing.  Skin – redness.  Ingestion – vomiting

Medial Conditions Aggravated by Exposure:  Heart disease, respiratory disorders

Boiling Point -40 to 343 degrees F

Vapor Pressure at 70 degrees F		60

Vapor Density (air=1)	4.0

Evaporation Rate (Butyl Acetate=1)	>1

Specific Gravity (water=1)	0.8

Solubility: nil

Appearance:  Black

Odor:	Solvent

pH:  Not applicable

Stability:	Stable

Incompatibility:	Strong oxidizers, strong alkalies, strong acids and bases, strong mineral acids. Alkylene Oxides

Hazardous Thermal Decomposition/Combustion Products:  Carbon dioxide, halogen acids, phosgene. Oxides of nitrogen.

Hazardous Polymerization:  Will not occur

California Proposition as:

Warning:  This product contains a chemical known to the State of California to cause cancer and birth defects or other reproductive harm.

WARNING:  Contains 1,1,1 Trichloroethane, a substance which harms public health and environment by destroying ozone in the upper atmosphere.

HMIS:  Flammability – 4, Reactivity – 0, Health – 2

NFPA:  Flammability – 4, Reactivity – 0, Health – 2

SARA 313 Title IV – Toxic Chemical List:  1.1.1 Trichloroethane (CAS 71-55-6) 2-Butoxyethanol (CAS 111-76-2) Toluene (CAS 108-88-3)None

DOT Shipping Name:  Consumer Commodity, ORM-D

Respiratory Protection Equipment:  Use NIOSH approved respirator if TLV limit is exceeded

Ventilation:  Local exhaust – yes

Protective Gloves:  None required if spraying

Eye and Face Protection:  Safety glasses

Other Protective Equipment:  Long sleeves and long pants

Safe Handling:  Do not smoke while using.  Wash hands after use.

Flash Point:  -342F TCC

Lower Explosion Limit:  1.8

Upper Explosion Limit:  9.5

Extinguishing Media:  Water fog, carbon dioxide, foam, dry chemical

Unusual Fire and Explosion Hazards:  Heated cans my burst.

Special Fire Fighting Procedures:  Keep containers cool.  Use equipment or shielding required to protect against bursting or venting containers.

Spill Procedures:  Use absorbent sweeping compound to soak up materials.  Put into container.  Dispose as hazardous waste.

Waste Disposal Methods:  Dispose as hazardous waste in accordance with EPA RCRA

Precautions to be taken in handling and storage:  Keep away from heat, sparks, or open flame.  Store at Temperatures below 120 degrees F.

Other Information:  When spraying more than one half can continuously or more then one can consecutively use NIOSH NP 300 level.