MATERIAL SAFETY DATA SHEET: X-433 AEROSOL

Page: 1

(000000-000000-

-5033

DA-IE   OF   ISSUE 12/07/2005

SUPERSEDES 1/05/2005

SECTION   I   -   GENERAL   INFORMATION

ical Name & Synonyms

Trade Name & Synonyms X-433 AEROSOL

Chemical Family: PETROLEUM MIXTURE

Formula Mixture —> X

Manufacturer's Name:

CERTIFIED LABS, DIV. OF NCH CORP.

Address:

BOX 152170

IRVING,	TEXAS 75015

Prepared By:

L   Boynton/Chemist

Product  Code  Number 5033

Emergency   Phone   Number 800-424-9300

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

| Chemical Name (Ingredients) | Hazard | TLV |  | PEL | STEL |  | CAS # |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SODIUM SULFONATE | IRRITANT | 5 MG/M3S | 1 | 5 MG/M3$ 2 | 1GMG/M3$ | 1 | 68608-26- | 4 |
| HEXYLENE GLYCOL | IRRITANT1 | N/E 1 |  | N/E 2 | 25 PPM* | 1 | 107-41-5 |  |
| MINERAL SPIRITS | IRRITANT | 5 MG/M3$ | 1 | 5 MG/'M3$ 2 | 10MG/M3$ | 1 | 8052-41-3 |  |
| SEVERELY HYDROTREATED LIGHT NAPHTHENIC DISTILLATE | IRRITANT | 5 MG/M3$ | 1 | 5 MG/M3$ 2 | 10MG/M3$ | 1 | 64742-53- | 6 |
| SEVERELY HYDROTREATED HEAVY NAPHTHENIC PETROLEUM DISTILLATE | IRRITANT | 5 MG/M3$ | 1 | 5 MG/M3S 2 | 10MG/M3$ | 1 | 64742-52- | 5 |
| PROPANE | ASPHYXIANT | 1000 PPM#1 |  | 1000 PPM 2 | N/E |  | 74-98-6 |  |
| N- BUTANE | ASPHYXIANT | 1000 PPMS1 |  | N/E 2 | N/E |  | 106-97-8 |  |
| POLYBUTENE COPOLYMER | IRRITANT | N/E 1 |  | N/E 2 | N/E |  | 9003-29-6 |  |
| $ OIL MIST VALUES |  |  |  |  |  |  |  |  |
| * CEILING LIMIT |  |  |  |  |  |  |  |  |
| # ALIPHATIC HYDROCARBON GASES |  |  |  |  |  |  |  |  |

SECTION Ila - NON-HAZARDOUS INGREDIENTS (NON-HAZARDOUS INGREDIENT NAMES AND CAS NUMBERS ARE PROTECTED UNDER NJ TRADE)

Secret Registry #:

SECTION III - PHYSICAL DATA

| Boiling Point (F) : 500" | Specific | Gravity (H20=l) : 0.9 |
| --- | --- | --- |
| Vapor Pressure (MM HG) : 2024 | Color: | BLACK |
| Vapor Density (Air=l) : 1.7 | Odor: | PETROLEUM |
| PH @ 100% : N/A | Clarity: | OPAQUE |
| ft Volatile by Volume; 22 | Evaporati | on Rate (BU A/C=1) : 24.6 |
| H20 Solubility: NEGLIGIBLE | Viscosity | •: non- viscous |
|  | SECTION IV - FIRE | AND EXPLOSION HAZARD |
| Flash Point >200°F / SETAFLASH | Flammable Limits HEX GLYCOL /PRO PANE | LEL UEL 1.2% 9.5% |
| Extinguishing Media X <--Foam X <--Alcohol Foam | X < — CO2 X < — Dry Chemical | X < — Water Spray < — Other |

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECT TVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF

THE SURROUNDING FIRE. COOL FIRE-EXPOSED CONTAINERS WITH HATER SPRAY TO PREVENT BURSTING.

Unusual Fire and Explosion Hazards:

THE USE OF WATER SPRAY (FOG) WHILE EFFECTIVE, MAY CAUSE FROTHING AND FOAMING.  NEVER USE A WATER JET AS THIS WILL JUST SPREAD THE FIRE. PRODUCT MAY

PRODUCE A FLOATING FIRE HAZARD AS LIQUID FLOATS ON WATER. FLAME EXTENSION: 10 INCHES, BURNBACK: 0 INCHES,

Aerosol Level (NFPA SOB):    3

NFPA 704 Hazard Rating       (0=Insignificant l=Slight  2=Moderate  3=High  4=Extreme) 2  <—Health       4  <—Flammability  0  <—Instability      <—Special

SECTION V - HEALTH HAZARD DATA

Threshold Limit Value:

NOT ESTABLISHED FOR MIXTURE.   SEE SECTION II.

Effec ts of Overexposure:

-Acute (Short Term Exposure)

eye contact: causes severe irritation seen as tearing, swelling, .stinging, redness, excessive blinking, a burning sensation, and blurred vision. may

MATERIAL SAFETY DATA SHEET: X-433 AEROSOL	Page: 2

SECTION V - HEALTH HAZARD DATA (Continued)

CAUSE CORNEAL DAMAGE.

SKIN CONTACT: MAY CAUSE ALLERGIC SKIN REACTION SEEN AS DELAYED SKIN RASH WHICH MAY BE FOLLOWED BY BLISTERING, SCALING, AND OTHER SKIN EFFECTS. MAY CAUSE IRRITATION SEEN AS BLISTERING, ITCHING, REDNESS, SWELLING, AND A BURNING SENSATION. PROLONGED OR REPEATED CONTACT AS FROM CLOTHING WET WITH MATERIAL MAY CAUSE DRYING, DEFATTING, AND CRACKING OF THE SKIN.

INHALATION: MAY CAUSE CHEST PAIN AND RESPIRATORY IRRITATION SEEN AS COUGHING, SNEEZING, AND NASAL DISCHARGE. AT LOW VAPOR CONCENTRATIONS, NO HARMFUL       ^"^ EFFECTS ARE EXPECTED. AT HIGH VAPOR CONCENTRATIONS, INHALATION MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SUCH AS HEADACHE, DIZZINESS, DROWSINESS, WEAKNESS, UNCONCIOUSNESS, POSSIBLE ANESTHETIC EFFECTS FROM CENTRAL NERVOUS SYSTEM DEPRESSION, AND MAY BE FATAL.

INGESTION: MAY CAUSE IRRITATION WITH POSSIBLE NAUSEA, VOMITING, AND DIARRHEA. MAY CAUSE CENTRAL NERVOUS SYSTEM EFFECTS SIMILAR TO INHALATION. INGESTION AND SUBSEQUENT VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL.

-Chronic (Long Term Exposure)

MAY CAUSE SKIN SENSITIZATION IS SOME INDIVIDUALS. ON RARE OCCASIONS, PROLONGED AND REPEATED EXPOSURE TO OIL MIST POSES A RISK OF CHRONIC LUNG INFLAMMATION. THIS CONDITION IS USUALLY ASYMPTOMATIC AS A RESULT OF REPEATED SMALL ASPIRATIONS. SHORTNESS OF BREATH AND COUGHING ARE THE MOST COMMON SYMPTOMS. ASPIRATION MAY LEAD TO PULMONARY EDEMA AND HEMORRHAGE AND MAY BE FATAL. SIGNS OF LUNG INVOLVEMENT INCLUDE INCREASED RESPIRATION AND HEART RATES AS WELL AS A BLUISH DISCOLORATION OF THE SKIN- CHRONIC SKIN CONTACT MAY PROMOTE DERMATITIS AND OIL ACNE.  IN RARER CASES, AN INCREASED SENSITIVITY TO SUNLIGHT <PHOTOSENSITrVITY) MAY OCCUR. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA, AND DERMATITIS. TARGET ORGANS: LUNGS, LIVER, KIDNEYS, BLADDER, AND CENTRAL NERVOUS SYSTEM.

Primary Routes of Entry:     X <—Inhalation       <—Ingestion      X <—Absorption

Emergency and First Aid Procedures:

-Inhalation: REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

-Eye Contact:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING FOR AT LEAST 15 MINUTES. HOLD THE EYELIDS APART TO ENSURE RINSING OF THE ENTIRE SURFACE OF THE EYES AND LIDS WITH WATER. GET IMMEDIATE MEDICAL ATTENTION.

-Skin Contact:

WIPE AWAY MATERIAL WITH A CLOTH WHILE REMOVING CONTAMINATED CLOTHING AND SHOES. WASH WITH SOAP AND WATER. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS. CLEAN CLOTHING AND SHOES.

-Ingestion:

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING.  IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

-Notes to Physician:

INGESTION AND SUBSEQUFJ>Tr VOMITING OF THIS PRODUCT CAN LEAD TO ASPIRATION	OF THE PRODUCT INTO THE LUNGS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL.

DEPENDING ON THE AMOUNT INGESTED AND RETAINED AS WELL AS THE TOXICITY OF	THE PRODUCT, GASTRIC LAVAGE SHOULD BE CONSIDERED. KEEP PATIENT'S HEAD BELOW

HIPS TO PRFA7ENT PULMONARY ASPIRATION.  IF COMATOSE, A CUFFED ENDOTRACHAEL	TUBE WILL PREVENT ASPIRATION,

SECTION VI - TOXICITY INFORMATION

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

IARC—> No	NTP—> No	OSHA—> No	ACGIH—> No	OTHER--> No

VOC CONTENT: 15.6% BY WEIGHT; 21.8% BY VOLUME; 131.1 G/L

PETROLATUM SKN-RBT TDLO: 100 ML/KG/30D-I   4.

SODIUM SUI.FONATE ORL-RAT LD50: >5 G/KG   3. SKN-RAT LD50: >2 G/KG   3.

HEXYLKNE GLYCOL

ORL-RAT LD50: 3700 MG/KG	4.

ORL-RAT LDLO: 33600 MG/KG/28D-I	CHANGES IN LIVER, BLADDER, KIDNEY  4.
IHL-RAT LC50: NO DEATHS: 66 PPM/8H 3.

SKN-RBT LD50: 8560 UL/KG	4.

SKN-RBT SDT: 465 MG/24H	4.

EYE-RBT SDT: 93 MG/24H SEVERE	4.

IN A REPEATED PATCH TEST STUDY OF ECSEMA PATIENTS, HEXYLENE GLYCOL ELICITED EDEMA AND ERYTHEMA IN 2.8% OF THE SUBJECTS TESTED (CONTACT DERMATITIS 21:154, 1989)   3.

HEXYL£NE GLYCOL HAS PRODUCED EVIDENCE OF SLIGHT FETAL TOXICITY AT A MATERNALLY TOXIC DOSE (1000 MG/KG/DAY)    3.

MINERAL SPIRITS

IHL-RAT LC: >5500 MG/M3/4H	4.

ORL-RAT LD50: >5 G/KG	4.

SKN-RBT LD50: >3 G/KG	4.

EYE-RBT SDT: 500 MG/24H MODERATE	4.

LIGHT NAPHTHENIC HYDROTREATED PTEROLEUM DISTILLATE ORL-RAT LD50: >5 GM/KG      4. SKN-RBT LD50: >2 GM/KG      4. SKN-RBT SDT: 500 MG SEVERE 4.

HEAVY NAPHTHENIC SEVERELY HYDROTREATED PETROLEUM DISTILLATE ORL-RAT LD50: >5 G/KG       3. SKN-RBT LD50: >3 G/KG       3.

SKN-RBT IRRITATION <0.5/8.0; NO APPRECIABLE EFFECT 3. EYE-RET IRRITATION <15/110; NO APPRECIABLE EFFECT  3.

MINERAL OIL AND HYDROCARBON MISTS DERIVED FROM HIGHLY REFINED OILS AND PETROLEUM DISTILLATES ARE REPORTED TO HAVE LOW ACUTE AND SUB-ACUTE TOXICITIES IN ANIMALS. EFFECTS FROM SINGLE AND SHORT-TERM REPEATED EXPOSURES TO HIGH CONCENTRATIONS WELL ABOVE APPLICABLE WORKPLACE EXPOSURE LEVELS INCLUDE LUNG INFLAMMATORY REACTION, LIPOID GRANULOMA FORMATION, AND LIPOID PNEUMONIA. IN ACUTE AND SUB-ACUTE STUDIES INVOLVING EXPOSURES TO LOWER CONCENTRATIONS AT OR NEAR CURRENT WORK PLACE EXPOSURE LEVELS PRODUCED NO SIGNIFICANT TOXICOLOGICAL EFFECTS.  IN LONG TERM STUDIES (UP TO TWO YEARS) NO CARCINOGENIC EFFECTS HAVE BEEN REPORTED IN ANY ANIMAL SPECIES TESTED. THESE PETROLEUM DISTILLATES ARE SEVERELY HYDROTREATED, SEVERELY SOLVENT EXTRACTED, AND/OR PROCESSED BY MILD HYDROTREATMENT AND EXTRACTION. FOR THIS REASON, THEY ARE NOT CLASSIFIED AS CANCER HAZARDS. 3.

PROPANE IHL-LC50 >40% BY VOLUME 3.

N-BUTANE IHL-RAT LC50: 658 G/M3/4H  4.

HUMAN VOLUNTEERS EXPOSED REPEATEDLY TO GASES OF SIMILAR HYDROCARON MIXTURES RANGING FROM 250 TO 1000 PPM EXHIBITED NO CARDIAC OR PULMONARY FUNCTION

MATERIAL SAFETY DATA SHEET: X-433 AEROSOL	Page: 3

SECTION VI - TOXICITY INFORMATION (Continued)

ABNORMALITIES-  3.

POLYBUTENE   COPOLYMER ORL-RAT   LD50:    >34600  MG/KG        3. JJSM-RAT   LD50:    XL0250  MS/KS        3.

-RBT:   1-5/8.0,    SLIGHT   IRRITANT         3.

-RET:   <8.0/110,    COMPLETE  DISAPPEARANCE   OF  EFFECTS  WITHIN   72   HOURS         3.

SECTION VII - REACTIVITY DATA

Stability:         X <—Stable      <—Unstable

Conditions to Avoid:

AVOID HEAT, HOT SURFACES, SPARKS, AND OPEN FLAMES.

Incompatibility (Materials to Avoid) :

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE, DICHROMATES, AND PERMANGANATES; STRONG ACIDS, ALDEHYDES, KETONES

ACID CLAYS.

Hazardous Decomposition Products:

OXIDES OF CARBON, NITROGEN, SODIUM, AND SULFUR; ALDEHYDES AND KETONES.

Hazardous Polymerization:	<--May Occur      X <--Will Not Occur

Conditions to Avoid:

N/A

SECTION VIII - SPILL OR LEAK PROCEDURES

Steps to be Taken if Material is Released or Spilled:

DUE TO THE NATURE OF THE AEROSOL PACKAGING, A LARGE 3PILL IS UNLIKELY. FOR A SMALL SPILL, WEAR APPROPRIATE PROTECTIVE CLOTHING, VENTILATE THE AREA,

ABSORB WITH AN INERT MATERIAL, AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL- USE CARE AS SPILLS MAY BE SLIPPERY.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS. TYPICAL DISPOSAL IS TO WRAP THE EMPTY AEROSOL CONTAINER IN SEVERAL LAYERS C

NEWSPAPER AND DISPOSE OF IN THE TRASH- AEROSOL RECYCLING PROGRAMS ARE AVAILABLE IN MANY AREAS- DO NOT PUNCTURE OR INCINERATE THIS CONTAINER-

Neutralizing Agent: N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

ji^-kiired Ventilatio , VENTILATION I

S RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE EXCESSIVE LEVELS OF VAPORS OR MISTS. LOCAL VENTILATION IS , BECAUSE iT PUEVENTS DISPERSION IWTO WORK. AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S

RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (288.2-1992). FOR CONCENTRATIONS ABOVE THE TLV AND/OR PEL BUT

LESS THAN 10 TIMES THESE LIMITS, A NIOSH APPROVED HALF-FACEPIECE RESPIRATOR EQUIPPED WITH APPROPRIATE CHEMICAL CARTRIDGES MAY BE USED. FOR

CONCENTRATIONS GREATER THAN 10 TIMES THE TLV AND/OR PEL, CONSULT THE NIOSH RESPIRATOR DECISION LOGIC FOUND IN PUBLICATION NO. 37-116 OR ANSI

S88.2-1992 .

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES SHOULD BE WORN, ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE)  STANDARD FOR HAND PROTECTION, 29

CFR 1910.138.

Eye Protection:

CHEMICAL GOGGLES SHOULD BE WORN. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE PROTECTION, 29 CFR

1910.133.

Other Protection:

WEAR PROTECTIVE CLOTHING WHEN HANDLING, REMOVE OIL SOAKED CLOTHING AND SHOES. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE. A SAFETY SHOWER AND AN

EYEWASH STATION SHOULD BE AVAILABLE.

SECTION X - STORAGE AND HANDLING INFORMATION

Storage Temperature:     Indoors--> X	Outdoors-->	Heated-->	Refrigerated-->

Minimum Temperature: 35°F    Maximum Temperature: 120°F

Precautions to be Taken in Handling and Storing:

USE WITH CAUTION AROUND HEAT, SPARKS, PILOT LIGHTS, STATIC ELECTRICITY, AND OPEN FLAME.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

SECTION XI - REGULATORY INFORMATION

Chemical Name                                                                  CAS Nurgber	Upper % Limit

None		 ~

ingredients listed above are subject to the reporting requirements of 313 of Title III of" the Superfund Amendments and Reauthorization Act of ^  -. and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. Tnis MSDS is not intended for users in the state of California.

MATERIAL SAFETY DATA SHEET: X-433 AEROSOL	Pa9e: 4

SECTION XII - REFERENCES (Continued)

- THRESHOLD LIMIT VALUES FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS
AND BIOLOGICAL EXPOSURE INDICES, ACGIH, 2004.

- OSHA PEL.

- VENDOR' S MSDS .

4.	REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFOWeb, 2004.	'•"^S
ALL THE COMPONENTS OF THIS PRODUCT ARE IN COMPLIANCE WITH THE TOXIC SUBSTANCES CONTROL ACT (TSCA) AND ARE EITHER LISTED ON THE TSCA INVENTORY OR

OTHERWISE EXEMPTED FROM LISTING.

IRR: IRRITANT, FLAM/FLAMM: FLAMMABLE, COMB: COMBUSTIBLE,

CORK: CORROSIVE, CARC: CARCINOGENIC, TOX:TQXIC, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC: CLEVELAND OPEN CUP, PMCC: PEN SKY-MART IN CLOSED COP, TCC:TAGLIABUE CLOSED CUP, LEL: LOWER EXPLOSION LIMIT, UEL:UPPER EXPLOSION LIMIT, NFPArNATIONAL FIRE PROTECTION ASSOCIATION, IARC: INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTPiNATIONAL TOXICOLOGY PROGRAM, OSHA: OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION, ACGIH: AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLV: THRESHOLD LIMIT VALUE, PEL: PERMISSIBLE EXPOSURE LIMIT, STEL: SHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX:ASPHYXIANT, PNOS:PARTICLES (INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT:STANDARD DRAIZE TEST, ORL:ORAL, IHL: INHALATION, HMN:HUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION.   HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THESE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

CERTIFIED LABS, DIV. OF NCH CORP. assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label.  Users assume all risks associated with such, unreconnnended use, storage, or disposal of the product.

^

^