MATERIAL SAFETY DATA SHEET: (4O4679-0476@-UO136644-5496

T-LUBE PLUS AEROSOL

-DATE OF ISSUE O7/18/97

SECTION I - GENERAL INFORMATION

PAGE :   1

SUPERSEDES 05/09/96

CHEMICAL NAME & SYNONYMS	TRADE NAME & SYNONYMS

N/A	T-LUBE PLUS AEROSOL

CHEMICAL FAMILY :	FORMULA

AEROSOL TELOMER LUBRICANT	X <--MIXTURE

MANUFACTURE'S NAME:

CERTIFIED LABS. DIV. OF NCH CORP.

ADDRESS (NUMBER. STREET, CITY, STATE & ZIP CODE)

BOX 15217O

IRVING.	TEXAS

75O15

PREPARED BY:	*• PRODUCT CODE NUMBER  ~ EMERGENCY TELEPHONE NUMBER

G ZIMMERMAN/CHEMIST	5496	972-438-1381

SECTION II - HAZARDOUS INGREDIENTS THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS

CHEMICAL NAME (INGREDIENTS)  :

POLY TFE. OMEGA-HYDRO-ALPHA-(METHYLCYCLOHEXYL)

HAZARD	>IRRITANT   ~  TLV	>NOT EST.  1 "  PEL	>NOT EST. 2 "

STEL(TWA)*->NOT EST.       CAS*-->6553O-85-O "

PROPANE

HAZARD	>FLAM/ASPHX "  TLV	>NOT EST.  1 *  PEL	>1OOOPPM  2 ~

STEL(TWA)*->NOT EST.       CAS*-->74-98-6

ISOBUTANE

HAZARD	>FLAMM/IRR  "  TLV	>NOT EST.  1 ~  PEL	>NOT EST. 2 "

STEL(TWA)*->NOT EST.       CAStf-->75-28-5

N-BUTANE

HAZARD	>FLAMMABLE  "  TLV	>8OO PPM  1 "  PEL	>8OO PPM  2 *

STEL(TWA)*->NOT EST.    ~  CAS«-->1O6-97-8

ISOPROPYL ALCOHOL

HAZARD	>FLAM/IRR    ~  TLV	>40O PPM  1 "  PEL	>4OO PPM  2 "

STEL(TWA)*->5OO PPM     ~  CAS»-->67-63-0

MATERIAL SAFETY DATA SHEET:    T-LUBE PLUS AEROSOL	PAGE

(CONTINUED) - SECTION II - HAZARDOUS INGREDIENTS

BOILING PT.  (F)

SECTION III - PHYSICAL DATA
18O	~ SPEC. GRAVITY (H2O=1)'

0.99

VAPOR PR.  (MM	HG) "	35O

NEGLIGIBLE NON-VISCOUS

VAPOR DENSITY	"	4.8

PH. » 1OO%	"	4-6

% VOLATILE BY	VOL "	SO

H20 SOLUBILITY VISCOSITY

~ COLOR

~ ODOR

- CLARITY

EVAPORATION RATE ( BU A/C = 1 )

- WHITE

- ALCOHOL
"	HAZY

~	< 1

SECTION IV - FIRE AND EXPLOSION HAZARD

FLASH POINT: 53  F	- FLAMMABLE LIMITS	'   LEL     ~   UEL

T.C.C.	-ISOPROPYL ALCOHOL	~2.O        "12.7

EXTINGUISHING MEDIA X <--FOAM

"ALCOHOL" <--FOAM

DRY <--CHEMICAL

WATER <--SPRAY

X <--C02

<--OTHER

r

SPECIAL FIRE FIGHTING PROCEDURES

WEAR SELF CONTAINED BREATHING APAPRATUS WHEN FIGHTING CHEMICAL FIRES.

USE WATER SPRAY TO COOL EXPOSED CONTAINERS.

UNUSUAL FIRE AND EXPLOSION HAZARDS

VAPOR FORMS EXPLOSIVE MIXTURE WITH AIR.   HEATING MAY RELEASE VAPORS WHICH CAN BE IGNITED.   VAPORS MAY TRAVEL TO IGNITION SOURCES AND FLASH BACK. FLAME EXTENSION = 24 INCHES.     BURNBACK = 6 INCHES.

NFPA HAZARD RATING  (0=INSIGNIFI CANT: 1=SLIGHT;2 = MODERATE:3 = HIGH;4 = EXTREME) 2 <--HEALTH   4 <--FLAMMABILITY   0 <-- REACT I V ITY     <--SPECIAL

MATERIAL SAFETY DATA SHEET:    T-LUBE PLUS AEROSOL	PAGE :  3

S

E

C

TION V - HEALTH HAZARD DATA THRESHOLD LIMIT VALUE: NOT ESTABLISHED FOR MIXTURE.  SEE SECTION II.

EFFECTS OF OVEREXPOSURE :

-	ACUTE -  (SHORT TERM EXPOSURE)

EYE: CAUSES TEARING, BLURRING OF VISION AND CORNEAL INJURY. SKIN: MAY CAUSE IRRITATION, DRYNESS OR RASH.

INHALATION: MAY CAUSE IRRITATION TO THE NOSE, THROAT, RESPIRATORY TRACT. SYMPTOMS MAY INCLUDE HEADACHE, DROWSINESS, AND COUGHING.  INHALATION OF HIGH CONCENTRATIONS MAY LEAD TO CENTRAL NERVOUS SYSTEM DEPRESSION WITH ANESTHETIC EFFECTS SUCH AS DIZZINESS, CONFUSION.  INCOORDINATION AND LOSS OF CONSCIOUSNESS.

INGESTION: LARGE QUANTITIES MAY CAUSE DROWSINESS, LOSS OF CONSCIOUSNESS, CRAMPS, PAIN, NAUSEA, VOMITING, DIARRHEA AND ABDOMINAL PAIN.

-	CHRONIC -   (LONG TERM EXPOSURE)

NO TOXICITY DATA AVAILABLE FOR CHRONIC HEALTH HAZARDS. TARGET ORGANS:  CENTRAL NERVOUS SYSTEM.

NOTE: INHALATION OF DECOMPOSITION PRODUCTS MAY CAUSE POLYMER FUME FEVER: OR LUNG IRRITATION REQUIRING MEDICAL TREATMENT FOR FLUORINE COMPOUNDS WHICH CAN CAUSE DELAYED PULMONARY EDEMA.   POLYMER FUME FEVER IS A FLU-LIKE ILLNESS WITH FEVER, CHILLS, AND SOMETIMES COUGH, WHICH OCCURS SEVERAL HOURS AFTER EXPOSURE AND SUBSIDES WITHIN 24 HOURS EVEN IN THE ABSENCE OF TREATMENT.   POLYMER FUME FEVER DOES NOT CAUSE PERMANENT INJURY AND THE EFFECTS ARE NOT CUMULATIVE.

PRIMARY ROUTE OF ENTRY: X<-- INHALATION  <-- INGESTION    <-.- ABSORPTION

EMERGENCY & FIRST AID PROCEDURES

INHALATION :

REMOVE FROM THE AREA TO FRESH AIR. SEEK MEDICAL ATTENTION IF RESPIRATORY IRRITATION DEVELOPS OR IF BREATHING BECOMES DIFFICULT.

EYE CONTACT:

RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING WITH PLENTY OF WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

SKIN CONTACT:

WASH AFFECTED AREAS WITH PLENTY OF SOAP AND WATER FOR SEVERAL MINUTES. SEEK MEDICAL ATTENTION IF IRRITATION DEVELOPS.

MATERIAL SAFETY DATA SHEET:    T-LUBE PLUS AEROSOL	PAGE :  4

(CONTINUED) - SECTION V - HEALTH HAZARD DATA

INGESTION :

GIVE 3 TO 4 GLASSES OF WATER, BUT DO NOT INDUCE VOMITING.  IF VOMITING OCCURS, GIVE FLUIDS AGAIN. SEEK MEDICAL ATTENTION IF DISCOMFORT OCCURS.

NOTES TO PHYSICIAN :

ACTIVATED CHARCOAL SLURRY MAY BE ADMINISTERED.   TO PREPARE ACTIVATED CHARCOAL SLURRY. SUSPEND 5O GRAMS ACTIVATED CHARCOAL IN 4OOML OF WATER AND MIX THOROUGHLY.  ADMINISTER 5ML/KG. OR 35OML FOR AN AVERAGE ADULT.

SECTION VI - TOXICITY INFORMATION

PRODUCT CONTAINS CHEMICAL LISTED AS CARCINOGEN OR POTENTIAL CARCINOGEN BY:

IARC   <--YES   NTP   <--YES  OSHA  <--YES  ACGIH   <--YES  OTHER   <--YES X<--NO     NTP  X<--NO   OSHA X<--NO   ACGIH  X<--NO   OTHER  X<--NO

ISOPROPYL ALCOHOL

SKN-RBT  5OO MG/KG MLD       3.

IPR-GPG LD50: 2S6O MG/KG     3.

IHL-MUS LCLO:  128OO PPM/3H  3.

ORL-RAT LD50: 47OO MG/KG    3. POLY TFE, OMEGA-HYDRO-ALPHA-(METHYLCYCLOHEXYL)

ORL-ALD*: RAT >17OOO MG/KG   4. PROPANE

NO DATA AVAILABLE N-BUTANE

NO DATA AVAILABLE ISOBUTANE

IHL-RAT LC5O: 658 G/MG/4H    3.

IHL-MUS LCSO: 68O G/M3/2H    3.

SECTION VII - REACTIVITY DATA <--STABLE    <--UNSTABLE "  CONDITIONS TO AVOID

STABILITY

HEAT, SPARKS. OPEN FLAME AND OTHER IGNITION SOURCES.

INCOMPATABILITY  (MATERIALS TO AVOID)  :

CONCENTRATED NITRIC AND SULFURIC ACIDS. STRONG OXIDIZERS, HALOGEN

COMPOUNDS. ALDEHYDES, HALOGENS, AMINES, AMMONIA AND ALUMINUM.

HAZARDOUS DECOMPOSITION PRODUCTS

CARBON DIOXIDE, CARBON MONOXIDE, HYDROGEN FLUORIDE AND HYDROGEN

CHLORIDE.

WILL NOT         MAY   ~  CONDITIONS TO AVOID
HAZARDOUS    ~ X <--OCCUR       <--OCCUR
POLYMERIZATION-^

SECTION VIII - SPILL OR LEAK PROCEDURES

STEPS TO BE TAKEN IF MATERIAL IS RELEASED OR SPILLED:

AEROSOL CONTAINER THEREFORE SPILL IS UNLIKELY.   IF HOWEVER SPILL DOES OCCUR USE INERT ABSORBANT MATERIAL TO CONTAIN AND COLLECT SPILL AND PLACE INTO LABELED CONTAINERS FOR PROPER DISPOSAL.

WASTE DISPOSAL METHOD(S):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE AND LOCAL REGULATIONS.

WRAP EMPTY CANS IN PAPER AND DISPOSE OF IN TRASH.

NEUTRALIZING AGENT : N/A

SECTION IX - SPECIAL PROTECTION INFORMATION

REQUIRED VENTILATION :

USE VENTILATION THAT IS ADEQUATE TO KEEP EMPLOYEE EXPOSURE TO AIRBORNE CONCENTRATIONS BELOW EXPOSURE LIMITS.

RESPIRATORY PROTECTION :

TYPICAL USE OF PRODUCT DOES NOT REQUIRE THE USE OF A RESPIRATOR    IF PRODUCT IS MISTED OR USED IN CONFINED OR POORLY VENTILATED AREAS WEAR A NIOSH APPROVED RESPIRATOR.

GLOVE PROTECTION :

NEOPRENE RUBBER GLOVES IF REPEATED OR

PROLONGED SKIN CONTACT IS LIKELY.

EYE PROTECTION :

SAFETY GLASSES IF METHOD OF USE PRESENTS THE

LIKELIHOOD OF EYE CONTACT.

OTHER PROTECTION : SPLASH PROOF APRON.

SECTION X - STORAGE AND HANDLING INFORMATION

STORAGE TEMPERATURE MAX: 8O F.    WIN:  32 F.

INDOOR X

HEATED

REFRIGERATED

OUTDOOR

PRECAUTIONS TO BE TAKEN IN HANDLING & STORING DO NOT USE AROUND TOBACCO PRODUCTS.  INHALATION OF VAPORS IN THE PRESENCE OF TOBACCO PRODUCTS CAUSES POLYMER FUME FEVER.

OTHER PRECAUTIONS

READ THE ENTIRE LABEL BEFORE USING THIS PRODUCT.

KEEP OUT OF REACH OF CHILDREN.

SECTION XI - REGULATORY INFORMATION

C.A.S NUMBER       UPPER % LIMIT

MATERIAL SAFETY DATA SHEET:    T-LUBE PLUS AEROSOL

(CONTINUED) - SECTION XI - REGULATORY INFORMATION

THOSE INGREDIENTS LISTED ABOVE ARE SUBJECT TO THE REPORTING REOUIRMENTS OF 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT OF 1986 AND 40 CFR PART 372.

IF UE (USE EXEMPTION) APPEARS UNDER UPPER % LIMIT. END USERS ARE EXEMPT FROM NOTIFICATION BECAUSE THE PRODUCT IS USED AND LABELED FOR ROUTINE JANITORIAL WORK, OR THE PRODUCT IS USED AND LABELED FOR FACILITY GROUNDS MAINTENANCE (SUCH AS FERTILIZERS AND HERBICIDES), OR THE PRODUCT IS USED AND LABELED FOR MAINTAINING MOTOR VEHICLES.

THIS MSDS IS NOT SUITABLE FOR USERS IN THE STATE OF CALIFORNIA.

SECTION XII - TRANSPORTATION  * (FOR FUTURE USE)

LABELS	* LIMITED QTY

UNIT CONTAINER

DOT SPS CONTAINER	~ NET EXPLOSIVE WT .

AEROSOL PROPELLANT(S)

SECTION XIII - REFERENCES

- THRESHOLD LIMIT VALUE FOR CHEMICAL SUBSTANCES AND PHYSICAL AGENTS
AND BIOLOGICAL EXPOSURE INDICES,  199O-1991, ACGIH.

- OSHA PEL.

- DANGEROUS PORPERTIES OF INDUSTRIAL MATERIALS, 7TH EDITION.
N.  IRVING SAX AND RICHARD J. LEWIS, SR .

- VENDOR'S MATERIAL SAFETY DATA SHEET.

* SHORT TERM EXPOSURE LIMIT (TWA) LISTED AS FINAL RULE LIMITS PUBLISHED IN THE FEDERAL REGISTER/VOL. 54 NO.  12.  1-19-89

THE INFORMATION CONTAINED HERIN IS BASED ON DATA CONSIDERED ACCURATE IN

MATERIAL SAFETY DATA SHEET:    T-LUBE PLUS AEROSOL	PAGE :  8

(CONTINUED) - SECTION XIII - REFERENCES

LIGHT OF CURRENT FORMULATION.  HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THIS DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

CERTIFIED LABS, DIV. OF NCH CORP. ASSUMES NO RESPONSIB ILTY FOR PERSONAL

INJURY OR PROPERTY DAMAGE CAUSED BY THE USE, STORAGE

OR DISPOSAL OF THE PRODUCT IN A MANNER NOT RECOMMENDED ON THE

PRODUCT LABEL.  USERS ASSUME ALL RISKS ASSOCIATED WITH SUCH

UNRECOMMENDED USE. STORAGE. OR DISPOSAL OF THE PRODUCT.