Material Safety Data Sheet

INGESTION : Ingestion may cause severe irritation to the mucous membranes and stomach. EYE CONTACT : Contact with eyes may cause severe irritation. SKIN CONTACT : Contact with skin may cause severe irritation.

SECTION VII FIRST AID INHALATION: Remove subject to fresh air.

INGESTION: Drink a large amount of water. If discomfort persists, seek medical attention. EYE CONTACT: Immediately flush eyes with a large amount of water for at least 1 5 minutes. Washing eyes within one minute is essential to achieve maximum effectiveness. Seek medical attention immediately. SKIN CONTACT: Flush contacted area with water.

SECTION VIM EMPLOYEE PROTECTION

VENTILATION REQUIREMENTS : Mechanical (General) - Acceptable RESPIRATORY PROTECTION : Not required under normal handling conditions.

PROTECTIVE CLOTHING : When handling, wear full protective clothing including goggles with face shield, rubber gloves, boots and apron.

SECTION IX - SPILL AND DISPOSAL DATA

SPILL: Confine spill. Dilute with water and flush to waste with excess water. WASTE DISPOSAL : Dispose of in accordance with local, state, and federal regulations for liquid and solid wastes.

SECTION X - TRANSPORTATION DATA

PROPER SHIPPING NAME : SODIUM HYDROXIDE SOLUTION, UN 1824 HAZARD LABEL : CORROSIVE PACKING GROUP : II

SECTION XI - OTHER REGULATORY INFORMATION HMIS HEALTH : 2 HMIS FLAMMABILITY : 0 HMIS REACTIVITY : 0 HMIS PERSONAL PROTECTION : X

SECTION XII - ADDITIONAL PRECAUTIONARY INFORMATION

Do not transfer to aluminum or galvanized containers.	Protect containers from extreme temperature

conditions.         When in storage, be sure this label is clearly visible.        Do not get in eyes, on skin or on clothing. Keep out of reach of children.       Handle with care. Avoid eye and skin contact.

MSDS 3393 PREPARED BY : Kiran B. Jain, Ph.D

Page 2 of 2

•rgANCO

Anderson Chemical Company, Inc.

Material Safety Data Sheet

Anderson Chemical Company, Inc.

P.O. Box 4507	MACON, GA   31208

Ph : (478) 803-6100	Fax : (478) 803-6170

CHEMTREC:	(800) 424-9300

DATE PREPARED: REVISION DATE: ISSUE DATE: PRODUCT NUMBER:

1/3/1995

1/7/2005

5/23/2005

3393

PRODUCT NAME: CHEMICAL NAME :

CHEMICAL FAMILY FORMULA :

SECTION I - IDENTIFICATION

ANCOOL 3393

Aqueous Solution: Organophosphonates; Triazole; Synthetic Polymers; Sodium

Molybdate

Scale and Corrosion Inhibitor

Proprietary formulation

SECTION II - HAZARDOUS INGREDIENTS

CAS NUMBER    HAZARD DATA

MATERIAL:

| Molybdenum (soluble compound) Sodium Hydroxide | 7631950 1310732 | TWA/TLV = 5 mg/m3 TWA/TLV = 2 mg/m3 (ceiling limit) OSHA/ACGIH LD50 = 240 mg/kg (rat) |
| --- | --- | --- |

This product contains no known or suspected carcinogens.

pH:	12.0-13.5

SECTION III - PHYSICAL DATA

VAPOR PRESSURE: APPEARANCE: VAPOR DENSITY(AIR=1): ODOR: Like Water

SPECIFIC GRAVITY:	1.05-1.07

BOILING POINT:	212°F

SOLUBILITY IN WATER:      Complete EVAPORATION RATE (_=1): VOLATILITY INCLUDING WATER :       NE

Like Water Light Yellow Liquid Like Water Slight Aromatic

SECTION IV - FIRE AND EXPLOSION DATA FLASH POINT : NONE

FLAMMABLE LIMITS AIR (% by Volume) : NA AUTOIGNITION TEMPERATURE : Non-Flammable EXTINGUISHING MEDIA : NA

FIRE FIGHTING PROCEDURES: Material will not burn. If involved in fires, thermal decomposition may yield oxides of nitrogen (NOX) and carbon. Firefighters should wear full protective clothing and use self contained breathing apparatus. UNUSUAL FIRE AND EXPLOSION HAZARDS : None.

SECTION V - REACTIVITY DATA STABILITY : Stable CONDITIONS TO AVOID : NA HAZARDOUS POLYMERIZATION : Does Not Occur INCOMPATIBILITY : No chemical incompatibilities.

HAZARDOUS DECOMPOSITION PRODUCTS : Thermal decomposition may yield oxides of nitrogen (NOX) and carbon.

SECTION VI - HEALTH DATA THRESHOLD LIMIT VALUE : No TLV established MEDICAL CONDITION AGGRAVATED :

INHALATION : Inhalation may cause severe irritation to the mucous membranes.

Page 1 of 2