MSDS#: 9686 Status: Current

Status Date: 4/16/01 Product Name: C.P. LANCE Internal MSDS#: 835

Current Company Information:

U.N.X. INCORPORATED

707 ARLINGTON BLVD.

GREENVILLE, NC USA 27858

Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

Current Company Short Name:

Manufacturer/Supplier: U.N.X. INCORPORATED 707 E ARLINGTON BLVD GREENVILLE, NC USA 27858 Phone: 252-756-8616     Emergency: 800-255-3924 CHEMTEL

MSDS Prepared: 12/10/98 MSDS Revised: 4/15/01 Keyword: HOUSEKEEPING Synonym(s): DEGREASER

Physical/Chemical Characteristics

Boiling Point: EQ 99   C Specific Gravity: EQ 1.05 H2O=1 Melt/Freeze Point: BT -25 -20   F

pH:GT 12

Solubility In Water: EXCELLENT

Appearance/Odor: AMBER LIQUID WITH SOLVENT ODOR Physical State: Liquid

FIRE AND EXPLOSION DATA:

Closed Cup Flash Pt.: GT 63   C

******************** DOT INFORMATION:

Label: NON-HAZARDOUS

Proper Shipping Name: NON-HAZARDOUS

Preparers Info.: Amendt, Evans, Kincaid

********************************* COMPOSITION/INFORMATION ON INGREDIENTS

Component Name: POTASSIUM HYDROXIDE Notes: LD(50) ORAL RAT 365mg/kg Percent: BT    3    4 CAS No.: 1310-58-3 Exposure Limits

OSHA PEL: 2 MG/M3

ACGIH TLV: 2 MG/M3

Component Name: 2-BUTOXYETHANOL

Notes: LD(50) ORAL RAT 1746mg/kg; LD(50) DERMAL RABBIT 435mg/ke

Percent: BT    2    3

CAS No.: 111-76-2

Exposure Limits

OSHA PEL: 25 PPM

ACGIH TLV: 25 PPM

Component Name: WATER Percent: BT    75    85 CAS No.: 7732-18-5 Exposure Limits

Component Name: DIPHOSPHORIC ACID, TETRAPOTASSIUM SALT Notes: LD(50) ORAL RAT 2980mg/kg; LD(50) DERMAL RABBIT >7940mg/kg Percent: LT    5 CAS No.: 7320-34-5 Exposure Limits

OSHA PEL: 15 MG/M3

ACGIH TLV: 10MG/M3

Component Name: TS#PSRN57948000-500IP Notes: NJ TRADE SECRET NUMBER Percent: LT    2 Exposure Limits

Component Name: TS#PSRN57948000-5058P Percent: LT    2 Exposure Limits

FIRE AND EXPLOSION DATA

SPECIAL FIRE FIGHTING PROCEDURES: USE NIOSH/MSHA APPROVED SELF CONTAINED

BREATHING APPARATUS WHERE PRODUCT IS INVOLVED IN FIRE.

UNUSUAL FIRE AND EXPLOSION HAZARD: AVOID CONTACT WITH ACIDS AND LAUNDRY

COMPOUNDS. STRONG DECOMPOSITION REACTION CAN RESULT.

AVOID CONTACT WITH ALUMINUM,ZINC,MAGNESIUM,COPPER. EXPLOSIVE HYDROGEN GA

CAN BE RELEASED.

REACTIVITY DATA

VERY STABLE. CONDITION TO AVOID: EXTREME HEAT

INCOMPATIBILITY: SEE UNUSUAL FIRE AND EXPLOSIVE HAZARDS SECTION.

HAZARDOUS POLYMERIZATION WILL NOT OCCUR.

HEALTH HAZARD DATA

ACUTE OR CHRONIC: INHALATION, CAN IRRITATE NOSEJHROAT, AND LUNGS

SKIN CONTACT, CAN CAUSE TISSUE DESTRUCTION.

INGESTION, EXTREMELY HIGH TOXICITY. WILL CAUSE BURNS TO

MOUTH, THROAT.

CARCINOGENICITY: NOT APPLICABLE

SIGNS AND SYMPTOMS OF EXPOSURE: PRODUCTJS DESTRUCTIVE TO TISSUE. SEVERE BURNS CAN BE PRODUCED. CONTACT WITH EYES CAN PRODUCE PERMANENT DAMAGE.

FIRST AID: FOR SKIN, FLUSH THOROUGHLY WITH WATER FOR 15 MINUTES. FOR EYES FLUSH WITH WARM WATER FOR AT LEAST 15 MINUTES. CONTACT PHYSICIAN IF INGESTED,DRINK LARGE VOLUMES OF WATER. INDUCE VOMITING. CALL PHYSICIAN

PRECAUTIONS FOR SAFE HANDLING AND USE

STEPS FOR SPILL OR RELEASE: FOR SMALL SPILL, IMMEDIATELY HOSE DOWN WITH COOL

WATER AND DISPOSE TO DRAIN.FOR LARGE SPILL, DIKE,COLLECT AND ADD SLOWLY TO LA

VOLUME OF COOL WATER WITH CONSTANT STIRRING. THEN SLOWLY NEUTRALIZE TO pH OF

7.0 WITH MURIATIC OR PHOSPHORIC ACID SOLUTIONS. DISPOSE OF DOWN DRAIN AFTER

APPROVAL OF APPROPRIATE AUTHORITY. CLEAN AND NEUTRALIZE ALL AREAS OF

CONTACT.

WASTE DISPOSAL METHODS: SEE STEPS FOR SPILL OR RELEASE.

HANDLING AND STORING: EMPTY DRUMS MUST BE TREATED AS HAZARDOUS BECAUSE OF

RESIDUES. STORE IN DRY PLACE. DO NOT STORE WITH STRONG ACIDS. KEEP CONTAINER

CLOSED.

CONTROL MEASURES

RESPIRATORY PROTECTION: AVOID BREATHING MIST. IF EXPOSURE LIKELY, USE FILTER

OR PROTECTIVE RESPIRATOR.

MECHANICAL VENTILATION IS ACCEPTABLE.

RUBBER GLOVES SHOULD BE WORN.

SAFETY GOGGLES SHOULD BE WORN.

OTHER PROTECTIVE MEASURES: HAVE EYE BATH AND SAFETY SHOWER CLOSE BY. IF

POTENTIAL FOR SKIN CONTACT, WEAR PROTECTIVE GLOVES AND HAVE AN ACID HAND WASH

NEAR BY(ie. VINEGAR SOLUTION.).

WORK/HYGENIC PRACTICES: WASH HANDS.CHANGE OUT OF CLOTHES AS SOON AS

POSSIBLE. WASH CLOTHES. SHOWER OR BATHE AS SOON AS POSSIBLE.

OTHER INFORMATION

HMIS: HEALTH-1, FLAMMABILITY-0, REACTIVITY-1, SPECIAL-CORR

PERSONAL PROTECTION - B

UNLESS OTHERWISE NOTED, NO COMPONENTS ARE SARA TITLE 3 SECTION 313 40 CFR LISTED MATERIALS

THIS PRODUCT CONTAINS NO PHOSPHATES

NOT MADE WITH VOC'S THAT COULD CAUSE DAMAGE TO THE OZONE LAYER THE INGREDIENTS OF THIS PRODUCT ARE LISTED ON THE TSCA INVENTORY SHIPPING INFORMATION: COMPOUND, CLEANING LIQUID, INORGANIC, BASIC, (POTASSIUM HYDROXIDE), PG II

END OF REPORT.