SELECT SPECIALTY PRODUCTS, INC. P.O. BOX 5572

Cl-^^OTTESVILLE VA 22905-5572 mh _RIAL SAFETY DATA SHEET

REVISED:JULY 15, 1993

DATE PREPARED: JUNE 12, 1992

EMERGENCY TEL'NUMBER: ( 8O4 ) 296-3937

section 1 - identification oe product

trade name:   p1452 gum ex chemical family:  mixture

| SECTION 3 SECTION 2 - | HAZARDOUS INGREDIENTS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NAME | CAS tt | OSHA | PEL | ACGUI Tl | .V |
| PROPANE | 74-98-6 | 1000 | ppm | N .F . |  |
| BUTANE | 1O6-97-8 | 8OO | pprn | BOO pprn |  |

PHYSICAL & CHEMICAL CHARACTERISTICS

BOILING POINT

SPECIFIC GRAVITY

VAPOR PRESSURE (psig 9 70F)

PERCENT VOLATILE BY VOLUME

EVAPORATION RATE (Butyl Acetate = 1)

PH

SOLUBILITY IN WATER APPEARANCE & ODOR

OTHER LIMITS

N/A N/A

-7.1 to + 3.1 . I FAHRENHEIT

0 .56

46

100

FASTER

N/A

NIL

CLEAR COLORLESS LIQUID - SWEET

PETROLEUM ODOR

SECTION 4 - FIRE & EXPLOSION DATA

^.AMMABILITY PER FLAME PROJECTION: FLAMMABLE FL  MABLE LIMITS IN AIR % BY VOLUME:     LOWER     UPPER

1.9        9.5

EXTINGUISHER MEDIA: FOAM, DRY CHEMICAL (B, C)

SPECIAL FIRE FIGHTING PROCEDURES: KEEP CONTAINERS COOL USING WATER SPRAY. USE PROPER EQUIPMENT TO PROTECT PERSONNEL FROM BURSTING CONTAINERS. EXTINGUISH ALL SOURCES OF IGNITION.

UNUSUAL FIRE & EXPLOSION HAZARDS: CONTENTS UNDER PRESSURE. DO NOT EXPOSE TO TEMPERATURES EXCEEDING 120F AS CONTAINERS MAY VENT, RUPTURE OR BURST. VAPORS ARE HEAVIER THAN AIR & MAY TRAVEL ALONG THE GROUND OR MAY BE MOVED BY VENTILATION SYSTEMS AND IGNITED BY PILOT LIGHTS, OTHER. FLAMES, SPARKS, HEATERS, SMOKING/ ELECTRIC MOTORS, STATIC DISCHARGE, OR OTHER IGNITION SOURCES AT LOCATIONS DISTANT FROM MATERIAL HANDLING POINT.

SECTION 5 - PHYSICAL HAZARDS

STABILITY:

CONDITIONS TO AVOID:

HAZARDOUS POLYMERIZATION:

CONDITIONS TO AVOID:

INCOMPATIBLE MATERIALS:

HAZARDOUS DECOMPOSITION PRODUCTS:

STABLE

HIGH HEAT, SPARKS, OPEN FLAME

WILL NOT OCCUR

NONE

NONE	:

CO, C02, VOLATILE HYDROCARBON VAPORS

SECTION 6 - HEALTH HAZARDS •     •

ROUTES OF ENTRY:    INHALATION?         SKIN?	INGESTION?

YES	YES	UNLIKELY

SIGNS & SYMPTOMS OF EXPOSURE (ACUTE, CHRONIC): PRODUCT CAN CAUSE BURNS &

' FROSTBITE ON CONTACT WITH SKIN DUE TO HIGH FREEZING EFFECT. DO NOT SPRAY ON SKIN, HAIR OR EYES. DO NOT INHALE VAPORS. AT HIGH CONCENTRATIONS PRODUCE SIGNS OF ANESTHESIA.

MSDS P1452 GUM EX

PAGE 2

SECTION 6 - HEALTH HAZARDS (CONTINUED)

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE: NONE KNOWN

CHEMICALS LISTED AS CARCINOGEN OR POTENTIAL CARCINOGEN:
NTP?	IARC?	OSHA?

NO	NO	NO

EMERGENCY & FIRST AID PROCEDURES:

INHALATION: REMOVE VICTIM TO FRESH AIR. APPLY ARTIFICIAL RESPIRATION IF •NECESSARY. GET MEDICAL ATTENTION.

EYES: FLUSH EYES WITH WATER FOR AT LEAST 15 MINUTES. GET MEDICAL ATTENTION IMMEDIATELY.

SKIN: FLUSH AREAS WITH PLENTY OF WATER. GET MEDICAL ATTENTION IF THERE IS EVIDENCE OF TISSUE DAMAGE.

INGESTION: DO NOT INDUCE VOMITING. GET MEDICAL ATTENTION IMMEDIATELY

H M I S  RATINGS

N F P A  RATINGS

HEALTH

FLAMMABILITY

REACTIVITY

1 4 0

TOXICITY

FIRE

REACTIVITY

1 4 0

SECTION 7 - SPECIAL PROTECTION INFORMATION

RESPIRATORY PROTECTION:	NONE REQUIRED FOR NORMAL USE.

VENTILATION -       LOCAL EXHAUST:	MAINTAIN ADEQUATE VENTILATION

MECHANICAL (GENERAL:	N/A

SPECIAL:	N/A

OTHER:	N/A  ,

PROTECTIVE GLOVES:	NONE REQUIRED FOR NORMAL USE

EYE PROTECTION:	NONE REQUIRED FOR NORMAL USE

OTHER PROTECTIVE CLOTHING OR EQUIPMENT:	NONE REQUIRED

SECtion 8 - special precautions & spill/leak procedures

PRecautions to be taken - handling & storage: do not store at temperatures above 120f. do not puncture or incinerate containers. keep out of reach of children. store in accordance with nfpa 30b for level 3 aerosols.

OTher precautions: read label:precautions carefully

STeps to be taken in case material is released or spilled: avoid .sources of r ignition. ventilate areawuse^water fog to evaporate or ventilate. 8-

PROTECT BODY AGAINST CONTACT'WITH LIQUID. IF CONFINED SPACE^r-- USE SELF CONTAINED BREATHING APPARATUS. CONSULT LOCAL FIRE AUTHORITIES. >JE DISPOSAL METHODS: DOSPOSE OF IN ACCORDANCE WITH LOCAL, STATE & FEDERAL REGULATIONS.

0

DISCLAIMER: SELECT SPECIALTY PRODUCTS BELIEVES THAT THE INFORMATION- CON-
TAINED ON THIS MATERIAL SAFETY' DATA SHEET IS ACCURATE.  THE SUGGESTED PRO-
CEDURES ARE BASED ON EXPERIENCE AS OF DATE OF PUBLICATION.  THEY ARE NOT
NECESSARILY ALL INCLUSIVE NQR ARE THEY FULLY ADEQUATE IN EVERY CIRCUMSTANCE.
ADDITION, THE SUGGESTIONS .ON .THIS MATERIAL SAFETY DATA SHEET SHOULD NOT
CONFUSED WITH, NOR SHOULD- THEY BE FOLLOWED IN VIOLATION OF, APPLICABLE
LAWS, REGULATIONS OR INSURANCE REQUIREMENTS.