AP9084

flinn scientific, inc.

Material Safety Data Sheet (MSDS)		MSDS#: 7350°

Revision DateL April 2,2001

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Sodium Hydroxide Solution

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 — COMPOSITION, INFORMATION ON INGREDIENTS

Sodium Hydroxide (1310-73-2) 4-24%, and Water (7732-18-5) 76-96%

6M, 3M, 1M

CAS#: None Established

SECTION 3 — HAZARDS IDENTIFICATION

Colorless, odorless liquid.	flinn AT-A-GLANCE

Moderately toxic by ingestion and skin absorption. Corrosive to body tissues. Causes severe eye burns.      Health-2
Avoid all body tissue contact.	Flammability-0

Reactivity-2 Exposure-2 Storage-0

0 is low hazard, 3 is high hazard

SECTION 4 — FIRST AID MEASURES

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give no more than 1-2 cups of water for dilution. Do not induce vomiting. Call a physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Non combustible liquid.	NFPA CODE
When heated to decomposition, emits toxic fumes of Na2O.	H-3

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear	-    F-0
PPE and SCBA with full facepiece operated in positive mode.	R-l

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Restrict unprotected personnel from area and ventilate area. Contain spill with sand or absorbent material; deposit in sealed bag or container. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Inorganic #4. Store with hydroxides, oxides, silicates and carbonates. Keep container tightly closed.

SECTION 8 — EXPOSURE CONTROLS, PERSONAL PROTECTION

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron.

1999 Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

flinn scientific, inc.

Material Safety Data Sheet     Sodium Hydroxide Solution

MSDS #:     735.00

Revision Date:   April 2,2001

SECTION 9—PHYSICAL AND CHEMICAL PROPERTIES

, odorless liquid.

Concentrations: 6 Molar = 24%, 1 Molar = 4%,

0.5 Molar = 2%, 0.2 Molar = 0.8%, 0.1 Molar = .48% NaOH

SECTION 10 — STABILITY AND REACTIVITY

Shelf life: Good, if kept tightly closed.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: Toxic, corrosive Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. - Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Data not yet available.

SECTION 13 — DISPOSAL CONSIDERATIONS

Please consult with state and local regulations. Flinn Suggested Disposal Method #10 is one option.

SECTION 14 — TRANSPORT INFORMATION

Shipping Name: Sodium hydroxide solution Hazard Class: 8, Corrosive UN Number: UNI824 N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION"

Not listed.

SECTION 16 — OTHER INFORMATION

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY, ^Any use of this data

and information must be determined by the science instructor toj^j; jnjiccj^jncg wjtji^a^^	laws and

regulations, the conditions or methods of handling, storage, use and disposal of the product(s) described are-beyor4d4Re-eoRtrel— of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual for additional information about laboratory chemicals

© 1999 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2