FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies" Material Safety Data Sheet (MSDS)

MSDS #:    296.00

Revision Date:   November 25, 2002

Section 1 — Chemical Product and Company Identification

Dextrose

Flinn Scientific, Inc. P.O. Box 219 Batavia, IL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

Section 2 — Composition, Information on Ingredients

Dextrose

CAS#: 5996-10-1 (monohydrate); 492-62-6 (anhydrous)

Section 3 — Hazards Identification

Colorless to white crystals. Odorless.

Substance not considered hazardous. However, not all health aspects of this substance have been

thoroughly investigated.

flinn at-a-glance

Health-0 Flammability-0 Reactivity-0 Exposure-0 Storage-1

0 is low hazard, 3 is high haz

Section 4 — First Aid Measures

Call a physician, seek medical attention for further treatment, observation and support after first aid. Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately. Eye: Immediately flush with fresh water for 15 minutes. External: Wash with mild soap and water. Internal: Give large quantities of water.

Section 5 — Fire Fighting Measures

Non flammable solid.

NFPA code

None Established

Section 6 — Accidental Release Measures

Restrict unprotected personnel from area. Sweep up, place in sealed bag or container and dispose. Ventilate area and wash spill si after material pickup is complete. See Sections 8 and 13 for further information.

Section 7 — Handling and Storage

Flinn Suggested Chemical Storage Pattern: Organic #2. Store with alcohols, glycols, sugars, amines, amides, imines, and imides. Store in a cool dry place. Store in a Finn Chem-Saf bag.

Section 8 — Exposure Controls . Personal Protection

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron

© 2002 Flinn Scientific, Inc. All Rights Reserved.

page 1 o

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Material Safety Data Sheet (MSDS)

Dextrose

MSDS #:    296.00

Revision Date:   November 25, 2002

Section 9 — Physical and Chemical Properties

Colorless to white crystals. Odorless. Solubility: Soluble in water. Formula: C6H12O6H2O Formula Weight: 198.18

Specific Gravity: 1.544 Melting Point: 83 C

Section 10 — Stability and Reactivity

Shelf Life: Good, if kept dry.

Section 11 —Toxicoloqical Information

Acute effects: N.A. Chronic effects: N.A. Target organs: N.A.

ORL-RAT LD50: N.A. IHL-RAT LC50: N.A. SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

Section 12 — Ecological Information

Data not yet available.

Section 13 — Disposal Considerations

Please consult with state and local regulations. Flinn Suggested Disposal Method #26a is one option.

Section 14 — Transport Information

Shipping Name: Not regulated Hazard Class: N/A UN Number: N/A

N/A = Not applicable

Section 15 — Regulatory Information

EINECS-listed (207-757-8).

Section 16 — Other Information

Consult your copy of the Flinn Scientific Catalog/Reference Manual for additional information about laboratory chemicals. This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. Flinn Scientific Inc. assumes no legal responsibility for use or reliance upon this data.

FLINN SCIENTIFIC INC.

"Your Safer Source for Science Supplies"

Improve Safety—Use Flinn Chemicals

flinn@flinnsci.com   www.flinnsci.com

P.O. Box 219   Batavia IL 60510

(800) 452-1261 Fax (866) 452-1436

) 2002 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2