AP9083	1-4

flinn scientific, inc.

Material Safety Data Sheet (MSDS)	MSDS #: 592 °°

Revision Date: June 29,2004

SECTION 1 — CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Phenolphthalein Solution

Flinn Scientific, Inc. P.O. Box 219 Batavia, EL 60510 (800)452-1261 CHEMTREC Emergency Phone Number: (800) 424-9300

SECTION 2 — COMPOSITION, INFORMATION ON INGREDIENTS

Phenolphthalein (77-09-8) 1%, and Ethyl Alcohol (64-17-5) 99%. CAS#: None Established

SECTION 3 — HAZARDS IDENTIFICATION

Colorless, clear solution in neutral or acid solution. Pink to deep red in the presence of alkali, but	FLINN AT-A-GLANCE

colorless, again, in the presence of a large amount of alkali. Odor of alcohol.	Health-3

Contains denatured ethyl alcohol. Toxic by ingestion and inhalation. Irritating to body tissues. Avoid         Flammability-3
all body tissue contact.	"	Reactivity-1

Flammable liquid.	Exposure-1

Storage-3

0 is low hazard, 3 is high hazard

SECTION 4 — FIRST AID MEASURES

Call a physician, seek medical attention for further treatment, observation and support after first aid.

Inhalation: Remove to fresh air at once. If breathing has stopped give artificial respiration immediately.

Eye: Immediately flush with fresh water for 15 minutes.

External: Wash continuously with fresh water for 15 minutes.

Internal: Give large quantities of water. Call a physician or poison control at once.

SECTION 5 — FIRE FIGHTING MEASURES

Class 1A Flammable liquid.	NFPA CODE
Flash Point: 48 F    Upper: 24.5%    Lower: 3.3%    Autoignition Temperature: 683 F	None

Fire Fighting Instructions: Use triclass, dry chemical fire extinguisher. Firefighters should wear	Established
PPE and SCBA with full facepiece operated in positive pressure mode.

SECTION 6 — ACCIDENTAL RELEASE MEASURES

Restrict unprotected personnel from area. Remove all ignition sources and ventilate area. Contain spill with sand and absorbent material; deposit in sealed bag or container. See Sections 8 and 13 for further information.

SECTION 7 — HANDLING AND STORAGE

Flinn Suggested Chemical Storage Pattern: Organic #9. Store with dyes, indicators, and stains.

Store in a dedicated flammables cabinet. If a fiammables cabinet is not available, store in Flinn Saf-Stor can.

Use and dispense in a hood.

section s — exposure controls, personal protection	~

Avoid contact with eyes, skin and clothing. Wear chemical splash goggles, chemical-resistant gloves and chemical-resistant apron. Use ventilation to keep airborne concentrations below exposure limits. Always wear a NIOSH-approved respirator with proper cartridges or a positive pressure, air-supplied respirator when handling this material in emergency situations (spill or fire). Exposure guidelines: TWA 1000 ppm, as ethyl alcohol (OSHA)

© 1999 Flinn Scientific, Inc. All Rights Reserved.	PAGE 1 OF 2

flinn scientific, inc.

Material Safety Data Sheet

Phenolphthalein Solution

MSDS #:     592.00

Revision Date:   June 29,2004

SECTION 9 — PHYSICAL AND CHEMICAL PROPERTIES

Colorless, clear solution in neutral or acid solution.

SECTION 10 — STABILITY AND REACTIVITY

Avoid contact with oxidizers, peroxides, acids, acid chlorides, acid anhydrides, alkali metals, ammonia, moisture, heat, open flames, or any source of ignition. Shelf life: Indefinite.

SECTION 11 — TOXICOLOGICAL INFORMATION

Acute effects: Irritant, nausea, poison, dizziness, headache Chronic effects: Phenolphthalein is a suspect cancer agent. Target organs: Eyes, liver, kidneys, nerves

ORL-RAT LD50: 7060 mg/kg as ethyl alcohol IHL-RAT LC50: 20000 ppm/lOH ethyl alcohol SKN-RBT LD50: N.A.

N.A. = Not available, not all health aspects of this substance have been fully investigated.

SECTION 12 — ECOLOGICAL INFORMATION

Data not yet available.

SECTION 13 — DISPOSAL CONSIDERATIONS

Please consult with state and local regulations. Flinn Suggested Disposal Method 18b is one option.

SECTION 14 — TRANSPORT INFORMATION

Shipping Name: Ethyl Alcohol Hazard Class: 3, Flammable liquid UN Number: UNI 170 N/A = Not applicable

SECTION 15 — REGULATORY INFORMATION"

Not listed.

SECTION 16 — OTHER INFORMATION

This Material Safety Data Sheet (MSDS) is for guidance and is based upon information and tests believed to be reliable. Flinn Scientific, Inc. makes no guarantee of the accuracy or completeness of the data and shall not be liable for any damages relating thereto. The data is offered solely for your consideration, investigation, and verification. The data should not be confused with local, state, federal or insurance mandates, regulations, or requirements and CONSTITUTE NO WARRANTY. Any use of this data and information must be determined by the science instructor to be in accordance with applicable local, state or federal laws and regulations. The conditions or methods of handling, storage, use and disposal of the product(s) described are beyond the control of Flinn Scientific, Inc. and may be beyond our knowledge. FOR THIS AND OTHER REASONS, WE DO NOT ASSUME RESPONSIBILITY AND EXPRESSLY DISCLAIM LIABILITY FOR LOSS, DAMAGE OR EXPENSE ARISING OUT OF OR IN ANY WAY CONNECTED WITH THE HANDLING, STORAGE, USE OR DISPOSAL OF THIS PRODUCT(S).

Consult your copy of the Flinn Chemical Catalog/Reference Manual for additional information about laboratory chemicals

© 1999 Flinn Scientific, Inc. All Rights Reserved.

page 2 of 2