Performance specialties    336 760-OE45

p.i

21   02   04:35p

North America

Performance

Polymers

491 7 Dawn Avenue

East Lansing, Ml 48823-5691

Ciba

Value beyond chemistry

8am to4:30pm Phone: (517) 351-5900

24-Hour Health/Environmental Emergency Phone: 1-800-873-1138

Effective Date:    S/19/98

Material Safety Data Sheet

MSDSNo: 5900

j. PRODUCT IDENTIFICATION Trade Name: Impact HARDENER Chemical Family: Ainine

Intended Use or Product Type: Adhesive Hardener

NFPA RATING

| HEALTH | 3 |
| --- | --- |
| FLAMMABlLITy | I |
| REACTIVITY | 0 |
| PROTECTIVE EQUIPMENT |  |

HMISRATJNG

2. COMPOSITION / INFORMATION ON INGKEDIENTS

| o s H | CAS# | CHEMICAL IDENTITY | EXPOSURE LIMITS |  |  |  |  | CARCINOGEN STATUS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ACGIH |  | OSHA |  | MFR. | IAR.C | NTT | OSHA |
| A |  |  | TWA STEL |  | PEL STEL |  |  |  |  |  |
| * | 108-95-2 | Phenol | 5ppm | NE | 5 ppm | NE | NE | NR | NR | NR |
| * | 112-24-3 | 1,2-Ethanediamine, N,N'-bis(2-aminoethyl)- | NE | NE | NE | NE | NE | NR | NR | NR |
|  | Common Name; | Trielhylenctetramine |  |  |  |  |  |  |  |  |
| * | 140-31-8 | 1 -Piperazineethanaraine | NE | NE | NE | NE | NE | NR | NR | NR |
|  | Common Name; | Aminoethylpiperazine |  |  |  |  |  |  |  |  |
| * | 25154-52-3 | Phenol, Nonyl- | NE | NE | NE | NE | NE | NR | NR | NR |
|  | Common Name: | Nonyl Phenol | • | _ |  |  |  |  |  |  |
| * | 68410-23-1 | Fatty acids, C18-unsatd,, dimers, reaction products with polyethylenepolyamines | NE | NE | NE | NE | NE | NR | NR | NR |
|  | Common Nome: | Polyamide Resin |  |  |  |  |  |  |  |  |

^

Printed: 3/4/00

Page 1 of 6

21 02 04:35p

Performance Spec

laities    336 760-0245

P-2

Effective Date; 8/19/98

Impact HARDENER

Ciba Specialty Chemicals Corporation Performance. Polymers

| * | 68683-29-4 | Pentanoic Acid, 4,4'-Azobis(4- | NH | NE | NE | NE | NE | NR | NR | NR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cyano-, polymer with 1,3- |  |  |  |  |  |  |  |  |
|  |  | Butadiene, 1-Piperazine |  |  |  |  |  |  |  |  |
|  |  | Ethauamine and 2- |  |  |  |  |  |  |  |  |
|  |  | Propenenitrile) |  |  |  |  |  |  |  |  |
|  | Cotnnion | Synthetic Rubber |  |  |  |  |  |  |  |  |
|  | Name: |  |  |  |  |  |  |  |  |  |
|  | 32610-77-8 | Formaldehyde, polymer with | NE | NE | NE | NE | NE | NR | NR | NR |
|  |  | N,N'-bis(2-ArninoethyI> 1 ,2- |  |  |  |  |  |  |  |  |
|  |  | Ethanediamine and phenol |  |  |  |  |  |  |  |  |
|  | Common | TETA, reaction products with |  |  |  |  |  |  |  |  |
|  | Name: | Phenol/Formaldehyde |  |  |  |  |  |  |  |  |

NE = Not Established    NR = Not Reviewed    * = OSHA Hazardous Ingredient

3. HAZARDS IDENTIFICATION

Emergency Overview; panger! Corrosive - Causes skin and eye burns. May cause allergic skin and respiratory reactions.

Primary Route(s) of Entry: Dermal; heated product may produce inhalable vapors.

Acute Exposure: Danger! Corrosive. Causes skin and eye burns.

4. FIRST Am MEASURES

Ingestion: If conscious, give plenty of water to drink. Do not induce vomiting. Call a physician.

Skin: Wash with soap and water. Remove contaminated clothing and launder before re-use,

Inhalation: Remove to fresh air. Give oxygen and/or artificial respiration, if needed. Call a physician.

Eyes: Immediately flush eyes with water for at least 15 minutes. Call a physician,

Overexposure Effects: Danger! Corrosive. Causes skin and eye burns. Causes allergic skin and respiratory reactions. Absorption through skin may cause nausea, vomiting or other signs of illness.

Medical Conditions Aggravated by Exposure: Skin and eye conditions.

Additional Information: Referral to a physician is recommended if there is any question about the seriousness of any injury.

21 02 04:35P     Performance Specialties

iidb

Impact HARDENER	Ciba Specialty Chemicals Corporation

Effective Date: 8/19/98	Performance Polymers

•i FIRE FIGHTING MEASURES

Flash Point:	>200°F(>93 "C)

Flash Point Method Used:	Estimated

Flammable Limits in Air (Lower - % by volume);	Not established

Flammable Limits in Air (Upper - % by volume):	Not established

Fire Fighting Extinguishing Media: Carbon dioxide, dry chemical, foam, water.

Fire Fighting Equipment: Use self-contained breathing apparatus.

Fire and Explosion Hazards: Decomposition and combustion products may be toxic.

6.	ACCIDENTAL RELEASE MEASURES

Accidental Release Measures: Remove spillage by absorbing in absorbent material. Avoid contact.

7.	HANDLING AND STORAGE

Precautions: Danger! Corrosive. Causes skin and eye burns. May cause allergic skin and respiratory reactions. Do not get in eyes, on skin, on clothing. Avoid breathing vapor or mist. Keep container closed. Use with adequate ventilation. Wash thoroughly after handling.

Other Handling Information: Nuisance dust may be generated when sanding or sawing cured material.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION
Skin Protection: Impermeable gloves and protective clothing.

Respiratory Protection: Use NIOSH approved organic vapor cartridge respirator when vapor/mist exposure is likely-Eye Protection: Full face shield. Engineering Controls: General mechanical and local exhaust in accordance with ACGIH recommendations.

Printed. 5/4/00	Page 3 of 6

Mar 21 02 04:35p

Performance

Specialties

336   760-0245

Effective Date: 8/19/98

Impact HARDENER

Clba Specialty Chemicals Corporation Performance Polymers

9. PHYSICAL AND CHEMICAL PROPERTIES

Color:

Physical State: Solubility in Water: Vapor Pressure: Specific Gravity: Boiling Point: Evaporation Rate: Vapor Density: VOC; pH:

Percent Volatile: Negligible.

Pale Yellow

Paste

Soluble

Not Determined

1.2-1      (water =1)

Not Determined

Not Determined

Not Determined

37.752 g/L

Not Determined

10. STABILITY AND REACTIVITY

Conditions to Avoid: Excessive heat for prolonged periods of time.

Stability: Stable.

Incompatibility: Strong oxidizing agents, acids, metal-organic compounds.

Hazardous Decomposition Products: Combustion may form toxic materials, such as carbon dioxide, carbon monoxide.

Hazardous Polymerization: Will not occur.

n/TOXICOLOGICAL INFORMATION

Sensitization: Possible in susceptible individuals. Skin Irritation: Corrosive. Causes burns, Eye Irritation; Corrosive  Causes bums.

12.JCOLOGICAL INFORMATION

13. DISPOSAL CONSIDERATIONS

Waste Disposal Method: Consult qualified local or corporate personnel for method that will comply with local, slate and federal health and environmental regulations.

Printed; 5/4/00

Page 4 of 6

3

Printed: 5/4/00

Page 2 of

Miir   21   02   04:36p Effective Date: 8/19/98

Performance   Specialties Impact HARDENER

336

Ciba Specialty Chemicals Corporation Performance Polymers

•'14. TRANSPORT INFORMATION

DOT: Non-Bulk Proper Shipping Name: Technical Shipping Name (If n.O.s.): Hazard Class: ID Number:

Packing Group; Label:

Polyamines, liquid, corrosive, N.O.S.

Triethylene tetramine, N-aminoethyl piperazine

S

UN 2735

PGII

Corrosive

.15. REGULATORY INFORMATION

Occupational Safety and Health Act (OSHA): This Material Safety Data Sheet (MSDS) has been prepared in compliance with the federal OSHA Hazard Communication Standard 29 CFR 1910.1200. This produU is considered to be a hazardous chemical under that standard.

Resource Conservation and Recovery Act (RCRA): Not a hazardous waste under RCRA (40 CFR 261) but handle with care due to corrosive effect on skin and eyes.

SARA Title HI: Section 313 * Toxic Chemical List (TCL): This product is (or contains) a toxic chemical for routine annual Toxic Chemical Release Reporting under section 313 (40 CFR 372). This information must be included in all MSDS's copied or distributed for this material.

3.1200%    108-95-2    Phenol

This information must be included in all MSDS's that are copied and distributed for this material. TSCA Section 8(b) - Inventory Status: Chemical components listed on TSCA Inventory.

TSCA Section 12(b) - Export Notification: This product contains the following chemicals that are subject to Section 12(b) export notification:

Chemical Name: Phenol CASRN: 108-95-2

California Proposition 65: The following is required composition information. This product contains the following chemicals which are listed on the California list of Known Carcinogens and reproductive Toxins.

Chemical Name: 2- PROPENENTnULE

CASRN: 107-13-1

Percent in Composition;  Tmcc

Comment: Warning! This chemical is known to the state of California to cause Cancer.

Pennsylvania Right-to-Know: The following is required composition information:

Chemical Name : Pentanoic Acid, 4,4'-A2obis(4-Cyano-, polymer with 1,3-Butadiene, 1-Piperaatie FJhanamine and 2-

Propencnitrilc)

CASRN : 68683-29-4

Common Name ; Synthetic Rubber

Comment : NwL on Pennsylvania Hazardous Substance List

Printed: 5/4/00

Page 5 of 6

Effective Date: 8/19/9S	Performance 'Poly mers

Chemical Name : Limestone CASRN : 1317-65-3 Common Name : Calcium Carbonate Comment : Hazardous Substance.

Chemical Name ; Formaldehyde, polymer with N,N'-bis(2-Aminoethyl)-l ,2-Ethanedtamine and phcuul

CAS11N : 32610-77-8

Common Name : TETA, reaction products with Phenol/Formaldehyde

Comment : Not on Pennsylvania Hazardous Substance List.

Chemical Name : l,2-Ethanediamine>N,N'-bis(2-aminoethyl)-

CASRN : 112-24-3

Common Name : Tricthylenctetramuie

Comment ; Hazardous Substance,

Chemical Nanic : Phenol

CASRN : 108-95-2

Comment : Hazardous Substance and Environmental Hazard.

Chemical Name : 1-Piperazineethanamine CASRN ; 140-31-8

Common Name : Aminoethylpiperazine Comment : Hazardous Substance.

Chemical Name : JPhenol, Nonyl-CASRN : 25154-52-3 Common Name : Nonyl Phenol Comment : Hazardous Substance.

MSDS No:	5900

Approved Uy:	Kenneth L. Payne

Title:	E.H&S Manager

Revision Summary: The following sections have been modified since the last effective date: 1, 2, 3, 7, 15, 16.

Disclaimer: The following supersedes Buyer's documents. SELLER MAKES NO REPRESENTATION OR WARRANTY, EXPRESS OR IMPLIED, INCLUDING OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE. No statements herein arc to be construed as bducemonts to infringe any relevant patent. Under no circumstances shall Seller be liable for incidental, consequential or indirect damages for alleged negligence, breach of warranty, strict liability, tort or contract arising in connection with the produces). Buyer's sole remedy and Seller's sole liability for any claims shall be Buyer's purchase price.   . Data and results are based on controlled or lab work ami must be confirmed by Buyer by testing for its intended conditions of use. The produces) has not been tested for, and is therefore not recommended for, uses for which prolonged contact with mucous membranes, abraded skin, or blood is intended; or for uses for which implantation within the human body is intended.

Printed: 5/4/00	Page 6 of 6