Johnson max

Material Safety Data Sheet

CREW HEAVY DUTY TOILET BOWL CLEANER

National Fire Protection Association (NFPA)

Health

Fire Hazard

Reactivity

Hazardous Material Information System (HMIS)

Specific Hazard

Protective Clothing

Emergency   Blue. Liquid. See Section 9.

Overview      DANGER. CORROSIVE. CAUSES EYE AND SKIN BURNS. HARMFUL OR FATAL IF SWALLOWED.

Section 1. Chemical Product and Company Identification

Product Name

Product Use

CREW HEAVY DUTY TOILET BOWL CLEANER

Industrial/Institutional: Cleaning product.

Code

PMS#

4560

3147964

MSDS#

114560002

Validation Date

6/30/2003

U.S. Headquarters

Johnson Wax Professional

8310 16th Street

Sturtevant, Wisconsin 53177-0902

Phone: (888) 352-2249

MSDS Internet Address:

www.jwp.com

Canadian Headquarters

Johnson Wax Professional

100 Matheson Blvd. East, Suite 203

Mississauga, Ontario L4Z 2G7

Phone: (905) 755-0913 or

(888) 746-5971

Print Date

Supersedes

In Case of Emergency

6/30/2003

6/13/2003.

(800) 851-7145

Section 2. Composition and Information on Ingredients

Ingredients

CAS#

% by Weight

Exposure Limits

LC50/LD50

n-Alkyl Dimethyl Benzyl Ammonium

Chloride

Hydrochloric Acid

68424-85-1 7647-01-0

0.05 23.0

Not available. Not available.

ORAL (LD50): Acute: 426 mg/kg

[Rat].

Not available.

Section 3. Hazards Identification

Routes of Entry

Inhalation. Skin contact. Eye contact.

Potential Acute Health Effects

Eyes Corrosive. May cause permanent damage including blindness.

Skin Corrosive. May cause permanent damage.

Inhalation May cause irritation and corrosive effects to nose, throat and respiratory tract.

Ingestion Corrosive. May cause burns to mouth, throat, and stomach.

Medical Conditions	Individuals with chronic respiratory disorders such as asthma, chronic bronchitis, emphysema, etc., may

Aggravated by Overexposure:be more susceptible to irritating effects.

See Toxicological Information (section 11)

Section 4. First Aid Measures

Eve Contact

Hold eye open and rinse slowly and gently with water for 15-20 minutes. Remove contact lenses, if
present, after the first 5 minutes, then continue rinsing eyes. Call medical attendant, doctor, or poison
control center immediately.

Skin Contact

Take off contaminated clothing. Rinse skin immediately with plenty of water for 15-20 minutes. Call
medical attendant, doctor, or poison control center immediately.

Inhalation

If inhaled, remove to fresh air. If person is not breathing, call 911 or an ambulance and then give artificial
respiration, preferably by mouth to mouth, if possible.
Call medical attendant, doctor, or poison control center immediately.

Ingestion

Call medical attendant, doctor, or poison control center immediately. Have person sip a glass of water if able to swallow. Do not induce vomiting unless told to do so by a poison control center or doctor. Never give anything by mouth to an unconscious person.

Continued on Next Page

WSX	Material Safety Data Sheet

CREW HEAVY DUTY TOILET BOWL CLEANER

'   Section 5. Fire Fighting Measures

Flaminability of the Product   None known.

Flash Points	Closed cup: >93.333°C (200°F).

Products of Combustion	None known.

Fire Fighting Media	Extinguish with water spray or carbon dioxide, dry chemical powder or appropriate foam. Normal fire

and Instructions	fighting procedure may be used.

Protective Clothing (Fire)	Put on appropriate personal protective equipment (see Section 8).

Special Remarks on Fire and	Corrosive material (See sections 8 and 10).

Explosion Hazards

Section 6. Accidental Release Measures

| Personal Precautions Put on appropriate personal protective equipment (see Section 8). |  |
| --- | --- |
| Environmental Precautions In the event of major spillage: Use appropriate containment to avoid environmental contamination. and Clean-up Methods or scrape up material. Place in suitable clean, dry containers for disposal by approved methods water rinse for final clean-up. | Sweep Use a |
| Section 7. Handling and Storage |  |

Handling                                  DANGER: Corrosive. CAUSES EYE AND SKIN BURNS. MAY BE FATAL IF SWALLOWED. Avoid
contact with eyes, skin and clothing. Do not taste or swallow. Avoid breathing vapors or spray mists.
Wash thoroughly after handling. Remove and wash contaminated clothing and footwear before re-use.
Product residue may remain on/in empty containers. All precautions for handling the product must be
	used in handling the empty container and residue. FOR COMMERCIAL AND INDUSTRIAL USE ONLY.

Storage	Store in a dry, cool and well-ventilated area. Protect from freezing. Keep container tightly closed. KEEP

OUT OF REACH OF CHILDREN.

Section 8. Exposure Controls/Personal Protection

Engineering Controls              Good general ventilation should be sufficient to control airborne levels. Respiratory protection is not
	required if good ventilation is maintained.

Personal Protection

Eyes Chemical splash goggles. Face shield.

Hands Chemical resistant gloves. Includes: Neoprene gloves.

Respiratory If mists/vapors are not adequately controlled by ventilation, use appropriate respiratory protection to avoid
over exposure. A respiratory protection program that meets OSHA's 29 CFR 1910.134 and ANSI Z88.2
requirements must be followed whenever workplace conditions warrant a respirator's use.

Feet No special requirements under normal use conditions.

Body If major exposure is possible, wear suitable protective clothing and footwear.

Section 9. Physical and Chemical Properties

Physical State and	Liquid.

Appearance

Odor	Characteristic.

Color	Blue.

pH	<1 [Acidic.]

Specific Gravity	1.1

Solubility in water	Complete.

Section 10. Stability and Reactivity

Stability and Reactivity	The product is stable.

Conditions of Instability	None known.

Incompatibility with Various  Reactive with alkalis.   Chlorine products.   Do not mix with any other chemicals or products unless
Substances	specified by label. Avoid mixing with:Chlorine. Mixing with other cleaning products may produce toxic gas

Hazardous Decomposition      When exposed to fire: May form an acid mist. Produces normal products of combustion. Products

Hazardous Polymerization      Will not occur.

Continued on Next Page

UIHX	Material Safety Data Sheet

CREW HEAVY DUTY TOILET BOWL CLEANER

| J Section 11. lexicological Information |  |  |
| --- | --- | --- |
|  | Acute toxicity Corrosive. |  |
|  | Effects of Chronic Exposure None known. Other Toxic Effects Not available. |  |
|  | Section 12. Ecological Information |  |
|  | Not available. |  |
|  | Section 13. Disposal | Considerations |
|  | Waste Information | Undiluted product is regulated under environmental and transportation laws as a corrosive waste. Dispose of according to all federal, state and local applicable regulations. |
|  | Section 14. Transport Information |  |
|  | DOT Classification DOT Proper Shipping Name | Please refer to the Bill of Lading/receiving documents for up to date shipping information. |
|  | TDG Classification TDG Proper Shipping Name TDG Class | Please refer to the Bill of Lading/receiving documents for up to date shipping information. |
| /^ |  |  |
|  | [ Section 15. Regulatory Information |  |
|  | Reporting in this section I US Regulations Federal State Registered Product Information | is based on ingredients disclosed in Section 2 SARA 313 toxic chemical notification and release reporting: Hydrochloric Acid Clean Water Act (CWA) 311: Hydrochloric Acid Clean air act (CAA) 112 accidental release prevention: Hydrochloric Acid CERCLA: Hazardous substances.: Hydrochloric Acid, New Jersey spill list: Hydrochloric Acid New Jersey: Hydrochloric Acid New Jersey toxic catastrophe prevention act: Hydrochloric Acid Massachusetts RTK: Hydrochloric Acid Pennsylvania RTK: Hydrochloric Acid This product is not subject to the reporting requirements under California's Proposition 65. EPA Registration Number: 8155-3-70627 |
|  | Canadian Regulations Canadian NPRI WHMIS Classification WHMIS Icon Registered Product Information | Canadian NPRI: Hydrochloric Acid. Not controlled under WHMIS (Canada). Exempt Not applicable. |
|  | Chemical Inventory Status | All ingredients of this product are listed or are excluded from listing on the U.S. Toxic Substances Control Act (TSCA) Chemical Substance Inventory |

Continued on Next Page

Material Safety Data Sheet CREW HEAVY DUTY TOILET BOWL CLEANER

Section 16. Other Information

Other Special Considerations

Version	1.04

Notice to Reader

This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained within. Actual conditions of use and handling are beyond seller's control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State, Provincial and Local laws and regulations.

Page: 114