MATERIAL SAFETY DATA SHEET: AQUABROM PLUS

Section I - General Information

(000000-000000-    - 578C)

Date of Issue:

1/20/2004 12:00:00 AM

Chemical Name & Synonyms:

N/A

Chemical Family:

AQUEOUS, ALKALINE HALOGEN SOLUTION

Manufacturer Name:

CHEM-AQUA, INC

Manufacturer Address:

BOX 152170 IRVING, TEXAS 75015

Supercedes:

6/27/2003 12:00:00 AM

Trade Name & Synonyms:

AQUABROM PLUS

Formula is a mixture: [V]

| Prepared By: L Boynton/Chemist | Product Code Number: Emergency Phone Number: 578C 800-424-9300 |  |  |  |
| --- | --- | --- | --- | --- |
| Section II - Hazardous Ingredients |  |  |  |  |
| THE HAZARDS PRESENTED BELOW ARE THOSE OF THE INDIVIDUAL COMPONENTS Chemical Name (Ingredients) Hazard TLV PEL STEL |  |  |  | CAS# |
| BROMINE CHLORIDE SODIUM HYDROXIDE * AS BROMINE; $ AS CEILING LIMIT | IRR/OX CORROSIVE | 0.1 PPM* 1 0.1 PPM* 2 2 MG/M3 3 2 MG/M3 2 | 0.2 PPM* 1 2 MG/M3S 1 | 13863-41-7 1310-73-2 |

Section III - Physical Data

Boiling Point (°F):	214°

Vapor Pressure (mm Hg):	19

Vapor Density (Air=1):	<1

pH@100%:	13.7

% Volatile by Volume:	80-85

H20 Solubility:	COMPLETE

Specific Gravity (H20=1):   1.32-1.37 Color:  YELLOW Odor:   MILDLY SWEET Clarity:  TRANSPARENT Evaporation Rate (BuAc=1):  <1

Viscosity:  NON-VISCOUS

Section IV - Fire and Explosion Hazard

Flash Point: NON-FLAM	Method Used: N/E

Flammable Limits: HYDROGEN GAS	UEL: 75%

LEL: 4%	Aerosol Level (NFPA SOB): N/A

I Extinguishing Media:

[V] Foam	[V] Alcohol Foam         [V] CO2

[V] Dry Chemical         [V] Water Spray	[ ] Other

NFPA 704 Hazard Rating:

4-Extreme 3-High 2-Moderate 1-Slight 0-lnsignificant

Health: 3 Flammability: 0 Instability: 0 Special: OX

Special Fire Fighting Procedures:

FIREFIGHTERS SHOULD WEAR A SELF-CONTAINED BREATHING APPARATUS AND FULL PROTECTIVE GEAR. EXTINGUISHING MEDIA SHOULD BE CHOSEN BASED ON THE NATURE OF THE SURROUNDING FIRE.

Unusual Fire and Explosion Hazards:

USE WATER SPRAY TO COOL FIRE EXPOSED CONTAINERS TO PREVENT BURSTING. CONTACT WITH FINELY DIVIDED METALS SUCH AS ALUMINUM, BRASS, COPPER, IRON, AND STEEL MAY PRODUCE FLAMMABLE HYDROGEN GAS.

of4

MATERIAL SAFETY DATA SHEET: AQUABROM PLUS

Section V - Health and Hazard Data

Threshold Limit Value:

NOT ESTABLISHED FOR MIXTURE. SEE SECTION II.

Effects of Overexposure:

Acute: (Short Term Exposure)

EYE CONTACT: CORROSIVE. CAUSES BURNS, CORNEAL DAMAGE, AND POSSIBLE BLINDNESS. SKIN CONTACT: CORROSIVE. CAUSES BURNS AND POSSIBLE DEEP ULCERATIONS AND SCARRING. PRODUCT MAY BE ABSORBED THROUGH THE SKIN IN HARMFUL AMOUNTS. INHALATION: MIST CAUSES BURNS TO THE RESPIRATORY TRACT, NOSE, MOUTH AND THROAT, WITH DISCOMFORT, NASAL DISCHARGE, SNEEZING, COUGHING, RAPID HEARTBEAT AND CHEST PAIN. INHALATION OF MIST MAY CAUSE CHEMICAL PNEUMONITIS WHICH CAN CAUSE DAMAGE AND MAY BE FATAL. INGESTION: CORROSIVE. CAUSES BURNS TO THE MOUTH, THROAT, ESOPHAGUS, AND STOMACH WITH NAUSEA AND PAIN. SYMPTOMS MAY INCLUDE VOMITING OF BLOOD. BLOOD LOSS THROUGH DAMAGED TISSUE CAN LEAD TO LOW BLOOD PRESSURE AND SHOCK. MAY BE FATAL.

Chronic: (Long Term Exposure)

CHRONIC SKIN CONTACT MAY PROMOTE DERMATITIS. MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE ARE PRE-EXISTING RESPIRATORY AND SKIN CONDITIONS SUCH AS ASTHMA, EMPHYSEMA, AND DERMATITIS. TARGET ORGANS: NONE KNOWN. THE PRIMARY ROUTES OF EXPOSURE ARE SKIN AND EYE CONTACT.

Primary Routes of Entry	-	-	-	-

[] Inhalation	[Jlngestion	[V] Absorption

Emergency First Aid Procedures:

Inhalation:

REMOVE FROM THE AREA TO FRESH AIR. IF NOT BREATHING, CLEAR THE AIRWAY AND START MOUTH TO MOUTH ARTIFICIAL RESPIRATION. GET IMMEDIATE MEDICAL ATTENTION.

Eye Contact:

IMMEDIATELY RINSE THE EYES WITH WATER. REMOVE ANY CONTACT LENSES AND CONTINUE FLUSHING FOR AT LEAST 15 MINUTES.

MEDICAL ATTENTION.

Skin Contact:

WASH AFFECTED AREAS WITH LARGE AMOUNTS OF SOAP AND WATER FOR 15 MINUTES. REMOVE CONTAMINATED CLOTHING AND SHOES. GET IMMEDIATE MEDICAL ATTENTION. WASH CLOTHING AND CLEAN SHOES BEFORE REUSE.

Ingestion:

HAVE THE PERSON SIP A GLASS OF WATER IF ABLE TO DO SO, BUT DO NOT INDUCE VOMITING. IF VOMITING OCCURS, GIVE FLUIDS AGAIN. GET IMMEDIATE MEDICAL ATTENTION. DO NOT GIVE ANYTHING BY MOUTH TO AN UNCONSCIOUS OR CONVULSING PERSON.

Notes to Physician:

PROBABLE MUCOSAL DAMAGE MAY CONTRAINDICATE THE USE OF GASTRIC LAVAGE.

Section VI - Toxicity Information

Product Contains Chemicals Listed as Carcinogen or Potential Carcinogen By:

[ ] IARC	[ J NTP	[ ] OSHA	[ ] ACGIH	[ ] Other

VOC CONTENT: 0% BY WEIGHT; 0% BY VOLUME; 0 G/L

PRODUCT MIXTURE

IHL-RAT LC50: >20.37 MG/L 3.

ORL-RAT LD50: 2491 MG/KG 3.

SKN-RBT LD50: >2000 MG/KG 3.

SKN-RBT SDT: 0.5 ML/4H; DARK DISCOLORATION, SEVERE ERYTHEMA AND EDEMA. AT 24 HOURS, FIVE OUT OF SIX ANIMALS EXHIBITED

EVIDENCE OF DERMAL CORROSION. THE PRIMARY DERMAL IRRITATION INDEX WAS 5.4. 3.

SKN-GPG: NON-SENSITIZING 3.

BROMINE CHLORIDE

NO TOXICITY DATA AVAILABLE

SODIUM HYDROXIDE ORL-MSE LD50: 40 MG/KG 4. ORL-RBT LDLo: 500 MG/KG 4. SKN-RBT TCLo: 25 PPH 4. SKN-RBT SDT: 500 MG/24H SEVERE 4. EYE-RBT SDT: 1 MG/24H SEVERE 4.

2 of 4

MATERIAL SAFETY DATA SHEET: AQUABROM PLUS

Section VII - Reactivity Data

Stability-

[>/] Stable	[] Unstable

Conditions to Avoid:

NONE KNOWN.

Hazardous Polymerization

[V] Will not occur	[ ] May occur

Conditions to Avoid:

N/A

Incompatibility (Materials to Avoid):

STRONG OXIDIZING AGENTS SUCH AS CHLORINE BLEACH AND CONCENTRATED HYDROGEN PEROXIDE; REDUCING AGENTS SUCH AS SODIUM THIOSULFATE; ACIDS, ALCOHOLS, ALDEHYDES, AMMONIA-CONTAINING PRODUCTS, ALKALOIDAL AND HEAVY METAL SALTS, AND CONTACT WITH FINELY DIVIDED METALS SUCH AS ALUMINUM, BRASS, COPPER, IRON, AND STEEL MAY PRODUCE FLAMMABLE HYDROGEN GAS.

Hazardous Decomposition Products:

OXIDES OF NITROGEN; HYDROGEN BROMIDE, CHLORINE, AND BROMINE.

Section VIII - Spill Or Leak Procedures

Steps to be Taken if Material is Released or Spilled:

WEAR APPROPRIATE PROTECTIVE CLOTHING. USE CARE AS SPILLS MAY BE SLIPPERY. DIKE AND CONTAIN SPILL. ABSORB WITH AN INERT MATERIAL AND TRANSFER ALL MATERIAL INTO A PROPERLY LABELED CONTAINER FOR DISPOSAL. PREVENT PRODUCT FROM CONTAMINATING SOIL OR FROM ENTERING SEWAGE AND DRAINAGE SYSTEMS AND BODIES OF WATER. FLUSH AREA WITH WATER.

Waste Disposal Method(s):

DISPOSE OF IN ACCORDANCE WITH ALL FEDERAL, STATE, AND LOCAL REGULATIONS.

Neutralizing Agent:

USE WATER AND A DEHALOGENATING AGENT SUCH AS SODIUM THIOSULFATE SOLUTION. ADD CAUTIOUSLY WHILE MIXING. WEAR APPROPRIATE PROTECTIVE CLOTHING.

Section IX - Special Protection information

Required Ventilation:

LOCAL VENTILATION IS RECOMMENDED TO CONTROL EXPOSURE FROM OPERATIONS THAT CAN GENERATE EXCESSIVE LEVELS OF MISTS. LOCAL VENTILATION IS PREFERRED, BECAUSE IT PREVENTS DISPERSION INTO WORK AREAS BY CONTROLLING IT AT ITS SOURCE.

Respiratory Protection:

RESPIRATORS SHOULD BE SELECTED BY AND USED UNDER THE DIRECTION OF A TRAINED HEALTH AND SAFETY PROFESSIONAL FOLLOWING REQUIREMENTS FOUND IN OSHA'S RESPIRATOR STANDARD (29 CFR 1910.134) AND ANSI'S STANDARD FOR RESPIRATORY PROTECTION (Z88.2-1992). FOR CONCENTRATIONS ABOVE THE TLV AND/OR PEL BUT LESS THAN 10 TIMES THESE LIMITS, A NIOSH APPROVED HALF-FACEPIECE RESPIRATOR EQUIPPED WITH APPROPRIATE CHEMICAL CARTRIDGES MAY BE USED. FOR CONCENTRATIONS GREATER THAN 10 TIMES THE TLV AND/OR PEL, CONSULT THE NIOSH RESPIRATOR DECISION LOGIC FOUND IN PUBLICATION NO. 87-116 OR ANSI Z88.2-1992.

Glove Protection:

NEOPRENE OR NITRILE RUBBER GLOVES SHOULD BE WORN WHEN HANDLING. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR HAND PROTECTION, 29 CFR 1910.138.

Eye Protection:

CHEMICAL GOGGLES AND A FACE SHIELD SHOULD BE WORN WHEN HANDLING. ENSURE COMPLIANCE WITH OSHA'S PERSONAL PROTECTIVE EQUIPMENT (PPE) STANDARD FOR EYE AND FACE PROTECTION, 29 CFR 1910.133.

Other Protection:

WEAR PROTECTIVE CLOTHING WHEN HANDLING. A SAFETY SHOWER AND AN EYEWASH STATION SHOULD BE AVAILABLE.

Section X - Storage and Handling Information

Storage Temperature -	-	j     i-Storage Conditions	-	-	—	-

Max:<90°F	Min:45°F	|       [V] Indoors	[] Outdoors	[] Heated	[] Refrigerated

Precautions to be Taken in Handling and Storing:

ALWAYS STORE MATERIAL IN ITS ORIGINAL CONTAINER IN A COOL, DRY AREA AWAY FROM DIRECT SUNLIGHT. THIS PRODUCT IS NOT COMPATIBLE WITH PLASITE(R) 4300, KYNAR(R) (PVDF), NEOPRENE, NYLON, SILICONS RUBBER, BUNA-N, BRASS, ALUMINUM, CARBON STEEL, STAINLESS STEEL, AND OTHER METALS. KEEP CONTAINER TIGHTLY CLOSED WHEN NOT IN USE. KEEP FROM FREEZING. IF

BUILD-UP OF PRESSURE OVER TIME, SLOWLY VENT CONTAINER WHEN OPENING.

Other Precautions:

KEEP OUT OF REACH OF CHILDREN. READ THE ENTIRE LABEL BEFORE USING THE PRODUCT. FOLLOW THE LABEL DIRECTIONS.

3 of 4

MATERIAL SAFETY DATA SHEET: AQUABROM PLUS

Section XI - Regulatory Information

Chemical Name	CAS Number	Upper % Limit

None.

Those Ingredients listed above are subject to the reporting requirements of 313 of Title III of the Superfund Amendments and Reauthorization Act of 1986 and 40 CFR part 372.

Please call 1-800-527-9919 for additional information if you are a California customer. This MSDS is not intended for users in the state of

California.

Section XII - References

2. OSHA PEL. 3. VENDOR'S MSDS. 4. REGISTRY OF TOXIC EFFECTS OF CHEMICAL SUBSTANCES, CCINFOWeb, 2003. ALL THE COMPONENTS OF THIS PRODUCT ARE IN COMPLIANCE WITH THE TOXIC SUBSTANCES CONTROL ACT (TSCA) AND ARE EITHER LISTED ON THE TSCA INVENTORY OR OTHERWISE EXEMPTED FROM LISTING. IRR:IRRITANT, FLAM/FLAMM:FLAMMABLE, COMB'.COMBUSTIBLE, CORR:CORROSIVE CARC:CARCINOGENIC, TOX:TOXIC, OX:OXIDIZER, N/A:NOT APPLICABLE, N/E:NOT ESTABLISHED, COC:CLEVELAND OPEN CUP, PMCC:PENSKY-MARTIN CLOSED CUP, TCC:TAGLIABUE CLOSED CUP, LELLOWER EXPLOSION LIMIT, UEL.'UPPER EXPLOSION LIMIT, NFPAiNATIONAL FIRE PROTECTION ASSOCIATION, IARC:INTERNATIONAL AGENCY FOR THE RESEARCH ON CANCER, NTP:NATIONAL TOXICOLOGY PROGRAM, OSHA:OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION, ACGIH:AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS, TLVTHRESHOLD LIMIT VALUE, PEL:PERMISSIBLE EXPOSURE LIMIT, STELSHORT-TERM EXPOSURE LIMIT, MLD:MILD, MOD:MODERATE, SEV:SEVERE, MUT:MUTAGENIC, ASPHYX:ASPHYXIANT, PNOS:PARTICLES (INSOLUBLE) NOT OTHERWISE SPECIFIED, SDT:STANDARD DRAIZE TEST, ORLORAL, IHLINHALATION, HMN:HUMAN

THE INFORMATION CONTAINED HEREIN IS BASED ON DATA CONSIDERED ACCURATE IN LIGHT OF CURRENT FORMULATION. HOWEVER, NO WARRANTY IS EXPRESSED OR IMPLIED REGARDING THE ACCURACY OF THE DATA OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF.

CHEM-AQUA, INC assumes no responsibility for personal injury or property damage caused by the use, storage, or disposal of the product in a manner not recommended on the product label. Users assume all risks associated with such unrecommended use, storage, or disposal of the product.

©2006 NCH Corporation All rights reserved.

4 of 4